Gpyro – A Generalized Pyrolysis Model for Combustible Solids

Technical Reference

Version 0.800

May 1, 2014

Chris Lautenberger

Reax Engineering Inc.

1921 University Ave

Berkeley, CA 94704

510-629-4930 x801

lautenberger@reaxengineering.com

Acknowledgments

The financial support of NSF and NASA is gratefully acknowledged. Gpyro development was funded as part of NSF Award 0730556, “Tackling CFD Modeling of Flame Spread on Practical Solid Combustibles” (September 2007-September 2010). Development of early Gpyro versions was funded by the NASA Graduate Student Researcher Program under Grant #NNC-04HA08H, “Piloted Ignition and Flame Spread on Composite Materials in Partial and Normal Gravity” (August 2004 – August 2007).

Esther Kim at WPI and Amanda Dodd at UC Berkeley have played important roles in model development by serving as beta testers and exercising Gpyro in ways that were not contemplated during its initial development. I would like to thank them for their patience, their bug reports, and their suggestions as to how to improve Gpyro. Bryan Klein at NIST has helped get the source code and other files under Subversion control on Google Code. Thomas Steinhaus from the University of Edinburgh showed great patience as we worked together to get Gpyro running on a cluster there.

Contents

Nomenclature	iv

1.0 Introduction	9

2.0 Preliminaries, Properties, and Definitions	10

2.1 Bulk density and porosity	10

2.2 Effective thermal conductivity	11

2.3 Apparent specific heat capacity	12

2.4 Radiative properties	13

2.5 Permeability	13

2.6 Gas density and molecular weight	14

2.7 Gaseous specific heat capacity and enthalpy	14

2.8 Gaseous mass diffusivity, thermal conductivity, and viscosity	14

3.0 Reaction Rates and Source Terms	16

3.1 Heterogeneous reaction stoichiometry	16

3.2 Heterogeneous reactions - volumetric source terms	17

3.3 Heterogeneous reactions - noncharring, charring, and intumescent	19

3.4 Heterogeneous reaction kinetics	20

3.4.1 Specialized reaction models	23

3.5 Heat release/absorption due to heterogeneous reactions	24

3.6 Homogeneous gas phase reactions	27

3.7 Gaseous species total source terms	29

4.0 Governing Equations	30

4.1 One-dimensional (with volume change)	30

4.1.1 Control volume system	30

4.1.2 Condensed phase mass conservation	31

4.1.3 Condensed phase species conservation	31

4.1.4 Gas phase mass conservation	32

4.1.5 Gas phase species conservation	32

4.1.6 Condensed phase energy conservation	33

4.1.7 Gas phase energy conservation	35

4.1.8 Gas phase momentum conservation	36

4.2 One-dimensional (no volume change)	37

4.3 Two-dimensional (no volume change)	38

4.3 Three-dimensional (no volume change)	39

4.4 Zero-dimensional	39

5.0 Boundary and Initial Conditions	42

5.1 Default – 1D virtual Cone Calorimeter (or similar) experiment	42

5.1.1 Condensed-phase mass conservation	42

5.1.2 Condensed-phase species conservation	42

5.1.3 Gas-phase mass conservation	42

5.1.4 Gas-phase species conservation	43

5.1.5 Condensed-phase energy conservation	43

5.1.6 Gas-phase energy conservation	44

5.1.7 Gas-phase momentum conservation	44

5.2 Generalized boundary conditions	44

5.2.1 Condensed-phase mass conservation	45

5.2.2 Condensed-phase species conservation	45

5.2.3 Gas-phase mass conservation	45

5.2.4 Gas-phase species conservation	45

5.2.5 Condensed-phase energy conservation	46

5.2.6 Gas-phase energy conservation	46

5.2.7 Gas-phase momentum conservation	47

5.4 Geometry masking and initial conditions	47

5.5 FDS-Gpyro coupling	47

6.0 Numerical Solution Methodology	48

6.1 Source term decomposition	48

6.2 Convective-diffusive solver	48

6.2 Tri–diagonal matrix (Thomas) algorithm	50

6.3 Convergence criteria and relaxation	52

6.4 Condensed phase mass conservation	Erreur ! Signet non défini.

6.5 Condensed phase species conservation	53

6.6 Gas phase mass conservation	55

6.7 Gas phase species conservation	56

6.8 Condensed phase energy conservation	Erreur ! Signet non défini.

6.9 Gas phase energy conservation	63

6.10 Gas phase momentum conservation	65

6.11 Extension to two dimensions	67

7.0 Material Property Estimation	71

7.1 Background	71

7.2 Genetic algorithm for estimating material properties	72

7.2.1 Initial population	72

7.2.2 Fitness	73

7.2.3 Selection for reproduction	74

7.2.4 Reproduction	75

7.2.5 Mutation	75

7.2.6 Replacement	75

7.3 Application to a synthetic material with known properties	76

8.0 Concluding Remarks	79

9.0 References	80

# Nomenclature

Letters

a	Coefficients in discretized equations or Thomas algorithm

A	Reactant species

A	Parameter in Equation 150

b	Coefficient in discretized equations or Thomas algorithm

b	Exponent in Equation 47

B	Product species or parameter in Equation 150

c	Specific heat capacity (J/kg–K)

c	Coefficient in Thomas algorithm

C	Parameter in Equation 150

d	Coefficient in Thomas algorithm or pore diameter (m)

D	Diffusivity (m2/s) or parameter in Equation 150

Coefficient in discretized Equations, see e.g. Equation 127c

E	Activation energy (kJ/mole) or parameter in Equation 150

F	Coefficient in convective diffusive equations (Equation 143); parameter in Eq. 150

G	Parameter in Equation 150

h	Enthalpy (J/kg)

hc	Convective heat transfer coefficient (W/m2–K)

hcr	Inverse contact resistance (W/m2–K)

hcv	Volumetric heat transfer coefficient (W/m3–K)

H	Change in enthalpy (J/kg)

Diffusive mass flux (kg/m2–s)

J	Coefficient in convective–diffusive equation, see e.g. Equation 143

k	Thermal conductivity (W/m–K)

kb	Boltzmann constant (J/K)

K	Permeability (m2)

K	Number of condensed phase reactions

L	Number of homogeneous gas phase reactions or thickness (m)

m	Mass (kg)

Mass per unit area (kg/m2)

Mass flux (kg/m2–s)

M	Number of condensed phase species

M	Molecular mass (g/mol or kg/mol)

n	Exponent (reaction order;  Property exponent, Equation 4)

N	Number of gaseous species

p	Exponent in Equation 47

P	Pressure (Pa)

P	Cell Peclet number, e.g. Equation 127e

P	Coefficient in Thomas algorithm, Equation 130

Pr	Prandtl number

q	Exponent in Equation 47

Heat flux (W/m2);  is conductive,  is radiative,  is external radiative

Q	Coefficient in Thomas algorithm, Equation 130

Volumetric rate of heat release or absorption (W/m3)

r	Reaction rate (kg/m3–s)

R	Universal gas constant (J/mole–K)

s	Position of solidification front in Stefan problem (m)

S	Source term

Sc	Schmidt number

t	Time (s)

T	Temperature (K)

t	Time step (s)

X	Volume fraction (–)

y	Yield

Y	Mass fraction (–)

z	Distance (m)

Z	Condensed phase pre–exponential factor (s–1)

Z	Gas phase pre–exponential factor, Equation 47 (units of m, mole, K, s)

z	Distance from center of grid one grid cell to center of adjacent grid cell (m)

z	Size (height) of grid cell (m)

Greek symbols

	Relaxation parameter or conversion

	Heating rate (K/min or K/s)

	Length scale (m) controlling radiant conductivity, see Equation 7

	Effective diffusion coefficient in discretized equations

	Thickness (m)

	Emissivity (–)

	Energy of attraction between two molecules, Equation 19 (J)

	Small value

	In–depth radiation absorption coefficient (m–1)

	Viscosity (m2–s)

	Density (kg/m3)

	Stefan–Boltzmann constant (W/m2–K4)

	Collision diameter of molecular species, Equation 19 (Å)

Parameter in Equation 9c (K2)

	Generic variable

	Parameter controlling swelling, see Equation 23 and 24

	Porosity (–)

	Function of dimensionless temperature

Volumetric reaction rate (kg/m3–s)

Subscripts

A	Species A

b	Baseline or bottom (interface value)

B	Species B

B 	Bottom (cell center value)

d	Destruction or datum

D	Diffusivity

e	East (interface value)

e	External (as in )

exp	Experimental

E	East (cell center value)

f	Formation

g	Gaseous, gas phase, or gasification

i	Condensed phase species i

j	Gaseous species j

k	Heterogeneous reaction k

Homogeneous gas phase reaction

m	Melting

mod	Model

nb	No blowing

O2	Oxygen

P	Point P

r	Reference (as in Tr) or radiative (as in kr)

s	Solid phase (really, condensed phase)

sol	Solid

t	Top (interface value)

T	Top (cell center value)

vol	Volatilization

w	West (interface value)

W	West (cell center value)

0	Initial (as in T0)

Ambient

	At z = 

	Integrated (summation)

Superscripts

+	Positive part of source term

–	Negative part of source term

Weighted or averaged

Value at present time

Ambient

# 1.0 Introduction

This document is the Technical Reference to Gpyro, a generalized pyrolysis model for combustible solids (see http://reaxengineering.com/trac/gpyro). General background information, a discussion of the model’s capabilities, and instructions for setting up and running Gpyro simulations can be found in the Users Guide [1]. Additional technical details can be found in Reference [2]. Both the Users’ Guide and Technical Reference will be continuously updated as Gpyro evolves.

Gpyro solves a generalized set of coupled equations, rather than separate model equations for different classes of materials. A particular material is simulated by specifying a set of model parameters (thermophysical properties, reaction mechanism, etc.), and a particular experimental configuration is simulated by specifying initial and boundary conditions. The flexibility to invoke or omit submodels for various transport phenomena is provided because there may be little consequence to omitting a particular phenomenon from a simulation other than reducing the computational expense and the number of adjustable parameters. Instead of hardcoding a certain level of complexity in Gpyro, the user determines how much complexity or detail to include in a simulation. For example, thermophysical properties may be temperature–dependent or invariant with temperature; decomposition mechanisms can be single–step or multi–step; thermal radiation can be absorbed only at the surface or in–depth, and so on. Some of the physics embedded in the governing equations may be superfluous for practical engineering applications so they can be omitted from a simulation if desired.

At the heart of Gpyro, separate conservation equations are solved for gaseous and condensed phase mass, species, and energy as well as gas phase momentum using a Darcy’s law approximation (Stokes flow). An arbitrary number of gas phase and condensed phase species can be accommodated, each having its own thermophysical properties. The user may specify any number of heterogeneous (solid/gas) or homogeneous (gas/gas) reactions. Both in–depth radiation absorption in a semi–transparent medium and radiation transport across pores are considered. Melting is modeled using a localized increase (Gaussian peak) in the apparent specific heat capacity. All volatiles generated inside the solid escape to the ambient with no resistance to flow unless the pressure solver is invoked to solve for the pressure distribution in the solid, in which case the resultant flow of volatiles is calculated according to Darcy’s law. Similarly, the user may invoke a gas phase convective–diffusive solver that determines the composition of the volatiles, including diffusion of gaseous species from the ambient into the decomposing porous solid. Thus, in addition to calculating the mass flux of volatiles escaping from the solid, the actual composition of these vapors is calculated.

# 2.0 Preliminaries, Properties, and Definitions

Consider a condensed phase combustible material. It may contain as many as M chemically distinct condensed phase species. Examples of different condensed phase species include pure polymer, glass fiber reinforcements, char, and ash. Within a material, the initial concentrations of each species may be uniform (as in the case of a homogeneous blended composite) or vary spatially (as in the case of laminated composites). As the solid is heated, it may degrade to form N chemically distinct gaseous species. These species include hydrocarbon fragments, water vapor, carbon monoxide, etc. Furthermore, gaseous species may be consumed or produced by both heterogeneous (solid/gas) reactions and homogeneous (gas/gas) reactions

In this Technical Reference, the index i is used to denote condensed phase species and the index j is used to denote gaseous species. Thus, Yi is the mass of condensed phase species i divided by the total mass of all condensed phase species, and Yj is the mass of gaseous species j divided by the mass of all gaseous species. Note that Yj is not the mass of gaseous species j divided by the mass of all gaseous and condensed phase species. As implied earlier, M is the number of condensed phase species and N is the number of gaseous species.

Thermophysical properties (k, , and c) of each condensed-phase species are assumed to vary with temperature as , where 0 and n are user-specified parameters and Tr is a user-specified reference temperature. Thus, Gpyro can accommodate general trends such as an increase in c with temperature. However, detailed temperature dependencies of the underlying thermophysical properties cannot be included. This is not necessarily a drawback because rarely are accurate temperature–dependent measurements of k(T), c(T), or (T) available, particularly at temperatures above 300 ºC that are relevant to burning. Additionally, few detailed property measurements are available for intermediate species such as char or ash. It was felt that the simplicity of a two–parameter approximation for thermophysical properties outweighs any potential drawbacks associated with the inability to specify detailed temperature dependent properties.

## 2.1 Bulk density and porosity

Since the condensed phase is composed of M distinct species, each having its own thermophysical properties, the effective properties appearing in the conservation equations presented later must be calculated from an appropriately weighted local composition. Some quantities are weighted by the local volume fraction, and others are weighted by the local mass fraction. The relation between mass fraction and volume fraction is:

(11)

Here  is the weighted bulk density, i is the bulk density of condensed phase species i, Xi is the volume fraction of condensed phase species i, and Yi is the mass fraction of condensed phase species i. An overbar denotes an averaged quantity, i.e. the weighted bulk density is:

(22)

Weighted bulk density can also be calculated from condensed-phase species mass fractions:

(33)

The bulk density of each condensed phase species is assumed to vary with temperature according to the relation:

(44)

where  and  are user–specified parameters and Tr is a user–specified reference temperature, often (but not necessarily) taken as close to room temperature for convenience.

Note that in Equations 1 to 4, i is the bulk density of species i (in a vacuum if it is porous). Bulk density is defined as total mass divided by total volume, not the density of a pure nonporous solid. The designation of the bulk density of species i as i is different from nomenclature often used in other pyrolysis models where i is the mass of species i per unit volume of mixture (which would instead be represented here as ).

To simplify Gpyro, it is assumed that porosity () is a “property” of each condensed phase species. This eliminates the need to solve a separate equation for porosity evolution because the weighted porosity can be calculated from the local composition as:

(55)

The porosity of species i, denoted i, is calculated as follows:

ψiT=1-ρiTρs0,i	(66)

Where s denotes the “solid” density, i.e. the density of a pure nonporous solid, and is a user–specified constant corresponding to the solid density of species i at a particular temperature. The quantity  is the bulk density of species i (Equation 4). The temperature-dependent porosity of species i is fixed via specification of 0,i, n,,i, Tr, and s0,i. The small density contribution from the gases that fill the voids () does not affect Equation 4 because, as indicated earlier, the bulk density is formally defined under vacuum. This is more of a technical than a practical consideration because  except under special circumstances. The quantity  is calculated via the gas phase mass conservation equation, as described later.

## 2.2 Effective thermal conductivity

The effective thermal conductivity of a solid material is affected by its density, porosity, moisture content, microstructure, and orientation of wood grain or reinforcements. However, as with the presumed temperature dependency of condensed phase bulk density, a simple functional form for condensed–phase effective thermal conductivity is assumed:

(77)

As with bulk density and porosity, the averaged effective thermal conductivity is weighted by condensed phase volume fractions:

(88)

In Equation 7, the effective thermal conductivity of each condensed phase species is broken into a solid and a radiative component (ks,i and kr,i respectively). The latter is attributed to radiation heat transfer across pores and may become a dominant mode of heat transfer in porous media at high temperatures. The parameter i controls the radiative contribution to the effective thermal conductivity. It has units of length and depends on the pore structure and other considerations (see Ref. [2] for details). Radiative transport across pores can be omitted by setting i  = 0.

It is inherently assumed that the effective thermal conductivity specified by setting k0,i, nk,i, and i in Equation 7 takes into account bulk density, porosity, etc. This is commensurate with the spirit of the present work, which aims to formulate a generalized model and code the governing equations in a self–consistent manner rather than consider detailed submodels for thermophysical properties of specific materials. There are several models in the literature that can be used to estimate effective thermal conductivities on the basis of bulk density, porosity, grain orientation, etc. The user is free to use such models as the basis for specifying k0,i nk,i and i in Equation 7.

## 2.3 Apparent specific heat capacity

The temperature–dependent specific heat capacity of species i is assumed to be the sum of a “baseline” specific heat (cb,i) and (for polymers that melt) the apparent increase in the specific heat capacity due to the latent heat of melting (cm,i):

(99a)

(9b)

The latent heat of melting (Hm) is assumed to be distributed via a Gaussian peak centered at Tm, the melting temperature:

(9c)

In Equation 9c, is a model parameter having units of K2 that controls the “width” of the Gaussian specific heat capacity peak attributed to the latent heat of melting. If a condensed phase species with constant specific heat capacity is desired, then the user sets  and .

Inherent in Equation 9c is the assumption that a condensed phase species melts over a range of temperatures rather than at a single temperature. This is not necessarily a shortcoming because only crystalline solids have a well–defined melting temperature. Most polymers are a mixture of crystalline and amorphous components so that melting (or softening of the amorphous components) usually occurs over a finite temperature range.

The sensible enthalpy of condensed phase species i at temperature T is determined by integration of Equation 9a after substituting Equations 9b and 9c. The result is:

(1010)

Note that  is the temperature datum used to establish an absolute value of enthalpy (it is not necessarily the same as the ambient, initial, or reference temperature). In Gpyro, the temperature datum is arbitrarily taken as 200 K.

Enthalpy and specific heat capacity are weighted by mass fractions:

(1111)

(1212)

## 2.4 Radiative properties

The user must also specify for each condensed-phase species an emissivity (i, dimensionless) and radiative absorption coefficient (i, m–1). Emissivity controls the fraction of the incident radiation absorbed at the surface and the fraction of the blackbody emissive power of the surface that is radiated away. The radiative absorption coefficient controls the depth over which incident thermal radiation is attenuated, see Equations 66 and 67. Radiative properties are weighted by volume:

(1313)

(1414)

## 2.5 Permeability

If the pressure solver is invoked, each condensed phase species must be assigned a permeability.  The weighted permeability is calculated on a volume basis:

(1515)

## 2.6 Gas density and molecular weight

In addition to condensed-phase properties, specification of several gas-phase properties is required. The gas-phase density is calculated from the ideal gas law:

(1616)

In Equation 16, R is the universal gas constant , and  is calculated from the local volume fractions of all gaseous species:

(1717)

## 2.7 Gaseous specific heat capacity and enthalpy

It is assumed that all gaseous species have equal specific heat capacities that are independent of temperature. This approximation is justified given that little is known about the actual composition of the gaseous volatiles produced during the pyrolysis and gasification of practical materials. As with the condensed phase enthalpy, the gas phase sensible enthalpy is weighted by mass:

(1818)

where cpg denotes the (constant) gaseous specific heat capacity.

## 2.8 Gaseous mass diffusivity, thermal conductivity, and viscosity

Gaseous diffusion coefficients are calculated from Chapman–Enskog theory. The binary diffusion coefficient (units of m2/s) for species A diffusing into species B is:

(1919a)

In Equation 19a, MA and MB have units of g/mol, AB is a weighted collision diameter of species A and B (units of Å), kb is the Boltzmann constant, and  is the maximum energy of attraction between molecules A and B. Thus, AB and (/kb)AB are model parameters that describe the binary diffusion coefficient of species A into species B. These parameters are tabulated for several gases by Bird et al. [7], along with  as a function of . Note that  and  are weighted averages between molecules A and B:

(19b)

(19c)

Since little is known about the composition of gaseous volatiles for practical materials, several approximations are made to simplify the gas phase calculations. It is assumed that all gaseous species have the same diffusivity (DAB = D), taken as that of oxygen into the (user-specified) background species. Unit Prandtl, Schmidt, and Lewis numbers are (Pr = Sc = 1). It follows from these approximations:

(2020a)

(20b)

Equation 20a allows for considerable simplification of the gas phase energy equation.

To summarize, in addition to molecular weight, there are five gas phase properties:  kg, g, D, cpg, and . g is calculated from Equation 16, cpg is a user–specified constant, D is calculated from Equation 19, and the remaining two properties (kg and ) are calculated from the relations in Equation 20.

# 3.0 Reaction Rates and Source Terms

The governing equations, presented in the next chapter, contain source terms attributed to reactions that must be quantified. In Gpyro, two types of reactions are considered:  heterogeneous (solid/gas) and homogeneous (gas-gas).

Heterogeneous reactions involve the destruction of a condensed phase species to form gases and/or additional condensed phase species. An example is char formation wherein one condensed phase species (e.g., wood) is converted to a second condensed phase species (e.g., char) with the release of gases (pyrolysate). Some heterogeneous reactions may also involve the consumption of gases. For example, in heterogeneous char oxidation, oxygen is consumed and partially oxidized gaseous species are produced. Catalytic reactions are a special case of heterogeneous reactions and can be simulated with Gpyro. Solid–solid phase change or glass transition could be modeled as solid-phase reactions with Gpyro; technically these would be homogeneous solid-solid processes, but in the discussion that follows they would be considered heterogeneous reactions.

Homogeneous gas phase reactions involve only gases and do not involve the condensed phase. Examples of such reactions include tar cracking (where large tar molecules are broken into smaller hydrocarbon molecules) and the oxidation of gaseous pyrolysate.

## 3.1 Heterogeneous reaction stoichiometry

A particular condensed phase reaction is denoted by the index k, and the total number of condensed phase reactions is designated K (not to be confused with permeability; it should be clear from context which is meant). In the combustion literature, stoichiometry of gas phase reactions is often expressed using molar “” coefficients; the same is done here, with the important difference that the  coefficients are given on a mass basis. Heterogeneous reactions are represented in Gpyro as:

(2121a)

(21b)

(21c)

(21d)

Ak denotes the condensed phase reactant species (the condensed phase species consumed by reaction k) and Bk denotes the condensed phase product species (the condensed phase species generated by reaction k). The composition of the gases consumed and produced by a heterogeneous reaction is controlled by the parameters  and . In Equation 21a,  is the net mass of gaseous species j consumed by reaction k, and  is the net mass of gaseous species j produced by reaction k per unit mass of Ak consumed.

In order to generalize and simplify the specification of  and , a user–defined “species yield matrix” is introduced. This N by K matrix (recall that N is the total number of gaseous species and K is the number of heterogeneous reactions) is denoted ys,j,k and appears on the RHS of Equations 21c and 21d. The entries of ys,j,k may be positive or negative, with a positive entry corresponding to net production of a gaseous species by a particular reaction, and a negative entry corresponding to net consumption of a gaseous species by a particular reaction. The physical meaning of each entry in the species yield matrix is the net mass of gas phase species j consumed or generated by reaction k per unit mass of condensed phase that is converted to gases by reaction k. The sum of all entries in any column of the species yield matrix must add to 1 to conserve mass, and this must be strictly adhered to or nonphysical results will occur. Substitution of Equations 21b, 21c, and 21d into Equation 21a results in a balanced equation (conserves mass) only if .

The  coefficients are related to the “solid fraction” (SF) of reaction k, denoted SFk , see Equation 21b - 21d. The solid fraction of reaction k is the fraction of condensed phase species Ak’s mass that is converted to condensed phase species Bk. Extending this terminology, 1 – SFk could be called the “gas fraction” of reaction k. The solid fraction is related to the bulk densities of condensed phase reactant and product species Ak and Bk as:

(2222)

Since bulk densities may depend on temperature, SFk may be a function of temperature. In Equation 22, the parameter  is the fraction of the bulk density difference between condensed phase species Ak and Bk that is converted to gases. This is discussed in Section 3.3.

Gpyro assumes that each condensed phase reaction k consumes a single condensed phase species (Ak) and forms a single condensed phase species (Bk) plus gases. Although each condensed phase reaction k transforms a single condensed phase species (Ak) into at most one additional condensed phase species (Bk) plus gases, the user may specify multiple reactions that transform condensed phase species A to different species. Consequently, reactions of the type  can be accommodated with Gpyro via specification of multiple reactions that transform reactant A into different products.

A pyrolysis mechanism in the form of Equation 21a can be converted to the present treatment using the relations in Equations 21 and 22.

## 3.2 Heterogeneous reactions - volumetric source terms

In Section 3.3 below, kinetics considerations are used to develop an expression for , the local volumetric destruction rate of condensed-phase reactant Ak. In Gpyro, local volumetric formation or destruction rates of all other condensed/gaseous species by heterogeneous reactions are related to  via the stoichiometry presented in Section 3.1. Volumetric source terms are needed because the conservation equations presented later contain volumetric reaction rates.

The formation rate of condensed phase species Bk by reaction k () and the rate at which the mass of condensed phase species Ak is volatilized or converted to gases by reaction k () are calculated, respectively, from Equations 23 and 24:

(2323)

(2424)

The solid fraction of reaction k (SFk) that appears in Equations 23 and 24 is defined in Equation 22. Recall that SF is the fraction of the mass of condensed phase species A that is converted to condensed phase species B;  the remainder of the mass of condensed phase species A is volatilized, i.e. converted to gases (Equation 24). Adding Equations 23 and 24 gives:

(2525)

Equation 25 says that for reaction k, the rate at which condensed phase species Ak is converted to condensed phase species Bk plus the rate at which condensed phase species Ak is converted to gases equals the destruction rate of condensed phase species Ak.

Equation 24 above gives the formation rate of all gases by reaction k (). However, it does not provide any information about the production/destruction rates of individual gaseous species (or equivalently, the composition of the gases that are generated or consumed by that reaction). This information is provided via the user–specified species yield matrix ys,j,k mentioned earlier. Specifically, the formation and destruction rates of gaseous species j from condensed phase reaction k are calculated as:

(2626)

(2727)

where ys,j,k is the N by K species yield matrix. See the text following Equation 21 for additional discussion about ys,j,k. The subscript s stands for “solid” as a reminder that gaseous species may also be formed or destroyed by homogeneous gas phase reactions (discussed later).

Note that  is the rate at which the mass of condensed phase species Ak is volatilized (converted to gases) by reaction k. It is not necessarily the same as the total production rate of gases as illustrated by the following example:  Consider a char oxidation reaction where gaseous oxygen is consumed at a rate of 2 kg/m3–s and solid char is consumed at a rate of 1 kg/m3–s to produce 3 kg/m3–s of product gases. In this case,  would be 1 kg/m3–s (not 3 kg/m3–s) because although a total of 3 kg/m3–s of product gases are being produced, only 1 kg/m3–s of this originated in the condensed phase as the mass of species Ak.

The volumetric source terms above apply to a single reaction k. However, the conservation equations contain total source terms, which are obtained by summing over all reactions. The total destruction and formation rates of gases due to heterogeneous reactions are calculated by summing over Equations 26 and 27:

(2828)

(2929)

The total formation rate of all gases by volatilization of the condensed phase () is calculated by summing Equation 24 over all condensed phase reactions:

(3030)

The total destruction rate of condensed phase species i, and the total formation rate of condensed-phase species i, are calculated as:

(3131)

(3232)

In Equations 31 and 32,  is the Kronecker delta, which has the properties for general indices i and j:

(3333)

In Equation 33, i and j don’t represent condensed and gaseous species as elsewhere.

## 3.3 Heterogeneous reactions - noncharring, charring, and intumescent

A noncharring reaction forms no condensed phase residue, and condensed phase reactant species Ak is transformed completely to product gases. This type of reaction is invoked in Gpyro by setting  and specifying no condensed-phase product species Bk. It can be seen from Equation 23 that SFk becomes zero if  and . Consequently, the formation rate of condensed phase species Bk is also zero. At the same time, the quantity 1 – SFk in Equation 24 (one minus the solid fraction) becomes 1, meaning that only gases are generated. Mass conservation requires that this type of reaction causes surface regression. Note that  must be set equal to 1 for a noncharring reaction or mass will not be conserved.

For charring and intumescent reactions, the degree of shrinkage or swelling is controlled by the parameters , , and . As will be described below, these parameters can be used to simulate charring reactions that result in no net volume change, charring reactions where swelling occurs (intumescence), charring reactions where shrinkage occurs, and condensation reactions where gas-phase mass is converted to condensed-phase mass.

If  and , a charring reaction with no volume change will occur. It can be seen from Equations 23 and 24 that, in this case, the formation rate of condensed phase species B and gases are both nonzero. This is similar to the traditional approach to modeling the pyrolysis of charring materials.

Equations 23 and 24 can also be used to model an intumescent reaction with swelling by setting . Then, provided , a lower density solid is formed from a higher density solid and swelling occurs to conserve mass. Essentially, the decrease in  is compensated for by an increase in the size of a grid cell through the condensed phase mass conservation equation, leading to swelling. In the extreme case of , swelling occurs without any escape of gases. Values of  between 0 and 1 cause intumescence (swelling) to occur simultaneously with the release of gaseous vapors. The relative amounts of swelling and volatilization are dictated by the value of  (closer to 0 means more swelling, and closer to 1 means more volatilization).

Values of  between 1 and  can be used to simulate a charring reaction that results in shrinkage (provided ). Essentially, the decreased formation rate of condensed phase species Bk (see Equation 23 and note that the term in parentheses is negative) caused by a value of k greater than 1 leads to a decrease in the local grid size through the condensed phase mass conservation equation.

Finally, values of  greater than  can be used to simulate condensation reactions where gas phase mass is converted to condensed phase mass.

The preceding discussion assumes that  and therefore a lower density solid is formed from a higher density (for example, char formed from wood has a lower density than the virgin wood). However, the case where  is also conceivable, and the behavior is essentially the opposite of intumescence. If k = 0, then shrinkage would occur with no change in mass. This could be used to model a porous material that melts or collapses when heated. If k = 1, then no volume change would occur, but an increase in mass would occur (negative mass loss rate). For , shrinkage would occur simultaneously with mass gain, with the relative amounts of shrinkage and mass gain dictated by the value of k.

To summarize, a heterogeneous reaction causes shrinkage or swelling only when , with the noted exceptions that mass conservation requires noncharring reactions to cause surface regression and if  then shrinkage and mass gain can occur depending on the value of k (see preceding paragraph).

## 3.4 Heterogeneous reaction kinetics

Pyrolysis reaction stoichiometry and calculation of analogous volumetric source terms are described above, but the rate at which reactions occur (kinetics) must also be quantified. The starting point for the treatment of condensed phase decomposition kinetics is an equation often used to analyze TGA data with a single-step, nth order reaction:

(3434)

In Equation 34,  is the conversion (dimensionless), Z is the pre-exponential factor (s-1), E is the activation energy (kJ/mol), and n is the reaction order. Note that the conversion is related to the instantaneous mass (m) and the initial mass (m0) as  = 1 – m/m0. Since Equation 34 implies a single–step reaction, several changes are required before it can be generalized to accommodate multi–step reactions. Details are given in Ref. [2] and summarized below.

Define i as the conversion of condensed phase species i. This quantity is tracked locally in each cell, so 1 – i can be calculated in a cell having size z as:

(3535)

In Equation 35, the numerator is the mass per unit area of condensed phase species i in a particular cell;  the denominator is the initial (i.e., at t = 0) mass per unit area of condensed phase species i in that same cell plus the cumulative formation of species i through time t in that cell:

(3636)

The subscript  serves as a reminder that  includes an integral (evaluated numerically as a summation). Note that  because  is the formation rate of condensed phase species i (not the formation rate minus the destruction rate). Thus, the quantity  remains constant or increases monotonically; it can never decrease. Since conversion (Equation 35) is tracked separately in each cell, from a kinetics standpoint each cell behaves as if it was a TGA sample.

After some manipulation [2] the destruction rate of species i by reaction k is calculated as:

(3737)

The Kronecker delta is used to emphasize that for reaction k with Ak as the condensed-phase reactant species and stoichiometry given by Equation 21, species i is destroyed only if . Thus, Equation 37 can be recast in terms of Ak:

(3838)

Equation 38 is used as the basis for the treatment of heterogeneous reaction source terms in Gpyro after making use of Equation 35 and including a multiplicative function  to account for the effect of the presence of oxygen on the reaction rate. The final form of the heterogeneous reaction rate for a general nth order reaction is:

rk=ωdAk‴=fαAkρYAkΔzΣΔzZkexp-EkRTgYO2HT-Tcrit,k	(3939a)

(39b)

Here,  is the reaction model, assumed to be nth order by default per Equation 39b (alternate reaction models are discussed in Section 3.4.1).  is the Heaviside step function (equal to 0 for T – Tcrit,k < 0, and equal to 1 for T – Tcrit,k ≥ 0). This effectively forces the reaction rate to be zero below Tcrit,k. The kth reaction rate (rk) is the destruction rate of condensed phase species Ak by condensed phase reaction k and is denoted . For each reaction k, the index of the condensed phase reactant species (Ak) and the index of the condensed phase product species (Bk) are specified by the user. The function  in Equation 39 is calculated as:

(4040)

In Equation 40, the oxygen mass fraction () is the local value inside the decomposing solid as determined by solution of the gaseous species conservation equations. It is not the freestream value (except for TGA simulations, i.e. 0D transient formulation). The exponent  describes the oxygen sensitivity of reaction k, so a value of  is specified for reactions that are not sensitive to oxygen. The parameter IO2TYPE is a user-specified integer that selects via Equation 40 the functional form for the dependence of reaction rate on oxygen concentration is used by Gpyro (see User’s Guide).

Each reaction’s kinetics are characterized by these parameters: 1) nk (reaction order in remaining mass of condensed phase species Ak), 2)  (reaction order in gaseous oxygen mass fraction), also specify IO2TYPE, 3) Zk (pre–exponential factor), 4) Ek (activation energy), and 5) Tcrit,k.

Equation 39 looks somewhat complicated due to its generality, but it can be simplified for some special cases. When nk = 1 and , Equation 39 reduces to the following simple form:

(4141)

Also, for ,  in Equation 36 replaced with  (conventional reaction order approach), and no volume change (z = constant), Equation 39 reduces to:

(4242)

Equation 42 is the same reaction rate treatment used in NIST’s Fire Dynamics Simulator Version 5. Thus, it can be seen that the treatment of pyrolysis reactions in FDS is a special case of the more generalized treatment used in Gpyro.

### 3.4.1 Specialized reaction models

The reaction rate expression developed in the previous section assumes an nth order reaction model (see Equation 39b). However, several additional reaction models are also available.  In particular, by specifying a nonzero value of mk for reaction k, a multiplicative factor  is added to Equation 39a (recall that for reaction k, the condensed-phase reactant species has index Ak). Also, for each reaction, the user may specify an index IKINETICMODEL that invokes a different functional form of  corresponding to various reaction models. The available reaction models are summarized in Table 1.

Table 11. Specialized reaction models.

Reaction model is where  is given below.

| IKIETICMODEL |  | Description |
| --- | --- | --- |
| 0 |  | nth order |
| 1 |  | Nucleation and nucleus growing |
| 2 |  | Phase boundary reaction |
| 3 |  | Diffusion – plane symmetry |
| 4 |  | Diffusion – cylindrical symmetry |
| 5 |  | Diffusion – spherical symmetry |
| 6 |  | Diffusion – Jander’s type |
| 7 |  | Potential law |
| 8 |  | Reaction order |
| 9 |  | Catalytic |

Notes:

## 3.5 Heat release/absorption due to heterogeneous reactions

In Gpyro, the total heat of reaction is calculated as the sum of two separate contributions. One part of the heat of reaction applies only to the generation of condensed phase species and is denoted Hsol,k (where sol stands for solid);  the other part applies only to the generation of volatiles from the condensed phase and is denoted Hvol,k (where vol stands for volatiles). The volumetric rate of heat release or absorption to the solid phase due to reaction k is:

(4343)

Hsol,k is the user-specified chemical (not sensible) specific enthalpy difference between condensed phase species Bk and Ak, i.e.  where a superscript ˚ denotes chemical (not sensible or total) enthalpy. Recall that reaction k converts condensed phase species Ak to condensed phase species Bk plus gases. The change in sensible enthalpy is accounted for by the  term in Equation Erreur ! Source du renvoi introuvable., so it is important to emphasize that Hsol,k does not include sensible enthalpy differences between Bk and Ak. The units of Hsol,k are Joules per kilogram of species Bk produced by reaction k. Positive values of Hsol,k correspond to an endothermic reaction, and negative values correspond to an exothermic reaction. Hsol,k is often set to zero, unless reaction k represents a solid–solid or solid–liquid phase change with an associated change in chemical enthalpy. As will be explained below, Hsol,k may also be nonzero if it is desired to apply a heat of reaction having units Joules per kilogram of condensed phase reactant consumed.

The term Hvol,k is the user-specified quantity of heat required to convert unit mass of condensed-phase species Ak to gases at whatever temperature the reaction occurs. Hvol,k is treated in Gpyro as a user-specified semi-empirical constant because detailed knowledge of the chemical and sensible enthalpies of all gaseous and condensed phase species is required to assign physical/chemical significance to Hvol,k:  formally, Hvol,k is the total (chemical plus sensible) enthalpy of the gases () produced by volatilizing condensed-phase species Ak minus the total enthalpy of condensed-phase species Ak (), i.e. . Here, a subscript T denotes total enthalpy. Note that the rightmost parenthetical term is a constant, in general, only if the reaction always occurs at the same temperature (which is not the case for finite-rate reactions) or the solid reactant species Ak has the same specific heat capacity as the product gases.

In a two-temperature formulation, there is some ambiguity regarding the temperature at which the gaseous sensible enthalpy is evaluated (temperature of condensed or gaseous phase). When newly volatilized gases are produced, their temperature is probably very close to the condensed-phase temperature; however, molecular interactions will cause the newly-generated volatiles to quickly approach the temperature of the gas-phase, so it may be more appropriate to evaluate the gas enthalpy in the above equation at the gas-phase temperature. By assuming that the volatiles generated from the condensed phase have a temperature equal to the gas-phase temperature (i.e. the enthalpy of the gas produced from the condensed-phase is evaluated at the gas-phase temperature instead of the condensed-phase temperature) the term  appearing in the gaseous energy equation cancels because . This approximation is made by default in Gpyro, but the user can specify GASES_PRODUCED_AT_TSOLID = .TRUE. in the &GPYRO_GENERAL namelist group to remove this approximation.

Due to the complexities mentioned above, Gpyro by default makes the approximation that Hvol is a user-specified constant (implying that  is invariant when in actuality it may vary with reaction temperature and other factors). A slightly different treatment of the heat of reaction can be invoked by specifying CONSTANT_DHVOL = .FALSE. in the &GPYRO_GENERAL namelist group, causing the heat of reaction to be calculated as:

(4444)

This formulation ensures that the enthalpy balance is consistent with energy conservation. The principle is that the change in condensed-phase energy after a reaction equals the sensible enthalpy of the products plus the chemical energy released or absorbed, minus the sensible enthalpy of the reactants.

This relation ensures that the enthalpy of reaction always satisfies energy conservation. The change in energy of the condensed phase after a reaction, dhdt, is equal to the enthalpy available after the reaction plus the energy released or absorbed during the reaction, minus the enthalpy available before the reaction. This can be written as follows, where h is the total enthalpy, hi​ the sensible enthalpy of species i, and hi0​ its reference enthalpy of the species i:

Starting from the general energy balance:

dhdt=ωdA,k'''hA+hA0-ωfB,k'''hB+hB0-ωfg,k'''(hg+hg0)

After some manipulation:

dhdt=ωdA,k'''[hA+hA0-θkhB+hB0-(1-θk)(hg+hg0)

dhdt=ωdA,k'''θkhA+hA0-hB-hB0+1-θkhA+hA0-hg-hg0

dhdt=ωdA,k'''θk-ΔHsol,k0+hA-hB+1-θk-ΔHvol,k+hA-hg

dhdt=-ωfB,k'''ΔHsol,k0+hA-hB-ωFg,k'''ΔHvol,k0+hA-hg

The term -ωfB,k'''hA-hB is already included in the energy conservation equation, so the enthalpy of reaction is written as :

dhdt=-ωfB,k'''(hA-hB)+Qk'''

Qk'''=-ωfB,k'''ΔHsol,k0-ωFg,k'''ΔHvol,k0- ωFg,k'''hg-hA

In this case, instead of specifying , the user specifies  (where ) and the difference in the sensible enthalpy between the volatilized gases and the condensed-phase species Ak (the rightmost term in the above equation) is explicitly added to the heat of reaction. Note that the gas enthalpy is evaluated at the solid temperature, as if the volatiles are generated at the solid temperature.

Hvol,k  (or ) has units of J/kg of gases volatilized from the condensed phase (species Ak). A positive value of Hvol,k denotes an endothermic reaction, and a negative value of Hvol,k denotes an exothermic reaction. Hvol,k is sometimes called the heat of pyrolysis, heat of volatilization, or heat of vaporization. Note that Hvol,k is different from the heat of gasification (sometimes denoted Hg or Lg), which includes a contribution for the sensible enthalpy required to heat the condensed phase from its initial temperature to its volatilization temperature.

Both Hsol,k and Hvol,k have base units of J/kg, but some clarification is needed regarding Joules per kilogram of what. These quantities are defined per unit mass of product that originated in the condensed phase. That is, the units of Hsol,k are Joules per kg of condensed phase species Bk produced from condensed phase species Ak. Similarly, the units of Hvol,k are Joules per kg of gases generated by volatilizing condensed phase species Ak (or equivalently, Joules per kg of gaseous mass generated that originated in the condensed phase). Note that any mass that originated in the gas phase is not included in this heat of reaction. For example, assume a noncharring reaction converts 1 kg of condensed phase species A and 1 kg of gaseous oxygen to 2 kg of product gases, releasing 1 MJ of sensible enthalpy in the process. The value of Hvol,k for this reaction is –1 MJ/kg, not –1/2 MJ/kg.

In the literature, heats of reaction are sometimes given on a J per kg of reactant consumed basis. Using the present terminology, such a heat of reaction would have units of Joules per kilogram of species Ak destroyed by reaction k. This treatment of heats of reaction can be recovered in Gpyro by setting Hsol,k = Hvol,k = Hk. In fact, the second equality in Equation 43 follows from the first equality after setting Hsol,k = Hvol,k = Hk and then making use of Equation 25:

(4545)

The flexibility that this affords is the primary motivation for splitting the heat of reaction into two separate components.

Since the source term  appears in the condensed phase energy conservation equation (Equation Erreur ! Source du renvoi introuvable.), any heat release or absorption due to Equation 43 is distributed to the condensed phase (the subscript s denotes “solid” although “condensed” would be more accurate). Whether the heat released or absorbed by a heterogeneous reaction is distributed to the condensed phase or the gas phase is somewhat inconsequential since these phases are usually in a state close to thermal equilibrium on account of the much lower volumetric heat capacity of the gas phase.

## 3.6 Homogeneous gas phase reactions

In addition to heterogeneous reactions involving the condensed and gaseous phases, the user may also specify any number of homogeneous gas phase reactions. Such reactions can be used to account for tar cracking where large hydrocarbon molecules are broken into smaller hydrocarbon molecules or the oxidation of pyrolysate as it flows through a char layer toward the surface where both the oxygen concentration and temperature are highest.

Just as there are K condensed phase reactions and individual condensed phase reactions are indicated by the index k, there are L homogeneous gas phase reactions and individual reactions are indicated by the index ℓ. Each homogeneous gas phase reaction ℓ converts two gas phase reactants (Aℓ and Bℓ) to gaseous products. Since only the gas phase is involved, these reactions are termed “homogeneous” to differentiate them from the heterogeneous reactions discussed earlier.

Although the stoichiometry of gas phase reactions is usually expressed on a molar basis, the stoichiometry of homogeneous gas phase reactions is expressed here on a mass basis:

(4646)

In Equation 46,  is the N by L “homogeneous gaseous species yield matrix”, analogous to the gaseous species yield matrix (ys,j,k) discussed earlier with reference to heterogeneous reactions, and the negative sign on the second term on the LHS is deliberate. The physical meaning of individual entries in yg,j,ℓ is the net mass of gaseous species j produced by reaction ℓ (for positive entries) or consumed by reaction ℓ (for negative entries) per unit mass of gaseous species Aℓ consumed. Note that in writing Equation 46, it is assumed that , i.e. the yield corresponding to gas phase species Aℓ is –1. It is also assumed that for reaction ℓ, the only other negative entry in the yg,j,ℓ matrix is the yield corresponding to gas phase species Bℓ (denoted  in Equation 46). This is why there is a negative sign in front of the second term on the LHS of  Equation 46. All entries in a column of the homogeneous gaseous species yield matrix must add to 0 for mass to be conserved, and this must be strictly obeyed or nonphysical results will occur. The negative sign in the second term on the LHS is deliberate; it results from the convention that a negative entry in yg,j,ℓ causes corresponds to species consumption.

The reaction rate of the ℓth homogeneous gas phase reaction is the destruction rate of gas phase species Aℓ:

(4747)

In Equation 47,  is the destruction rate of gaseous species Aℓ due to homogeneous gas phase reaction ℓ, and [Aℓ] denotes the molar concentration of gaseous species Aℓ:

(4848)

Note that the entry corresponding to species Aℓ in the yg,j,ℓ matrix must be –1 for the first equality in Equation 47 to be true, and this is consistent with the discussion following Equation 46. Porosity () appears in Equation 47 because reaction rates are defined on a per unit volume of solid plus gas (i.e. per unit volume of mixture) basis. For dimensional consistency in Equation 47, the units of Z vary with the specified values of p, q, and b. For p = 1, q = 1, and b = 0, the units of Z are kg–m3/mole2–s.

The creation or destruction of gaseous species j by homogeneous gaseous reaction ℓ is calculated from the homogeneous gaseous species yield matrix (yg,j,ℓ) as:

(4949)

(5050)

Note the analogy to Equations 26 and 27.

Total species formation rates due to homogeneous gaseous reactions are determined by summing over all reactions (note the analogy to Equations 28 and 29):

(5151)

(5252)

The volumetric rate of heat release (or absorption) to the gas phase by homogeneous gaseous reaction ℓ is:

(5353)

where Hℓ is the heat of reaction associated with homogeneous gas phase reaction ℓ. Its units are J of heat released per unit mass of gas phase species Aℓ consumed by homogeneous gas phase reaction ℓ. As with the condensed phase reactions, positive values of Hℓ correspond to an endothermic reaction, and negative values correspond to an exothermic reaction. The subscript g in Equation 53 indicates that any heat absorbed or released is distributed to the gas phase.

## 3.7 Gaseous species total source terms

Gaseous species can be consumed or destroyed in both heterogeneous (solid/gas) and homogeneous (gas/gas) reactions. The source term needed in the gas-phase species conservation equation is obtained by summing the contributions from both types of reaction rates:

(5454)

(5555)

All expressions on the RHS have been defined in Equations 28, 29, 51, and 52.

# 4.0 Governing Equations

## 4.1 One-dimensional (with volume change)

### 4.1.1 Control volume system

The one-dimensional control volume system from which the governing equations are derived is shown schematically in Figure 1. Cell P (“point”) has neighboring cells T (“top”) and B (“bottom”). The interface between cell P and T is denoted t, and the interface between cell P and B is denoted b. The notation T indicates the value of variable  in cell T and t indicates the value of variable  at the interface between P and T. (z)t is the distance from P to T and (z)b is the distance from P to B. The size (height) of cell P is (z)P. For the purpose of calculating source terms, it is assumed that the value of  at the center of a particular cell prevails over the entire cell, but for calculating gradients at cell boundaries it is assumed that  varies in a piecewise linear manner between cell centers. The z dimension increases with depth into the solid, i.e. z = 0 corresponds to the surface and z =  corresponds to the back face. Note that P is located at the center of each cell, except for the two boundary nodes where P is at the edge (because these are “half” cells). The convention used is that  denotes the value of  at time t and  denotes the value of  at time t + t. The governing equations are solved using a fully implicit volume formulation.

Figure 11. Control volume system used for discretization.

A key aspect of Gpyro is that it is specifically designed to accommodate volume change (shrinkage or swelling). This is accomplished by assuming that bulk density is a property of a condensed phase species and that each grid cell (having height z) is permeable to gaseous mass transfer but is impermeable to condensed phase mass transfer and. Thus, if condensed phase bulk density remains constant but gases escape from a cell (e.g., due to pyrolysis) then z decreases to conserve mass (shrinkage). The same shrinkage occurs if no gases escape but the bulk density increases. Conversely, z must increase if the bulk density decreases while no gases escape. It is possible to have a release of gases occur simultaneously with a change in bulk density; in this case, the change in z, if any, depends on mass conservation. An increase in porosity (decrease in bulk density) can occur with no change in z, and this is the usual approach for modeling pyrolysis of charring solids. Due to this formulation, the grid spacing z appears in the conservation equations presented below.

### 4.1.2 Condensed phase mass conservation

The condensed phase mass conservation equation is:

(5656)

Recall that  is the weighted bulk density in a vacuum and that z is the cell size. No convective terms are present because, as mentioned above, an assumption of Gpyro is that condensed phase mass does not cross cell boundaries (only gaseous mass is permitted to cross cell boundaries). Equation 56 says that the product  decreases at a rate proportional to the total volumetric formation rate of all gases from the condensed phase (, kg/m3–s) multiplied by the cell size (z). The destruction (consumption) term on the RHS accounts for the conversion of condensed phase mass to gas phase mass, and  is defined on a per unit volume of gas plus solid (i.e. per unit volume of mixture) basis. Note that the source term on the RHS is the formation rate of gases from the condensed phase and does not include the formation rate of condensed phase species. This is by design since the formation of one condensed phase species from another, in the absence of gas formation, does not change the quantity . Although both  and  could change, their product would be unaffected.

Physically, Equation 56 says that the product of the condensed phase density in a cell and the size of that cell will decrease due to the formation of gases in that cell. For an idealized charring material, the formation of gases will result in a reduction of  (due to an increase in porosity). Similarly, for an idealized thermoplastic material, the formation of gases will result in a reduction of , i.e. the cell shrinks. For “real” (non–idealized) materials, a reduction of  may occur simultaneously with a reduction of . As will be discussed below, both  and  are uniquely determined by solving the condensed phase species conservation equation.

### 4.1.3 Condensed phase species conservation

The condensed phase species conservation equation is similar to the condensed phase mass conservation equation in that it is assumed that no condensed phase species cross cell boundaries:

(5757)

Here,  and  are, respectively, the volumetric total formation rate and volumetric total destruction rate of condensed phase species i. They are defined on a per unit volume of gas plus solid (per unit volume of mixture) basis.

Although it may seem unusual that the grid spacing z appears in Equations 56 and 57, these conservation equations are well–posed. Equations 3, 56, and 57 are M + 2 equations for M + 2 unknowns in each grid cell:  the M species’ mass fractions,, and .

### 4.1.4 Gas phase mass conservation

Unlike Equation 56 (the condensed phase mass conservation equation), the gas-phase mass conservation contains convective terms to account for the flow of volatiles:

(5858)

The storage term on the LHS is similar to that for the condensed phase mass conservation equation, except the density is that of the gas phase (g), and the weighted porosity () appears. Note that the source term is attributed to the conversion of condensed phase mass to gas phase mass. It is equal in magnitude to, but has the opposite sign of, the source term that appears in the condensed phase mass conservation equation. Porosity does not appear in the source term because, as mentioned above, it is calculated on a per unit volume of mixture (solid plus gas) basis. The only inflow and outflow terms are due to convective mass transfer () because the diffusive mass transfer terms cancel.

The gas-phase mass conservation equation is explicitly solved only if the pressure solver is not invoked. When the pressure solver is invoked, the gas-phase mass conservation equation is used to form the pressure evolution equation (see Section 4.1.8).

### 4.1.5 Gas phase species conservation

In the gas phase species conservation equation, both convective and diffusive transport across cell boundaries must be considered. The convective flux of gaseous species j is  and the diffusive flux of gaseous species j is . The gas-phase species conservation equation is:

(5959)

Similar to the condensed phase species conservation equation,  is the total formation rate of gaseous species j and  is the total destruction rate of gaseous species j. These terms are defined on a per unit volume of solid plus gas basis, i.e. on a per unit volume of mixture basis. Note that  and  include contributions from both heterogeneous (gas/solid) and homogeneous (gas/gas) reactions, see Equations 54 and 55.

Although the diffusive term in Equation 142 is written in its general form (), Fickian diffusion is assumed:

(6060)

### 4.1.6 Condensed phase energy conservation

The condensed-phase energy conservation equation is:

(6161)

Changes in kinetic energy, potential energy, and work done on the surroundings have been neglected. All source terms are defined on a per unit volume of gas plus solid (i.e. per unit volume of mixture) basis.

Fourier’s law is used to calculate , the conductive heat flux through the condensed phase:

(6262)

The term  is the volumetric rate of heat release (or absorption) due to condensed phase reactions (see Equation 43). It is obtained by summing over all heterogeneous reactions.

The source term  is the volumetric rate of heat transfer from the condensed phase to the gas phase. Note that , i.e. the rate of heat transfer from the gas to the solid is equal to the negative of the rate of heat transfer from the solid to the gas. Although  is sometimes neglected altogether in pyrolysis models on the basis that it is small, Gpyro provides two different options for calculating . In the first, thermal equilibrium between the gas-phase and the condensed-phase is assumed. This is called the thermal equilibrium formulation. In the second, called two-temperature formulation, the rate of heat transfer from the solid to the gas is explicitly calculated. These two formulations can be summarized as:

(6363)

Note that  is the volumetric rate of heat release due to homogeneous gaseous reactions. Essentially, with the thermal equilibrium approach, any heat released by gas-phase reactions is added directly to the solid instead of the gas.

The topmost equality in Equation 63 is obtained by first solving the gas-phase energy conservation equation (presented later as Equation 155b) for  assuming thermal equilibrium exists between the gaseous and condensed phases, i.e.:

(6464)

where  represents the gas enthalpy evaluated at the condensed-phase temperature, i.e. . Dropping the subscript P notation, making the approximation that , and taking the limit as  and  gives:

(6565)

where use was made of the definition . Note that this equation assumes thermal equilibrium and therefore all temperatures are condensed-phase temperatures. In writing the last equality, it is assumed that  and .

Gpyro contains an option to use the full expression for  (i.e., the middle equality in Equation 65) in thermal equilibrium mode by setting the parameter FULL_QSG = .TRUE. on the &GPYRO_GENERAL line.

In the second equality in Equation 63, hcv is the volumetric heat transfer coefficient (W/m3–K). Gpyro provides the ability to specify a constant (independent of Reynolds number and pore characteristics) volumetric heat transfer coefficient, or calculate it from a Nusselt number correlation of the form Nu = a + b Rec. See the Users Guide for details.

The divergence of the in-depth radiative heat flux vector, treated as a source term, is:

(6666)

In writing Equation 66 it has been assumed that radiation is applied only normal to the exposed surface and interior radiative emission is not accounted for, i.e. the in–depth radiative heat flux is:

(6767)

Note that this formulation accounts only for “one–way” radiation, meaning the penetration of radiation into the solid is calculated, but the emission from interior parts of the solid is not calculated (surface radiant emission is still considered, see Equation 111b). This treatment is different from the solid phase model in NIST’s Fire Dynamics Simulator Version 5 which accounts for two–way radiation and radiation applied other than normal to the surface. Note that if  is not a function of z, then Equation 66 simplifies to .

A decomposing solid frequently consists of a single “layer”. However, Gpyro can accommodate materials with a layered or laminated composition. For layers that are not in perfect thermal contact, the rate of heat transfer between layers calculated as hcrT where hcr is an inverse contact resistance (W/m2–K) and T is the temperature difference between the “back” of one layer and the “front” of the layer that it abuts. At the interface between two layers that are not in perfect thermal contact, the thermal conductivity in Equation 62 is replaced with hcr×z where z is the appropriate distance between cell centers (see Figure 1).

### 4.1.7 Gas phase energy conservation

The sum of gas-phase conductive and diffusive heat fluxes is:

(6868)

In Equation 68, Fickian diffusion has been assumed when calculating the diffusive flux term. By making the approximation that all gases have the same diffusion coefficient (Dj = D) the summation term in Equation 68 will drop out if gD = kg/cpg, i.e. by assuming that kggcpgD (which implies unit Lewis number, Pr = Sc = 1). As mentioned earlier this approximation is made in Gpyro, allowing for considerable simplification:

(6969)

With the simplification in Equation 69, the gas-phase energy conservation equation is:

(7070)

Note that  is the enthalpy of the gas-phase evaluated at the solid-phase temperature, and  is the enthalpy of the gas-phase evaluated at the gas-phase temperature. All source terms are defined on a per unit volume of solid plus gas basis, i.e. on a per unit volume of mixture basis. The term  is the volumetric rate of heat transfer from the condensed phase to the gas phase (Equation 63).

### 4.1.8 Gas phase momentum conservation

An equation describing the evolution of the pressure profile inside the decomposing solid is obtained from the gas phase mass conservation equation (Equation 58). Darcian flow with buoyancy is assumed:

(7171)

where the total pressure P is composed of a background pressure (P0), a flow-induced perturbation pressure (), and the hydrostatic gradient ():

(7272)

Substitution of Equation 72 into Equation 71 gives:

(7373)

It can be seen from Equation 73 that if  = 0 and  that , the desired result.

Although one could conceivably solve for the perturbation pressure  using Equation 73, it is more straightforward to work directly with Equation 71. Consequently, mass flux is related to pressure gradient as:

(7474a)

(74b)

A pressure evolution equation is obtained by replacing g with P using the ideal gas law () and substituting Equation 74 into Equation 58:

(7575)

Equation 75 can be solved for P because all other quantities are calculated elsewhere.

## 4.2 One-dimensional (no volume change)

For the special case where z is fixed, as occurs for charring fuels with no shrinkage or swelling, one can take the limit of the general equations presented earlier as  and  to obtain partial differential equations. These equations are summarized here:

Condensed-phase mass conservation:

(7676)

Gas-phase mass-conservation:

(7777)

Condensed-phase species conservation:

(7878)

Gas-phase species conservation:

(7979)

Condensed-phase energy-conservation:

(8080)

Gas-phase energy conservation:

(8181)

Pressure evolution equation:

(8282)

## 4.3 Two-dimensional (no volume change)

The two-dimensional governing equations are summarized below.

Condensed-phase mass conservation:

(8383)

Gas-phase mass-conservation:

(8484)

Condensed-phase species conservation:

(8585)

Gas-phase species conservation:

(8686)

Condensed-phase energy-conservation:

(8787)

Gas-phase energy conservation:

(8888)

Pressure evolution equation:

(8989)

## 4.3 Three-dimensional (no volume change)

The three-dimensional governing equations are summarized below.

Condensed-phase mass conservation:

(9090)

Gas-phase mass-conservation:

(9191)

Condensed-phase species conservation:

(9292)

Gas-phase species conservation:

(9393)

Condensed-phase energy-conservation:

(9494)

Gas-phase energy conservation:

(9595)

Pressure evolution equation:

(9696)

## 4.4 Zero-dimensional

Gpyro can solve 0D transient equations that represent the mass and species evolution of a “lumped” particle having negligible gradients of temperature species as occurs in idealized thermal analysis experiments. Zero–dimensional transient forms of the governing equations are presented below.

Condensed phase mass conservation:

(9797)

Condensed phase species conservation:

(9898)

Gas phase mass–conservation:

(9999)

Gas phase species conservation:

(100100)

Condensed phase energy conservation:

(101101)

Gas phase energy conservation:

(102102)

Gas phase momentum conservation:

(103103)

Here,  is the linear ramp rate in K/s. The temperature and gas phase mass fractions are taken as the ambient values. Thus, for the 0D transient formulation, T and Yj are specified by the user rather than calculated. Note that Equations 97 and 98 are essentially the same as the condensed phase mass and species conservation equations presented earlier for cell P in the 1D transient model. Thus, in the 1D transient model, each cell behaves as if it was a single thermogravimetric sample. It is not necessary to solve the gas phase mass conservation equation.

The above governing equations can be used to calculate both differential thermogravimetric curves () and thermogravimetric curves (m/m0). A differential thermogravimetric curve is calculated as:

(104104)

Integrating the differential thermogravimetric curve gives the thermogravimetric curve:

(105105)

Equations 104 and 105 are used primarily for simulating TGA experiments, in particular for extracting decomposition kinetics from TGA data using genetic algorithm optimization. Note that it is not necessary to explicitly solve Equation 99 to calculate TGA curves. The initial conditions on Equations 97 and 98 are the same as those applied to the 1D transient governing equations.

# 5.0 Boundary and Initial Conditions

NOTE: There is no longer a “default” boundary condition in Gpyro. This will be updated in the next draft.

The default configuration in Gpyro is a 1D formulation representative of a Cone Calorimeter (or similar) experiment. In Section 5.1, the initial and boundary conditions describing this configuration are presented, and in Section 5.2 more general boundary conditions (specified by the BCpatch worksheet) are presented.

## 5.1 Default – 1D virtual Cone Calorimeter (or similar) experiment

### 5.1.1 Condensed-phase mass conservation

Equation 56, conservation of condensed-phase mass, reduces to an ordinary differential equation for the quantity  in each cell P. Thus, a single initial condition is required in each cell:

(106106)

### 5.1.2 Condensed-phase species conservation

The condensed phase species conservation equation also reduces to an ODE for the quantity  in each cell P. A single initial condition is required in each cell:

(107107)

### 5.1.3 Gas-phase mass conservation

Equation 58, conservation of gas-phase mass, requires one initial condition and one boundary condition. The initial condition in each cell P is:

(108108a)

A single boundary condition is required. Normally, this is specification of the mass flux at the back face, which is zero for a negligibly permeable substrate:

(108b)

Note that the gas-phase mass conservation equation is explicitly solved only if the pressure solver is not invoked. When the pressure solver is invoked, the gas-phase mass conservation equation is used as the basis for soliving

### 5.1.4 Gas-phase species conservation

For each gaseous species j, Equation 142 requires one initial and two boundary conditions. The initial conditions in each cell P are the gaseous species mass fraction at time t = 0:

(109109a)

Two boundary conditions are required for each gaseous species j. Normally, the back face (z = ) is impermeable so there is no flow of volatiles across the back face and the species mass fraction gradient is set to zero:

(1109b)

No boundary condition is required for gaseous species at an outflow boundary if the Peclet number is sufficiently high [8]. However, this condition is not always met, so the diffusive mass flux of gaseous species into or out of the decomposing solid at the front face is approximated using the heat/mass transfer analogy:

(109c)

If the user desires, the effect of blowing on the heat transfer coefficient can be simulated using a Couette flow approximation [9]:

(110110)

where  is the heat transfer coefficient with no blowing, and  is the mass flux of gases at the surface. The user specifies whether the heat transfer coefficient is fixed at , or whether it is reduced due to blowing according to Equation 110.

### 5.1.5 Condensed-phase energy conservation

One initial and two boundary conditions are required on the condensed-phase energy conservation equation. The initial condition in each cell P is:

(111111a)

The front face boundary condition is:

(for )  	(111b)

(for ) 	(111b)

The back face boundary condition is:

(111c)

### 5.1.6 Gas-phase energy conservation

One initial and two boundary conditions are required for the gas-phase energy conservation equation. The initial condition in each cell P is:

(112112a)

The front face boundary condition is:

(112b)

The back face boundary condition is:

(112c)

### 5.1.7 Gas-phase momentum conservation

One initial and two boundary conditions are needed. The initial condition states that the pressure in each cell is atmospheric:

(113113a)

The boundary condition at the front face sets the pressure equal to the atmospheric value:

(113b)

The pressure gradient at the back face is set to zero to give an impermeable back face:

(113c)

## 5.2 Generalized boundary conditions

This section presents generalized boundary conditions that provide more flexibility than those discussed in Section 5.1. However, the initial conditions remain the same as in Section 5.1.

General boundary conditions are specified over a number of surface “patches”. In a 1D simulation, the only available patches are the front face (z = 0) and the rear face (z = ). However, in a 2D simulation, the user may specify different boundary conditions on the edges of the rectangle corresponding to the computational domain. As described in the Users’ Guide in greater detail, the user identifies the location of each boundary condition patch by specifying the coordinates x1, x2, z1, and z2. For example, if the user specifies x1 = x2 = 0.00 m, z1 = 0.01 m, and z2 = 0.02 m, then Gpyro will apply the specified boundary condition at the plane x = 0.00 m for 0.01 m  < z < 0.02 m.

In the following sections, the notation s is used to denote the surface “patch” over which the boundary condition is applied. Gradients are written  where n denotes the coordinate direction that is normal to the patch (i.e.,  for a boundary applied to a constant-x plane, and   for a boundary applied to a constant-z plane).

### 5.2.1 Condensed-phase mass conservation

No boundary conditions are required on Equation 56, conservation of condensed-phase mass. The initial condition is already given as Equation 106.

### 5.2.2 Condensed-phase species conservation

No boundary conditions are required on the condensed phase species conservation equation. The initial condition has already been given as Equation 107.

### 5.2.3 Gas-phase mass conservation

The initial condition on Equation 58, conservation of gas-phase mass, has already been given as Equation 108b. The single required boundary condition is specification of the mass flux at the back face:

(114114)

Recall that the gas-phase mass conservation equation is explicitly solved only if the pressure solver is not invoked. When the pressure solver is invoked, the gas-phase mass conservation equation is used to form the pressure evolution equation (see Section 4.1.8).

### 5.2.4 Gas-phase species conservation

The initial condition on the gas-phase species conservation equation has already been given as Equation 109a. Two types of gaseous species boundary conditions are available: fixed value and fixed gradient.

A fixed value boundary condition should be applied at inlet boundaries, i.e. the user must specify the composition (mass fractions) of the incoming flow:

(115115)

Here, denotes the mass fraction of species j in the incoming flow.

At non-inlet boundaries, a fixed gradient boundary condition can be applied by specifying hm (a mass transfer coefficient, kg/m2-s) and  (the ambient value of each species j). The gaseous species mass fraction gradient at the boundary is then calculated from the following relation:

(116116)

Note that a no flux (zero gradient) boundary condition can be specified by setting hm = 0.

### 5.2.5 Condensed-phase energy conservation

The initial condition on the condensed-phase energy conservation equation has already been given as Equation 111a.

Two types of boundary conditions may be applied to the condensed-phase energy equation: fixed value (specified temperature), and a radiative-convective balance.

The temperature at a boundary can be directly specified:

(117117)

where  (K) is the specified temperature.

If the user desires a radiative-convective balance, then  should be set to a negative number and the user should specify  (externally applied heat flux, W/m2), (convective heat transfer coefficient, W/m2-K), and  (ambient temperature, K). The following boundary condition is then applied:

(for )  	(118118a)

(for ) 	(118b)

If the user specifies RERADIATION = .FALSE. for a particular patch, then the rightmost term on the RHS of Equation 118 is set to zero.

### 5.2.6 Gas-phase energy conservation

The initial condition on the gas-phase energy conservation equation has already been given as Equation 112a.

As with the condensed-phase energy equation, two types of boundary conditions may be applied to the gas-phase energy equation: fixed value (specified temperature), and a radiative-convective balance.

The gas temperature at a boundary can be directly specified:

(119119)

where  (K) is the specified gas temperature.

Alternatively, if a radiative-convective balance is desired, then  should be set to a negative number and the user should specify  (externally applied heat flux, W/m2), (convective heat transfer coefficient, W/m2-K), and  (ambient temperature, K). The following boundary condition is then applied:

(120120)

### 5.2.7 Gas-phase momentum conservation

The initial condition on the gas-phase momentum conservation equation has already been given as Equation 113a.

Two types of boundary conditions are available: fixed value (specified pressure), and fixed gradient (specified mass flux).

If a fixed pressure boundary condition is desired, then the user specifies the pressure at the boundary patch and the following boundary condition is applied:

(121121)

Alternatively, the user may specify a mass flux at a boundary patch, which is implemented numerically by fixing the pressure gradient necessary to give this mass flux:

(122122)

## 5.4 Geometry masking and initial conditions

In next draft.

## 5.5 FDS-Gpyro coupling

In next draft.

# 6.0 Numerical Solution Methodology

In the previous sections, the governing conservation equations were derived and the treatment of the source terms was presented. In this section, it is explained how these equations are solved numerically. The equations are presented in one-dimensional form for brevity and because extension to two-dimensions is straightforward. However, in Section 6.11 the two-dimensional form of the condensed-phase energy conservation is presented as an example.

## 6.1 Source term decomposition

In Gpyro, net source terms are partitioned into positive and negative components for numerical convenience. A net source term is the sum of formation (or production) rates and destruction (or absorption) rates. The advantage of decomposing net source terms into its positive and a negative parts is that quantities which, from physical considerations, must remain positive can be prevented from inadvertently becoming negative (see Patankar [8] pg. 145). This is explained in greater detail below.

Consider a source–dominated conservation equation of the same form as Equation 57 where the net source term  is filtered into its positive and negative components:

(123123)

Here, , , and . Now multiply the second term on the RHS by P/P and rearrange:

(124124)

In practice, to prevent division by zero, this is implemented as:

(125125)

where  is a very small number. Note that all quantities on the RHS are greater than or equal to zero. Thus, p can approach zero, but it can never become negative. Treating mass and species conservation in this way was found to be critical for ensuring a stable solution.

## 6.2 Convective-diffusive solver

As will be shown below, the governing equations are discretized in the general form:

(126126)

For conservation equations involving convection and diffusion, the general form of the aB and aT coefficients in Equation 126 are:

(127127a)

(127b)

where the following definitions apply:

(127c)

(127d)

(127e)

(127f)

(127g)

(127h)

In Equation 127,  is a diffusion coefficient (its units and physical significance depends on the equation being solved), P is the cell Peclet number (not pressure), and F is the convective mass flux. The function A(|P|) is given by Patankar [8] in Table 5.2 and repeated below in Table 2:

Table 22. The function A(|P|) from Patankar [8].

|  | Scheme | Formula for A(¦P¦) |
| --- | --- | --- |
| 1 | Central difference | 1 – 0.5¦P¦ |
| 2 | Upwind | 1 |
| 3 | Hybrid | max(0, 1 – 0.5¦P¦) |
| 4 | Power law | max(0, (1 – 0.1¦P¦)5) |
| 5 | Exponential | ¦P¦ / [exp(P) – 1] |

Since the diffusion coefficient is required at the interface between cells, the interface value is calculated using the harmonic mean between cell P and the adjacent cells (B or T):

(128128a)

(128b)

## 6.2 Tri–diagonal matrix (Thomas) algorithm

Writing the governing equations in the form of Equation 126 makes it possible to use the efficient recursive algorithm known as the Thomas (or tri–diagonal matrix) algorithm. First, rewrite Equation 126 as:

(129129)

i.e., ai is aP, bi is aB, ci is aT, and di is b. Next, define:

(130130a)

(130b)

The Thomas algorithm begins by calculating P1 and Q1 (where the subscript 1 denotes that these values refer to the first cell):

(131131a)

(131b)

In Equation 131b, 0 is the ambient value of  (the subscript 0 denotes the 0th cell). Next, Equations 130a and 130b are used to calculate Pi and Qi for i = 2 to n where n is the number of cells. The value of  in cell n is calculated as:

(132132)

In Equation 132, n+1 represents the value of  flowing into the back face (or the ambient value of  at the back face). With  known, the remaining n –1  values are calculated using the relation:

(133133)

Some clarification is warranted regarding the coefficients in cell 1 and cell n. Since the TDMA solver is used to solve four separate conservation equations (gaseous species, condensed and gaseous energy, and gaseous momentum) the meaning of the boundary coefficients for each of these conservation equations is discussed separately below.

A fixed value boundary condition at the front face is implemented by specifying a1 = 1, b1 = 0, c1 = 0, and d1 = . Similarly, a fixed value boundary condition at the rear face is implemented by specifying an = 1, bn = 0, cn = 0, and dn = .

A specified mass flux is achieved by setting the pressure gradient to a desired value. The discretized equations at the front and back faces are:

(134134a)

(134a)

It follows from Equation 134a that  and from Equation 134b that . Note that flow into the back face corresponds to a negative value of  and flow into the front face corresponds to a positive value of . The discretized pressure evolution equation at the front and back face is:

(135135a)

(135b)

Setting bn = 0 and c1 = 0, and substituting Equation 134 into Equation 135 gives expressions for a1 and an:

(136136a)

(136b)

A forced mass flux boundary condition is implemented by setting the value of a1 or an per Equation 136.

In the condensed phase energy equation, the convective and radiative losses (or gains) are converted to volumetric source terms by dividing a flux by the boundary node grid size. For example, the convective loss or gain at the front face is accounted for by adding a source term of magnitude  to the source term in cell 1, where . Rather than explicitly setting the temperature gradient at the front and back faces, the coefficients c1 and bn are set to zero, and volumetric source terms are added to cells 1 and n. Numerically, this is equivalent to explicitly specifying the gradient at the front and back faces, but it is conceptually easier to implement.

In the gas phase species conservation equation, the meanings of c1 and bn require careful interpretation. Tracing backward from Equation 129 to Equation 126 to Equation 127, it is seen that  and . The usual definition of  and  are  and . However, in cell 1 and cell n, the heat/mass transfer analogy is used to calculate the value of  and  as:

(137137a)

(137b)

The boundary conditions on the gas phase energy conservation equation are implemented in a way that is directly analogous to the gas phase species conservation equation.

## 6.3 Convergence criteria and relaxation

Due to nonlinearity in the governing equations, a fully–implicit iterative numerical solution is used. Convergence is achieved when the solution in every grid cell chances by less than some predetermined tolerance. Relative tolerances are used to check convergence of the gas phase and condensed phase species equations. For example, the condensed phase species conservation equation is converged when the following is true in every grid cell:

(138138)

where rtol is a user–specified relative tolerance, and  is a small number to prevent division by zero. Note that Yi,old is the mass fraction at the previous iteration (not the previous time step).

For the other equations, an absolute tolerance is used as the convergence criterion. For example, the pressure evolution equation is converged when the following is true in every grid cell:

(139139)

where atol is a user–specified absolute convergence criterion. Again, Pold is the pressure at the previous iteration, not the previous time step.

In order to reduce the possibility of divergence between iterations, solution relaxation is implemented by replacing the coefficients aP and b in the discretized governing equations with aP,relax and brelax, as shown in Equation 140:

(140140a)

(140b)

(140c)

where  is the user–specified relaxation parameter () and P,old is the value of  from the previous iteration (not the previous time step). Note that when  = 1, aP,relax = aP and brelax = b. For simplicity, the same value of  is applied to all equations and  is static, meaning that it is not dynamically determined during a simulation. Normally,  = 1 and solution relaxation via Equation 140 would be invoked only when the solution fails to converge within a pre–determined number of iterations.

## 6.5 Condensed phase species and mass conservation

This section outlines the conservation approach for condensed phase species in a computational cell. The method is based on explicit time integration of the species mass balance and includes a correction mechanism to ensure physical consistency.

### 6.5.1 Governing Equation and Explicit Integration

The conservation of total condensed mass mtot,P ​ in a computational cell P is governed by the following ordinary differential equation (ODE):

dmtot,Pdt=dρ ΔVpdt=-ωfg'''ΔVp

The conservation of a condensed phase species A is given by:

dmA,Pdt=d (YAρ ΔV)p dt=ωnet,A'''ΔVp

These equations describe the time evolution of the species mass mA,P within cell P, which has a volume ΔVP ​. Since these are local ODEs, no boundary conditions are required, only initial conditions (see Equation 107XX).

Using an explicit formulation of the source term, the total mass at the next time step is computed as:

mtot,Pn+1=mtot,Pn-ωfg,P'''n+1ΔVpnΔt

And the masse of each condensed species A is computed as :

mA,Pn+1=mk,Pn+ωnet,A,P'''n+1ΔVpnΔt

To avoid numerical instabilities that may arise as the cell volume ΔVapproaches zero, such as when a charring polymer like PMMA is completely consumed, a stabilization technique is applied. If the ratio of the current cell volume to its initial volume falls below a user-defined threshold, the cell is considered "consumed" at the next time step. In this state, all reaction rates within the cell are set to zero, and the cell is treated as inert.

This approach allows the solver to retain extremely thin residual layers without divergence, ensuring numerical stability even in the final stages of material degradation.

### 6.5.2 Mass Conservation and Correction Mechanism

To maintain physical realism, the mass of each species A must remain non-negative:

mA,pn+1≥0

If this condition is violated, a correction is applied in the cell, in which all relevant reaction rates are adjusted. The subscripts err and corr are used to denote the erroneous (overestimated) and corrected values, respectively. The corrected net destruction rate is defined such that the mass of species A at the end of the time step reaches zero:

ωnet,k,p''' corr  n+1=mk,pnΔt ΔVpn

This correction is performed by adjusting the overestimated destruction rate of species A, effectively removing the excess mass loss (i.e -mA,Perr n+1 ​). The corrected destruction rate is then given by:

ωd,A,P''' corrn+1= ωd,A,P''' errn+1+mA,Pn+1, errΔVpnΔt

The correction coefficient (Kcorr<1) is then introduced:

Kcorr=ωd,A,P''' corrn+1ωd,A,P''' errn+1

This coefficient is applied to all reactions k involving the destruction of species A :

ωk,Pcorr=Kcorr*ωk,Perr

Similarly, the production rates of all condensed products (e.g., species B) and gaseous products (e.g., species g) resulting from the affected reactions are also scaled by Kcorr :

ωf,k,B,P''' corr=Kcorr*ωf,k,B,P''' err

ωf,k,g,P''' corr=Kcorr*ωf,k,g,P''' err

The overestimated production of condensed species B and gaseous species g is then subtracted from their respective total production rates and net source terms. This ensures that the excess destruction is consistently removed from the system:

ωf,g,P''' corr=ωf,g,P''' err-(ωf,k,g,P''' err-ωf,k,g,P''' corr)

ωnet,g,P''' corr=ωnet,g,P''' err-(ωf,k,g,P''' err-ωf,k,g,P''' corr)

However, this correction may reduce the production of other species (such as species B), potentially causing their mass to become negative. In such cases, the correction process is applied recursively. If the number of recursive iterations exceeds twice the number of condensed species, the time step is considered non-convergent and must be reduced.

If the correction procedure converges, both the total mass conservation and the conservation of each condensed species are recalculated (see Equation XX), since all formation and destruction rates—as well as the total production of gaseous species—may have been modified (see Equation XX).

### 6.5.3 Final Mass Fraction Calculation and deformation

Once the updated mass of all condensed species and the total mass mtot is known, the mass fraction of species A is computed as:

YA,Pn+1=mA,Pn+1mtot,Pn+1

Next, the weighted bulk density of the cell is calculated using the mass fractions and densities of all condensed species:

ρPn+1=A=1MYA,Pn+1ρA,P

Finally, the updated local cell volume is determined from the total mass and bulk density:

ΔVpn+1=mtot,Pn+1ρPn+1

Note that at present, mesh deformation is only implemented in 1D (ΔV=ΔZ). In 2D and 3D simulations, no deformation model is currently available. In these cases, Equation XX is not used, and the cell volume is assumed to remain constant. Implementing deformation in 2D and 3D is significantly more complex due to mesh distortion, loss of orthogonality, and the increased difficulty in accurately computing fluxes between adjacent cells.

This solver technique ensures that the species solver remains both conservative and explicit, while enabling the accurate resolution of condensed species mass fractions, bulk density, and mesh deformation.

## 6.6 Gas phase mass conservation

The gas phase mass conservation equation has been given as Equation 58:

This equation (at least in the above form) is explicitly solved only when the pressure solver is not used to calculate . If this is the case, then the gas phase mass conservation equation is used to calculate  assuming that all volatiles generated in–depth escape instantaneously with no resistance to mass transfer, i.e. the mass flux at the top of cell P is calculated as:

(141141)

Thus, provided that  is known at the back face,  can be determined at any point in the decomposing material by applying Equation 141 in every cell starting at the back face and moving toward the front face. Note that due to the coordinate system used (z increases with depth into the decomposing solid)  is negative when the flow direction is toward the surface.

## 6.7 Gas phase species conservation

The final form of the gas phase species conservation equation solved by Gpyro is obtained by multiplying the gas phase mass conservation equation (Equation 58) by Yj,P and subtracting the result from Equation 59. This results in the following equation:

(142142)

The discretization of and numerical solution of Equation 142 follows Patankar [8]. Begin by defining the following:

(143143a)

c

(143e)

Substituting Equation 143 into Equation 142 (and multiplying  by Yj,P/Yj,P) gives:

(144144)

Following Patankar [8] Equation 5.54–5.55:

(145145a)

(145b)

Substituting Equation 145 into Equation 144 gives:

(146146)

Define:

(147147a)

(147b)

The final form of the discretized equation is obtained by substituting Equation 147 into Equation 146:

(148148)

The coefficients aB and aT are obtained from Equation 127 by substituting .

## 6.8 Condensed phase energy conservation

This section presents the numerical strategy used to solve the energy conservation equation in the condensed (solid) phase. The approach is based on the resolution of the specific enthalpy using a finite volume discretization combined with an Alternating Direction Implicit (ADI) scheme. The resulting tridiagonal systems are efficiently solved using the Thomas algorithm. Once the enthalpy field is updated, the corresponding temperature is recovered through an inversion procedure based on the thermodynamic properties of the condensed species.

### 6.8.1 Spatial discretization

As a reminder, the energy conservation equation in a cell P, with a controlled volume ΔVP can be compactly expressed in the following form:

dhmpdt=∂V k∇T. n dSp +QsΔVp

where Qs represents the set of source terms, including the contributions from reaction heat, radiative absorption, gas-solid heat exchange, and sensible enthalpy changes:

| Qs=Qs-g+k=1KQs,k-δqrδz+i=1M ωfi''-ωdi''hi | (1) |
| --- | --- |

To facilitate numerical solution, the final form of the condensed phase energy equation that is solved by Gpryo is obtained by multiplying the condensed phase mass conservation equation (Equation 56XX) by hp and subtracting the result from Equation 61XX :

mpdhpdt=∂V k∇T. n dSp +QsΔVp++hωfg'''ΔVp

In this formulation, only the diffusive term requires spatial discretization, as the source terms and time derivatives are treated locally. The discretization approach depends on the dimensionality of the problem; the expressions provided here are written in the general three-dimensional (3D) form.

∂V k∇T. n dSp=IOR∈{T,B,E,O,N,S}FIOR=FN+FS+FW+FE+FT+FB

The terms FN,FS,FW,FE,FT and  FB represent, respectively, the heat fluxes through the north (+y), south (-y), west (-x), east (+x), top (+z), and bottom (-z) faces of the cell. These fluxes can be expressed as functions of enthalpy. For instance, considering the bottom direction  FB, the expression is as follows:

| FB=qB''ΔxΔy=-kBdTdzΔxΔy | (2) |
| --- | --- |

Substituting the relationship between temperature and enthalpy becomes (Equation 154XX):

| FB=-kcBδhδzB -A=1MhAδYδzB ΔxΔy | (3) |
| --- | --- |

Using a finite difference approximation for the gradients, the expression:

| FB=-kcBhP-hBδzBΔxΔy -A=1MhA,PkcB YA,P-YA,BδzBΔxΔy | (4) |
| --- | --- |

Here:

- kB and cB are respectively the thermal conductivity and the specific heat capacity calculated at the interface between the cell and its bottom cell.

- h is the specific enthalpy, and hA,P  is the sensible enthalpy of the species A at the cell P.

- YA is the mass fraction of the species A.

- M the Number of solid species

Similar expressions apply for the other fluxes FN,FS,FO,FE, and  FT, considering their respective directions and gradients.

Finally, the spatially discretized form of Equation XX is written as:

mpdhPdt=FN+FS+FW+FE+FT+FB+QsΔVP+hωfg'''ΔVp

### 6.8.2 Temporal Discretization

All terms in the solid-phase energy conservation equation are discretized in time. The choice of whether the spatial gradients (i.e., diffusive fluxes) are treated explicitly or implicitly depends on the solver direction, as explained later. The general form of the discretized energy equation in a cell P is:

A0hpn+1=IOR∈{T,B,E,O,N,S}AIOR(hIOR-hP) +B'

Where the coefficients are defined as:

| AN=kcNδyNΔxΔz ; As=kcsδysΔxΔz ; AE=kcEδxEΔyΔz AT=kcTδzTΔxΔy ; AB=kcBδzBΔxΔy ; AW=kcWδxWΔyΔz | (5) |
| --- | --- |

And

| A0=ρΔVn+1Δt | (6) |
| --- | --- |

| B'=ρΔVn+1Δthpn+QsΔVn+1+hPn ωf,g'''ΔVn+1+IOR∈{T,B,E,O,N,S}A=1MhA,PnkcIOR YA,P-YIORδzIORΔSIORn+1 | (7) |
| --- | --- |

The temporal discretization of the diffusive fluxes depends on the sweep direction of the solver. The idea is to write the system in a tridiagonal form, which can be efficiently solved using the Thomas algorithm. At each time step, only the fluxes aligned with the current sweep direction are treated implicitly, while the others are handled explicitly. If the sweep direction is Z, Top and Bottom fluxes are implicit. If X, East and West are implicit. If Y, North and South are implicit.

For example, if the solver direction is Z, the final equation becomes:

| AhPn+1=AThTn+1+ABhBn+1+B | (8) |
| --- | --- |

Where :

| B=IOR∈{E,O,N,S}AIOR(hIORn-hPn)+B' | (9) |
| --- | --- |

And

| A=A0+AT+AB | (10) |
| --- | --- |

This directional treatment ensures that all fluxes in a given direction are handled consistently (either all implicitly or all explicitly), making the scheme physically conservative. This is a key improvement over the previous solver in Gpyro version 0.8, which did not guarantee this consistency.

Considering stability constraint, In the direction selected for the implicit sweep, the numerical scheme is unconditionally stable due to the fully implicit treatment of the diffusive fluxes. This property allows for large time steps without compromising numerical stability. In contrast, the directions treated explicitly are subject to a stability constraint. To ensure stability in these directions, the time step must satisfy the following CFL-like condition. For example, if the X-direction is treated explicitly:

Δt<minP∈Gρcp Δx22kp

By default, the solver changes the implicit direction at each time step in a cyclic manner (typically in 3D: Z, then X, then Y). This strategy ensures a balanced treatment of all spatial directions over time. However, the user has the option to override this behavior and impose a fixed sweep direction. This is particularly advantageous in most 3D pyrolysis problems, where decomposition often occurs predominantly along the Z-axis, which is usually discretized more finely. In such cases, keeping Z as the implicit direction removes the stability constraint in that axis and allows for significantly larger time steps, improving computational efficiency.

It is also worth noting that in one-dimensional configurations, the system is always fully implicit, and therefore unconditionally stable by construction.

### 6.8.3 Getting Temperature from Enthalpy

At the end of a time step, the mass–weighted enthalpy (h=ΣYihi) is known but the temperature is not known. To extract the temperature from the enthalpy, the following equation must be solved for T:

i=1MYihiT=h

where Yi and  are known. From Equation 10, the sensible enthalpy of species i is:

hiT= Td Tcb,iθ+cm,iθ dθ =c0,inc,i+1TTTrnc,i-TdTdTrnc,i+ΔHm,i2erfT-Tm,i2σm,i2-erfTd-Tm,i2σm,i2 =c0,iTnci+1Trncinc,i+1-c0,iTdnci+1Trncinc,i+1+ΔHm,i2erfT-Tm,i2σm,i2-erfTd-Tm,i2σm,i2

(149149)

Equation 149 can also be written as:

hiT=AiTBi-Ci+DierfT-EiFi-Gi	(150150a)

Ai=c0,inc,i+1Trnc,i	(150b)

Bi=nc,i+1	(150c)

Ci=c0,inc,i+1Tdnc,i+1Trnc,i	(150d)

Di=ΔHm,i2	(150e)

Di=ΔHm,i2	(150f)

Fi=2σm,i2	(150g)

Gi=ΔHm,i2erfTd-Tm,i2σm,i2	(150h)

Thus, the following equation must be solved for the unknown temperature T:

i=1MYiAiTBi-Ci+DierfT-EiFi-Gi=h	(151151)

or putting the unknown temperature on the LHS and the known terms on the RHS:

i=1MYiAiTBi+YiDierfT-EiFi=h+i=1MYiCi+Gi	(152152)

For the special case of no melting, Equation 152 becomes:

i=1MYiAiTBi=h+i=1MYiCi	(153153)

Equation 152 or Equation 153 is solved for the unknown temperature T via the Newton-Raphson method.

For the special case of all species having constant (temperature invariant) specific heat capacities (i.e. ) and no melting, Equation 151 can be simplified by substituting in Equations 150b and 150d, leading to the following simple equation that can be used to extract temperature from enthalpy:

(154154)

The reduction in CPU time offered by using Equation 154 is substantial because Newton iteration is computationally expensive.

## 6.9 Gas phase energy conservation

The final form of the gas phase energy equation that is solved by Gpyro is obtained by multiplying the gas phase mass conservation equation (Equation 58) by (hg)P and subtracting the result from the gas phase energy equation (Equation 70):

(155155a)

By default, Gpyro assumes that gases produced by condensed-phase pyrolysis are generated at the gas temperature, which amounts to replacing  with  in Equation 155a. This approximation permits the simplification  because . Under these approximations, Equation 155a reduces to:

(155b)

However, the user can specify GASES_PRODUCED_AT_TSOLID = .TRUE. in the &GPYRO_GENERAL namelist group to recover the more general Equation 155a.

Equation 155b can be written as:

(156156a)

(156b)

Now split the source term into positive and negative parts:

(157157a)

In the two-temperature formulation, S+ and S- are calculated as:

(157b)

(157c)

It has been assumed in writing Equations 157b and 157c that  is positive, i.e. there are no endothermic homogeneous gas phase reactions.

In the thermal equilibrium formulation, the gas-phase energy equation is not explicitly solved so it is not necessary to calculate S+ and S-.

As with the gaseous species conservation equation, discretization and numerical solution of Equation 156 follows Patankar [8]. Begin by defining the following:

(158158a)

(158b)

(158c)

Substituting Equation 157 and Equation 158 into Equation 156 gives:

(159159)

Following Patankar [8] Equation 5.54–5.55:

(160160a)

(160b)

Substituting Equation 160 into Equation 159, and multiplying the negative part of the source term by  gives:

(161161)

Define:

(162a)

(162162b)

After substituting Equation 162 into Equation 161, the final form of the discretized equation becomes:

(163163)

The coefficients aB and aT are obtained from Equation 127 by substituting .

## 6.10 Gas phase momentum conservation

Begin with Equation 75:

Replace the pressure gradient with its discrete approximation:

(164164a)

(164b)

Expand the first term on the LHS as:

(165165)

Substituting Equations 164 and 165 into Equation 75 gives:

(166166)

Define:

(167a)

(167167b)

(167c)

Substitute Equation 167 into Equation 166:

(168168)

Distribute multiplication and combine like terms:

(169169)

Define:

(170170a)

(170b)

Substituting Equation 170 into Equation 169 gives:

(171171)

## 6.11 Extension to two dimensions

The equations presented above are given in one dimensional form. Extension to two dimensions is straightforward. As an example, the condensed-phase energy conservation equation is extended to two dimensions in this section. This section should be compared to Section 6.8.

The two-dimensional analog to Equation Erreur ! Source du renvoi introuvable. is:

(172172)

Where e and w denote the east and west interfaces between cells.

Equation 172 can be written as:

(173173a)

(173b)

Where  is calculated per Equation 63. For numerical convenience, the source term (Equation 173b) is split into its positive and negative components:

(174174a)

(174b)

(174c)

To facilitate use of the efficient Thomas algorithm/TDMA solver to solve Equation 173, the heat conduction terms must be expressed in terms of enthalpy, i.e.:

(175175a)

(175b)

(175c)

(175d)

Substituting Equations 174 and 175 into Equation 173 gives:

(176176)

Define:

(177177a)

(177b)

(177c)

(177d)

(177e)

(177f)

(177g)

(177h)

(177i)

The coefficients in the numerator of Equations 177b –177i are calculated using harmonic means (see Equation 128). Substitute Equation 177 into Equation 176:

(178178)

Define:

(179179a)

(179b)

The final form of the conservation equation is obtained from Equation 178 after distributing multiplication, multiplying the negative part of the source term by  to ensure that all coefficients remain positive, combining like terms, and substituting Equation 179:

(180180)

The two dimensional governing equations are solved with a line-by-line TDMA where alternating “sweeps” are made in the x and z directions. In a z-direction sweep, the  and  terms are treated as source terms, i.e. the discretization equations are:

(181181a)

(181b)

Equation 181a is then solved with the method described in Section 6.2.

In an x-direction sweep, the  and  terms are treated as source terms, i.e. the discretization equations are:

(182182a)

(182b)

Equation 182a is then solved with the method described in Section 6.2.

# 7.0 Material Property Estimation

To apply Gpyro to real–world combustible solids, the model input parameters required to characterize these materials must be established. Unfortunately, we have a limited ability to extract the required material properties from laboratory experiments such as the Cone Calorimeter, Fire Propagation Apparatus, thermogravimetric analysis, and differential scanning calorimetry. This is the biggest barrier preventing increased application of research–type pyrolysis models for practical purposes.

## 7.1 Background

Although techniques have been developed to estimate several material fire properties from laboratory fire tests, there is a disconnect between the material properties needed for numerical pyrolysis modeling and those properties that can be determined with existing techniques. This chapter presents a methodology that uses an automated optimization method based on a genetic algorithm (GA) to extract the material properties needed for numerical pyrolysis modeling from laboratory experiments, particularly traditional bench–scale flammability tests (cone calorimeter, FPA) and small–scale thermal analysis (TGA). This chapter is based on a paper published in Fire Safety Journal [10].

Genetic algorithms are a class of search tools that use the principles of Darwinian evolution to seek an optimal solution to a problem. In contrast to many classical search methods, genetic algorithms can handle nonlinear problems and search spaces having high dimensionality, i.e. a large number of adjustable parameters. The proposed methodology involves simulating a laboratory test or tests with the generalized pyrolysis model described earlier, and using a genetic algorithm to locate a set of model parameters (or “material properties”) that provide optimal agreement between the model calculations and the experimental data, subject to the constraints imposed by the model and the experiments.

Optimization techniques based on genetic algorithms are “embarrassingly parallel”, meaning that they can be easily divided into a large number of parallel tasks that can run on a separate CPU or core. The actual computer implementation of the genetic algorithm optimization methodology presented here has been parallelized using Message Passing Interface. The parallel code has been successfully run on Linux computer clusters at UC Berkeley using more than 60 CPUs. The code is massively parallel, meaning that it can use as many CPUs or cores as are available in a networked computer cluster.

The material properties found by the genetic algorithm will be close to the “actual” values provided: 1) the experimental measurements are sufficiently accurate and contain enough information to establish a unique set of model parameters, 2) the pyrolysis model is an adequate representation of the underlying physical processes, and 3) the specified boundary and initial conditions are an accurate representation of the experimental configuration. This approach is self–consistent in the sense that the same model that is used to estimate the material properties from small–scale experimental data can be coupled to a CFD code to model large–scale fire development.

The pyrolysis model described earlier can be used to simulate both traditional flammability tests (cone calorimeter, FPA) as well as TGA experiments. When samples are instrumented with thermocouples, cone calorimeter and FPA experiments can provide information regarding a material’s thermal properties, but probably cannot be used to establish detailed kinetics mechanisms. In comparison, TGA provides no information about a material’s thermal properties, but provides detailed information about a material’s decomposition kinetics. Differential scanning calorimetry or differential thermal analysis could potentially be used to infer specific heat capacity and heats of reaction. Due to the different types of information that can be obtained from flammability tests and thermal analysis experiments, both are potentially useful for material property estimation for pyrolysis modeling.

## 7.2 Genetic algorithm for estimating material properties

In this section, a methodology is presented that can be used to estimate unknown model input parameters from experimental data. It involves using a genetic algorithm to find a set of model input parameters that provides near–optimal agreement between the calculations of the generalized pyrolysis model and experimental measurements from a laboratory test. However, this inverse problem is ill–posed. That is, the solution (the set of model input parameters) is not necessarily unique and it is not necessarily stable to small changes in the input data. The proposed methodology is heuristic, meaning that the final solution is not necessarily the absolute optimal solution; however, the algorithm will find a solution that gives a near–optimal prediction of the experimental data given the constraints of the underlying physical model. That is, the results are almost as good as those that would be obtained with the optimal solution.

Genetic and evolutionary algorithms have been previously used in engineering. They have already been applied to the optimization of combustion and chemical kinetics problems, including heterogeneous [11] and homogeneous [12] reaction mechanisms. However, since the use of a GA to estimate solid phase material properties from fire test data is a specialized application, the algorithm used here is described in detail below. It is presented within the context of using cone calorimeter or FPA–type experiments for property estimation, but the basic methodology also applies to thermal analysis experiments (TGA, DSC, DTA). The reader desiring a general treatment of genetic or evolutionary algorithms is referred to several books covering the topic [13–15].

### 7.2.1 Initial population

The search process is initialized by randomly generating several candidate solutions (for the idealized charring material described earlier, this would be a vector of eight real numbers corresponding to kv, cv, Z, E, Hvol, kc, c, and cc). Each candidate solution is referred to as an individual or a chromosome, and the terms are used interchangeably throughout the genetic algorithms literature. A single parameter of an individual (or chromosome) is referred to as a gene, for example the numerical value of the virgin phase thermal conductivity is one gene. The entire group of candidate solutions is referred to as a population. Typical population sizes range from tens to hundreds of individuals. Populations constantly evolve to form subsequent generations;  the initial population is the first generation, the offspring of the first generation make up the second generation, and so on.

This chapter adopts a separate nomenclature from the rest of the dissertation. Let {a1, a2 . . .  an} denote the n parameters, or genes, that make up an individual, or chromosome. Here, a1 may be associated with kv, a2 with cv, and so on. For certain variables that may take on values over several orders of magnitude, it is convenient to have the gene represent the logarithm of that variable. Let {A1, A2 . . . AN} denote the N individuals that make up the population. The nomenclature AI() is used to denote the Ith individual of generation . Similarly,  represents the jth gene of the Ith individual of generation .

Begin by generating an initial population:

(183183)

The parameters aj,max and aj,min are the user–specified upper and lower bounds of each variable; all parameters are constrained by these values throughout the evolution process. In Equation 183,  is an N by n matrix of random real numbers distributed uniformly on the interval [0, 1]. The indices I and j are cycled from 1 to N and 1 to n, respectively.

### 7.2.2 Fitness

Next, the fitness of each individual is evaluated. Here, fitness is a measure of how well the candidate solution matches the experimental data. For demonstration purposes, it is assumed in the following discussion that the experimental data is obtained from a flammability test such as the cone calorimeter. It is also assumed that at a minimum, surface temperature and mass loss rate histories are available. Extension to additional measurements (e.g., in–depth thermocouple temperature measurements) or experimental configurations (e.g., TGA) is straightforward. The residuals that measure the level of agreement between the measured surface temperature/mass loss rate histories and the predictions of individual I are:

(184184a)

(184b)

Here, a subscript “exp” denotes the experimental data, and a subscript “try” denotes the trial solution generated by a certain set of parameters contained in the individual. The parameter  is a user–specified small number that prevents the fitness from approaching infinity as the first term in the denominator approaches zero, and  is a user–specified exponent. Equation 184 is written as a summation, rather than an integral, because experimental measurements are obtained at discrete time intervals. In Equation 184a, the summation is performed for surface temperature measurements/predictions only before the experimentally determined ignition time, but in Equation 184b the summation for the mass loss rate measurements/predictions is carried out over the entire duration of the test. This choice was made because the accuracy of surface temperature measurement becomes more questionable after ignition.

Next, the weighted fitness of individual I is calculated as:

(185185)

the  factors are user–specified constants that determine the relative importance of each fitness metric. If experimental data are obtained at multiple heat flux levels (or heating rates in the case of TGA experiments), then an individual’s final fitness is considered to be the sum of the fitness values calculated at each heat flux level:

(186186)

where  is the number of heat flux levels (or heating rates) at which data were obtained. The steps represented by Equations 184 through 186 are then repeated for all individuals to tabulate a numerical fitness value for each individual.

### 7.2.3 Selection for reproduction

The next generation is obtained through the reproduction process wherein parents’ genes are combined to produce offspring. The likelihood that an individual reproduces is determined by its fitness. In this way, relatively bad candidate solutions die out, while relatively good solutions survive and propagate. This “natural selection” process is the basis of genetic algorithms’ ability to exploit good solutions.

There are many different ways in which individuals can be selected for reproduction, but proportional selection [15] is used here for simplicity. The selection probability of individual I is:

(187187)

Thus, the higher an individual’s fitness, the greater the probability it will be selected for reproduction. In practice, the selection probability in Equation 187 is implemented by first sorting the current population in order of decreasing fitness, i.e. . Then, for each individual, the following is calculated:

(188188)

Note that by definition, q1 = 1 and . Next, a random number r belonging to a uniform distribution is generated on the interval [0, 1]. Selection proceeds by comparing r with q:  if qI+1 < r  qI, then AI is selected for reproduction. The selection process is repeated N times to choose N parents.

If one individual has a relative fitness much higher than the average fitness, it is likely that this individual will be selected several times for reproduction. To prevent premature convergence, a target (maximum) selection number S is used so that any individual may be selected for reproduction no more than S times per generation (1  S < N). If an individual has reproduced S times and is selected again for reproduction, then a new individual is randomly selected from the population for reproduction. Low values of S preserve variability at the expense of convergence.

### 7.2.4 Reproduction

Once individuals have been selected for reproduction, offspring are generated through a linear combination of two parents. Denote {B} as the subset of population {A} that was selected for reproduction. As many as S copies of a single individual may belong to the set {B}. The offspring are stored in a temporary intermediate population denoted {C}. This is accomplished by generating a matrix of random numbers (denoted  where i = 1 . . . N/2 and j = 1 . . . n) belonging to a uniform distribution on the interval [–0.5, 0.5] and then producing offspring as linear combinations of the parents:

(189189)

### 7.2.5 Mutation

After two parents have combined genes to produce a new individual, a process analogous to genetic mutation is used to introduce variability into the population, which ensures the entire search space is explored and that the solution does not become trapped at a local maximum. Mutation is accomplished by introducing random variations into one or more of an individual’s genes. The probability that a gene is mutated is relatively low, perhaps 0.05. Mutation is performed on a gene–by–gene basis. At the start of the calculation, each parameter (gene) is assigned a user–specified mutation probability pmut,j. Mutation is performed on the intermediate population {C} and begins by generating a matrix of random numbers, denoted  (where I = 1 . . . N and j = 1 . . . n) belonging to a uniform distribution on the interval [0,1]. Mutation occurs on individual I gene j if . If a gene is selected for mutation, then one of two types of equiprobable mutation occur. In the first (Equation 190a) the gene is simply replaced with a randomly generated value. In the second, the gene is replaced with an excursion from its current value (Equation 190b):

(190190a)

(190b)

In Equation 190a, r is a random number on the interval [0, 1], and in Equation 190b s is a random number on the interval [–0.5, 0.5]. The user–specified parameter vmut controls the severity of the mutation and is generally less than 1.

### 7.2.6 Replacement

The final step in the genetic algorithm is to replace the parents with the offspring. In the algorithm used here, the offspring (i.e., the individuals in the intermediate population) completely replace the parents:

(191191)

The processes of selection, reproduction, and mutation and replacement are repeated until a predetermined number of generations has passed or the solution converges, meaning no further improvement of the solution occurs with subsequent generations.

## 7.3 Application to a synthetic material with known properties

To assess the methodology’s capabilities, a hypothetical charring material with a thickness of 10 mm is used to generate a set of simulated experimental data under nonflaming conditions at 25 kW/m2 and 75 kW/m2 irradiance. A known set of input parameters is used to generate synthetic experimental data, i.e. real experimental data are not used. This represents a situation where the thermophysical properties of an idealized material are known exactly, and provides an opportunity to see how closely the genetic algorithm can match these known properties.

The overall fitness is evaluated from the surface temperature, back face temperature, mass loss rate, and cumulative mass loss. All eight  values are set to 1, a value of  = 0.1 is used, and the fitness exponent  is set to 2. In this case, the maximum attainable fitness is .

The evolution of the population–averaged fitness and the fitness of the best individual found at any point during the evolution process are shown in Figure 2. The average fitness, an indication of the quality of the “gene pool”, increases rapidly for the first ~40 generations but increases only slightly or plateaus thereafter. By 150 generations, the highest fitness found during in the evolution process reached over 600, or 75% of the maximum attainable fitness (800). For practical situations, it would have been adequate to stop the algorithm after ~150 generations.

Figure 22. Evolution of average and best fitness to ~250 generations for simulated experimental data case.

Table 3 gives a comparison of the optimal values found by the genetic algorithm and the actual values used to generate the simulated experimental data. The algorithm matched 7 of 13 parameters within 10%. The biggest error was for the specific heat capacity temperature exponents. The lower value of the pre–exponential factor found by the genetic algorithm is compensated for by the slightly lower activation energy. This compensation effect between the pre–exponential factor and the activation energy has long been observed to occur in the estimation of pyrolysis kinetics parameters from thermogravimetric analysis. Due to the kinetic compensation effect, the algorithm may not find the “true” values of the pre–exponential factor and the activation energy, but it will find values of these parameters that match the Arrhenius function over the temperature range within which pyrolysis occurs.

Table 33. Comparison of actual material properties used to generate simulated experimental data and material properties found with genetic algorithm.

| Property | Actual | GA | % Error |
| --- | --- | --- | --- |
| Pre–exponential factor (s–1) | 2.00 × 1011 | 1.56 × 1010 | –92.2 |
| Activation energy (kJ/mol) | 142.0 | 130.4 | –8.2 |
| Reaction order (–) | 2.00 | 1.90 | –5.0 |
| Char density (kg/m3) | 100.0 | 102.9 | 2.9 |
| Heat of pyrolysis (J/kg) | 5.00 × 105 | 5.21 × 105 | 4.1 |
| Virgin conductivity @ 300 K (W/m–K) | 0.200 | 0.207 | 3.6 |
| Char conductivity @ 300 K (W–m–K) | 0.060 | 0.095 | 59.0 |
| Virgin conductivity temperature exponent (–) | 0.500 | 0.397 | –20.5 |
| Char conductivity temperature exponent (–) | 0.800 | 0.395 | –50.6 |
| Virgin specific heat @ 300 K (J/kg–K) | 1400 | 1333 | –4.8 |
| Char specific heat @ 300 K (J/kg–K) | 1600 | 1610 | 0.6 |
| Virgin specific heat temperature exponent (–) | 0.200 | 0.379 | 89.6 |
| Char specific heat temperature exponent (–) | 0.100 | 0.477 | 376.8 |

A comparison of the simulated experimental surface temperature, back face temperature, and mass loss rate with the model predictions using the optimal material properties found by the genetic algorithm is given in Figure 3. It is seen that the agreement is excellent, which verifies the methodology’s capabilities to estimate material properties from experimental data.

(a)

(b)

Figure 33. Comparison of simulated experimental data and model calculations using optimized material properties determined by GA.

(a) 25 kW/m2 irradiance; (b) 75 kW/m2 irradiance.

# 8.0 Concluding Remarks

This technical reference presents the mathematical formulation of Gpyro. The document is constantly updated, so if parts of it are not clear, please let us know. Similarly, if there are features or functionality you would like to see added, please let us know, and they may make it into the next release of Gpyro!

# 9.0 References

[11] 	Lautenberger, C., “Gpyro – A Generalized Pyrolysis Model for Combustible Solids – Users’ Guide,” Version 0.800, May 1, 2014.

[22] 	Lautenberger, C.W., “A Generalized Pyrolysis Model for Combustible Solids,” Ph.D. Dissertation, Department of Mechanical Engineering, University of California, Berkeley, Fall 2007.

http://reaxengineering.com/media/docs/lautenberger_dissertation.pdf

[33] 	Lautenberger, C. & Fernandez-Pello, A.C., “Generalized Pyrolysis Model for Combustible Solids,” Fire Safety Journal 44 819-839 (2009)..

[44] 	Lautenberger, C. & Fernandez-Pello, A.C., “A Model for the Oxidative Pyrolysis of Wood,” Combustion and Flame 156 1503-1513 (2009).

[55] 	Dodd, A.B., Lautenberger, C. & Fernandez-Pello, A.C., “Numerical Examination of Two-Dimensional Smolder Structure in Polyurethane Foam,” Proceedings of the Combustion Institute 32: 2497-2504 (2009).

[66] 	Lautenberger, C., Kim, E., Dembsey, N. & Fernandez-Pello, C., “The Role of Decomposition Kinetics in Pyrolysis Modeling – Application to a Fire Retardant Polyester Composite,” Fire Safety Science 9: 1201-1212 (2008).

[77] 	Bird, R.B., Stewart, W.E., and Lightfoot, E.N., Transport Phenomena, John Wiley & Sons, New York, 1960.

[88]	Patankar, S.V., Numerical Heat Transfer and Fluid Flow, Hemisphere Publishing Corporation, New York, 1980.

[99] 	Mills, A.F., Mass Transfer, Prentice Hall, Upper Saddle River, NJ, 2001, pg. 161.

[1010] 	Lautenberger, C., Rein, G., and Fernandez–Pello, C., “The application of a genetic algorithm to estimate material properties for fire modeling from bench–scale fire test data,” Fire Safety Journal 41: 204–214 (2006).

[1111]	Park, T.Y. and Froment, G.F., “A hybrid genetic algorithm for the estimation of parameters in detailed kinetic models,” Computers & Chemical Engineering 22: S103–S110 (1998).

[1212]	Elliott, L., Ingham, D.B., Kyne, A.G., Mera, N.S., Pourkashanian M., and Wilson, C.W., “Genetic algorithms for optimisation of chemical kinetics reaction mechanisms,” Progress in Energy and Combustion Science 30: 297–328 (2004).

[1313]	Goldberg, D., Genetic Algorithms: In Search, Optimization & Machine Learning. Addison–Wesley Publishing Company, Inc., Menlo Park, 1989.

[1414]	Michalewicz, Z., Genetic Algorithms + Data Structures = Evolution Programs. Springer–Verlag, New York, 1992.

[1515]	Dumitrescu D., Lazzerini B., Jain, L.C., and Dumitrescu A., Evolutionary Computation. CRC Press, New York, 2000.