Gpyro - A Generalized Pyrolysis Model for Combustible Solids

Users’ Guide

Version 2.0

May 19, 2025

# Acknowledgments

The financial support of NSF and NASA is gratefully acknowledged. Gpyro development was funded as part of NSF Award 0730556, “Tackling CFD Modeling of Flame Spread on Practical Solid Combustibles” (September 2007-September 2010). Development of early Gpyro versions was funded by the NASA Graduate Student Researcher Program under Grant #NNC-04HA08H, “Piloted Ignition and Flame Spread on Composite Materials in Partial and Normal Gravity” (August 2004 – August 2007).

Esther Kim at WPI and Amanda Dodd at UC Berkeley have played important roles in model development by serving as beta testers and exercising Gpyro in ways that were not contemplated during its initial development. I would like to thank them for their patience, their bug reports, and their suggestions as to how to improve Gpyro. Bryan Klein at Thunderhead Engineering helped get the source code and other files under Subversion control on Google Code (before the repository was moved to a Reax Engineering server). Thomas Steinhaus from the University of Edinburgh showed great patience as we worked together to get Gpyro running on a cluster there.

Others that have contributed to the development of Gpyro over the years include Nicolas Bal, Bangalore Yashwanth, Lucie Hasalova, Davood Zeinali, Xinyan Huang, Salah Benkorichi, and probably many others that I am forgetting!

# Contents

Acknowledgments	i

Contents	ii

List of Tables	v

1.0	Introduction	1

1.1.	What is Gpyro?	1

1.2.	Philosophy behind Gpyro	2

1.3.	Types of problems that can be analyzed with Gpyro	2

1.4.	How to use this guide	3

2.0	Getting started	4

2.1.	Downloading Gpyro	4

2.2.	Compiling Gpyro	4

2.2.1.	Build Procedure (Linux and macOS)	4

2.2.2.	Windows	5

2.2.3.	Requirements	5

2.2.4.	Adding Gpyro to Your PATH	5

2.3.	Running Gpyro (standalone simulation)	5

2.4.	Running Gpyro (property estimation program)	6

3.0	format of Input files	7

4.0	Governing Equations and Model Activation	8

4.1.	Mass and Species Conservation	8

4.2.	Energy Conservation	9

4.3.	Gas Transport	10

4.4.	Gas Species Transport	12

4.5.	Gas-Phase Energy Conservation	12

5.0	Setting up simulations to run	14

5.1.	Defining Simulations and Batch Mode	14

5.2.	0D Calculations (ATG)	14

6.0	Building Meshs	15

6.1.	Mesh Definition and Dimensionality	15

6.2.	Cell Size, Stability Constraints, and Solver Direction	15

6.3.	Subdividing the Mesh into Multiple Regions and Creating Complex Geometries	16

6.4.	Defining Initial Conditions	16

6.5.	Defining Boundary Conditions	17

6.6.	Examples of Geometry Definitions	17

7.0	Initial Conditions	20

7.1.	Condensed-phase temperature	20

7.2.	Gas-phase temperature	20

7.3.	Pressure	20

7.4.	Condensed species mass fractions	20

7.5.	Gas species mass fractions	20

7.6.	Example	20

8.0	Boundary conditions	22

8.1.	Time-dependent boundary condition definition	22

8.2.	Thermal boundary conditions (condensed phase)	22

8.2.1.	Fixed temperature	22

8.2.2.	Imposed heat flux	22

8.3.	Thermal boundary conditions (gas phase)	23

8.4.	Gas transport boundary conditions	24

8.4.1.	Instantaneous release boundary condition	24

8.4.2.	Pressure-driven gas flow boundary condition	24

8.5.	Gas species transport boundary conditions	24

8.6.	Example	25

9.0	Condensed-phase thermophysical properties	27

9.1.	Material Definition	27

9.2.	Thermal properties	27

9.2.1.	Thermal conductivity	27

9.2.2.	Specific Heat Capacity	28

9.2.3.	Bulk density	28

9.2.4.	Radiative Properties	28

9.2.5.	Example	28

9.3.	Transport Properties in Porous Media	29

9.3.1.	Solid density	29

9.3.2.	Permeability	30

9.3.3.	Pore diameter	30

10.0	Gas-phase thermophysical properties	31

10.1.	Gaseous species definition	31

10.2.	Special oxidative species	31

10.3.	Gas heat capacity	31

10.4.	Gas molecular weight	31

10.5.	Diffusive gas properties	31

10.6.	Gas viscosity	32

10.7.	Example	32

11.0	Condensed-phase reactions	33

11.1.	Declaring Condensed-Phase Reactions	33

11.1.1.	Declaring a reaction	33

11.1.2.	Special case: non-charring reactions	34

11.1.3.	Special case: multiple condensed products	34

11.2.	Condensed-Phase Stoichiometry and  Deformation	34

11.2.1.	Definition of the Stoichiometric Coefficient	34

11.2.2.	Deformation and Volume Change	34

11.3.	Gaseous stoichiometry: the species‑yield matrix GYIELDS	35

11.4.	Kinetics of Condensed-Phase Reactions	35

11.4.1.	General formulation	35

11.4.2.	Temperature Dependence (Arrhenius Law)	36

11.4.3.	Kinetic models	36

11.5.	Enthalpy of reaction	38

11.5.1.	Constant Heat of Reaction	39

11.5.2.	Conservative Formulation of Heat of Reaction	39

11.6.	Example	40

12.0	Homogeneous gas-phase reactions	43

12.1.	Declaring Gas-Phase Reactions	43

12.2.	Chemical Kinetics	43

12.3.	Heat of Reaction	44

12.4.	Example	44

13.0	Output files	45

13.1.	Available quantitites to dump	45

13.2.	Point dumps	46

13.2.1.	Configuration	46

13.2.2.	Example	47

13.2.3.	Output File Format	48

13.3.	Profile dumps	48

13.3.1.	Configuration	48

13.3.2.	Example	49

13.3.3.	Output File Format	50

13.4.	Smokeview visualization	50

13.4.1.	Configuration	51

13.4.2.	Example	51

13.4.3.	Output File Format	51

13.5.	Changing the dump interval mid-simulation	51

13.6.	Other Output Files	52

13.6.1.	.out File	52

13.6.2.	CPU Timing File	52

13.6.3.	Iteration Convergence File	52

14.0	Material property estimation	53

14.1.	Format of input files for material property estimation program	54

14.2.	Miscellaneous parameters – GA_GenInput worksheet	55

14.3.	Fitness metric weightings – GA_phi worksheet	56

14.4.	Variables to optimize – GA_Vars worksheet	57

14.5.	Experimental data – 01, 02, 03, etc. worksheets	58

References	60

Appendix A. Nomenclature	61

Appendix B. Namelist group names and available keywords.	63

Appendix C. Gas properties	68

# List of Tables

Table 1: Description of Gpyro Namelist groups	7

Table 2: Orientation index of the boundary faces.	17

Table 3: Available kinetic models in Gpyro.	37

Table 4: Quantities available for dump.	45

Table 5: Gpyro property estimation Fortran Namelist groups, associated worksheet in spreadsheet-based front end, and description of Namelist group entries.	54

Table 6: &GPYRO_GENERAL keywords (general parameters)	63

Table 7: &GPYRO_CASES keywords (cases to run)	63

Table 8: &GPYRO_GEOM keywords (geometry specification)	63

Table 9: &GPYRO_IC keywords (initial condition specification).	64

Table 10: &GPYRO_ALLBC keywords (boundary condition specification).	64

Table 11: &GPYRO_SPROPS keywords (solid properties).	64

Table 12: &GPYRO_GPROPS keywords (gas properties).	65

Table 13: &GPYRO_RXNS keywords (condensed phase reaction properties).	65

Table 14: &GPYRO_GYIELDS keywords (the species‑yield matrix GYIELDS).	66

Table 15: &GPYRO_HGYIELDS keywords (the species‑yield matrix HGYIELDS).	66

Table 16: &GPYRO_HGRXNS keywords (gas phase reaction properties).	66

Table 17: &GPYRO_OUTPUT keywords (output quantities).	66

Table 18: &GA_GENINPUT keywords (property estimation general/miscellaneous).	67

# Introduction

Gpyro is a generalized pyrolysis model for combustible solids (https://gitlab.imft.fr/gpyro/gpyro). This document presents the User Guide for Gpyro V2, a substantially redesigned and extended evolution of the original Gpyro pyrolysis code developed by C. Lautenberger (Gpyro V0.8). Although Gpyro V2 retains the conceptual foundations of the initial model, it represents a major rework of the entire codebase. The structure has been reorganized and rewritten in a clearer, more modular, and more maintainable form, resulting in a significantly more robust and stable implementation. Several new numerical solvers have been introduced, the modelling capabilities have been considerably extended, particularly for fully three‑dimensional configurations, and the computational performance has been greatly improved. Depending on the test case, Gpyro V2 is between 4× and 200× faster than the original version.

The development history of the initial Gpyro code can be followed through Lautenberger’s User Guide, Technical Reference, and the PhD dissertation on which the original implementation was based [ref]. These documents establish the theoretical and numerical basis from which Gpyro V2 originates. The present version continues this lineage while introducing a complete redesign of the numerical solvers, extended verification procedures, and updated documentation.

The recent developments of Gpyro V2 are documented in Y. Abdelhafiz’s ongoing PhD thesis (2026), on which much of the current redesign is based, as well as in the new Technical Guide accompanying this release and in the following publications [ref, ref]. This User Guide is specifically adapted to Gpyro V2, and while it primarily reflects the structure and capabilities of this new version, many elements remain applicable to the original Gpyro implementation. It may also serve as a complementary reference for users of the original code, as this document completes and clarifies Lautenberger’s original User Guide, particularly in sections where the earlier documentation was incomplete or only partially drafted.

## What is Gpyro?

Gpyro is open-source, freely available computer software that simulates thermo-chemical processes occurring in thermally stimulated solids (loosely, it is a pyrolysis model). It can be used for 0D (lumped), 1D, 2D, or 3D simulations of thermally stimulated solids, including thermal and thermo-oxidative decomposition of condensed-phase species. Gpyro is coupled to Version 6.2.0 of NIST’s Fire Dynamics Simulator (FDS) [10] where it can be applied as a boundary condition to facilitate coupled modeling of ignition and flame spread. That is, calculations can be conducted either as standalone simulations or in a coupled simulation where Gpyro is applied as a boundary condition in FDS.

An integral part of Gpyro is a material property estimation program that can be used to estimate material properties from laboratory tests such as ThermoGravimetric Analysis (TGA), Differential Scanning Calorimetry (DSC), and conventional flammability tests such as the Cone Calorimeter and the Fire Propagation Apparatus (FPA). The basic property estimation routine was originally limited to genetic algorithm optimization [7] and was later extended to additional methods [8]. The property estimation routines use Message Passing Interface (MPI) to reduce run times with parallel processing. The property estimation routines are “embarrassingly parallel”, and Gpyro’s property estimation routines have been successfully run on 128+ computational cores with near-linear speedup.

## Philosophy behind Gpyro

Rather than hard-coding a certain level of complexity, Gpyro provides the user with the ability to include as much or as little complexity in a calculation as desired. In its simplest form, Gpyro is a 1D transient heat conduction code for a constant property inert (nonreactive) solid. In its most complicated form, Gpyro can be used to solve 3D conjugate heat transfer, fluid flow/pressure evolution, gaseous species diffusion/convection in a multi-component, heterogeneous, reacting porous medium with temperature-dependent thermophysical properties, in-depth radiation absorption, radiation heat transfer across pores, multi-step heterogeneous decomposition kinetics of arbitrary order or complex reaction model, and multi-step homogeneous gas-phase reactions. The level of complexity included in a simulation, and the requisite number of model input parameters/material properties, is decided entirely by the user.

Gpyro development efforts have focused on providing generalized software containing physical phenomena that can be enabled or disabled as desired by the user. Similarly, the material property estimation routines require the user to specify valid parameter ranges as well as other variables/flags that affect how the material property estimation process occurs. Gpyro, and its associated material property estimation routines, are merely tools that can be used to determine which physics matter under which circumstances. Equally as important, Gpyro can be used to develop “validated” sets of material properties that can be used to model the pyrolysis behavior of solid materials under fire conditions.

## Types of problems that can be analyzed with Gpyro

Much effort has been devoted to keeping Gpyro as general as possible to expand the types of problems that can be analyzed. The following is a brief partial list of the types of problems that Gpyro has been used to simulate at some point during its development:

- TGA experiments (primarily for estimating reaction kinetics)

- DSC experiments  (primarily for estimating heats of reaction)

- Cone Calorimeter and FPA experiments (primarily for estimating reaction kinetics, heats of reaction, and thermophysical properties)

- Thermal pyrolysis and surface regression of noncharring thermoplastics

- Thermal pyrolysis of charring solids

- Thermal pyrolysis of heterogeneous fiber reinforced polymer composites

- Heating, gasification, swelling of intumescent coatings, and the thermal resistance offered by such coatings

- Effect of ambient oxygen concentration on thermo-oxidative pyrolysis of charring solids

- Smolder wave propagation in porous media

- Transition from smolder to flame in porous media

- Propellant cookoff in a confined housing

- Self-heating and spontaneous ignition of rags soaked in linseed oil

- Ignition of fuel beds by embers, firebrands, and sparks

- Laminar opposed flow flame spread (coupled to FDS)

Other problems that could be simulated with Gpyro either “out of the box” or with minor modifications include:

- Concrete spalling

- Ablation of sacrificial protective coatings

- Behavior of thermally stimulated anisotropic solids (wood, fiber reinforced polymer composites)

- Biomass pyrolysis in fast/flash pyrolysis reactors

## How to use this guide

The best way to use this guide is not necessarily to read it cover to cover. It is recommended that new users run the sample cases included with the Gpyro distribution and then read the sections of this guide as necessary to understand these samples.

# Getting started

Gpyro is a console application that is executed from a Windows command prompt or Linux console. This chapter describes how to download, compile, and run Gpyro under Windows, Linux and MacOS.

## Downloading Gpyro

Gpyro exists in two versions. The original version, developed and published in the early 2010s, is available on GitHub at https://github.com/lautenberger/gpyro . Although this version is no longer actively maintained, it remains a valuable reference for understanding the initial structure and functionality of the code.

In parallel, a maintained fork of Gpyro has been developed to improve performance, extend capabilities, and ensure compatibility with modern systems. This documentation is based on that maintained version available in GitLab at https://gitlab.imft.fr/gpyro/gpyro.While the input file structure remains largely consistent between the two versions; some newly introduced options may affect compatibility with the legacy version.

The maintained repository includes the source code for Gpyro Gpyro_propest, and coupled Gpyro/FDS, validation cases, compilation instructions, and several documentation resources: a user guide, a technical guide, and a verification guide.

## Compiling Gpyro

The build directory in the Gpyro repository contains all necessary Makefiles to compile the executables gpyro and gpyro_propest (the Gpyro property estimation program) and fds6_gpyro (The coupling between Gpyro and FDS 6.2.0). Compilation is supported on Linux, macOS, and Windows (via provided Makefiles), using either GNU or Intel Fortran compilers. Both release and debug builds are available.

### Build Procedure (Linux and macOS)

Two build scripts are provided in build/linux:

- make_gnu.sh: Compiles both release and debug versions of gpyro, fds6_gpyro and gpyro_propest using the GNU Fortran compiler.

- make_intel.sh: Compiles the same versions using the Intel Fortran compiler.

These scripts automatically invoke the appropriate Makefiles and compile all variants in a single run.

Example (GNU Fortran):

cd build/linux

bash make_gcc.sh   # Compile with GCC

Note that if the compilation fails, it may be due to incorrect compiler paths or environment variables in the script. You can edit the script to match your system’s compiler installation.

Compiled executables will be located in:

build/linux/gpyro/

build/linux/gpyro_propest/

### Windows

Make instructions are available for Windows, but they have not been tested by the developer.It is recommended to use Windows Subsystem for Linux (WSL) and follow the Linux build procedure.

### Requirements

Fortran Compiler: GCC ≥ 8.1 or Intel Fortran (we use gcc  9.1).

MPI: A working MPI installation is required to compile gpyro_propest (we use OpenMPI 3.1.3).

If you plan to conduct 2D or 3D simulations, then you should also install NIST’s Smokeview because Smokeview is used to visualize 2D Gpyro simulations. Smokeview can be downloaded from https://pages.nist.gov/fds-smv/downloads.html.

### Adding Gpyro to Your PATH

To run gpyro or gpyro_propest from any location, add their build directory to your system’s PATH environment variable. Example :

export PATH=/path/to/gpyro/build/linux:$PATH

To make this change permanent, add the line to your shell configuration file (e.g., ~/.bashrc).

## Running Gpyro (standalone simulation)

Gpyro is executed from the Windows command, or the Linux shell as follows:

gpyro input_file.data

where input_file.data is a text file that specifies the geometry, boundary conditions, thermophysical properties, reaction kinetics, etc. See Section 3.0 for an explanation of input files. The format of this input file is described in Section 3.0

To run Gpyro in parallel using OpenMP, set the environment variable OMP_NUM_THREADS to the desired number of threads before executing the program. For example, on Linux or macOS:

OMP_NUM_THREADS=4

gpyro input_file.data

and on Windows PowerShell:

$env:OMP_NUM_THREADS=4

.\gpyro.exe input_file.data

It is generally recommended to use no more than one processor per 500 computational cells, as using more threads may not improve performance and can even degrade it due to communication overhead.

## Running Gpyro (property estimation program)

The Gpyro property estimation algorithm is a separate executable file from the standalone Gpyro executable. It is run in a way that is similar to the standalone executable, but no input file name is specified:

gpyro_propest

The reason that no input file is specified is that when running the Gpyro property estimation program, the input file name is assumed to be gpyro.data. The filename is hardcoded to simplify running in parallel with Message Passing Interface (MPI). See Section  3.0 for an explanation of input files. As described in Section  3.0, when running the property estimation algorithm, it is also necessary to provide (in the working directory) comma separated value files that contain experimental data from which model input parameters will be estimated (unless synthetic experimental data are used).

The Gpyro property estimation program can be run in serial or in parallel, but MPI must be installed even for a serial run or the executable will not run. The above command would launch a serial run of the Gpyro property estimation program. The procedure for starting a parallel run depends on your operating system and the installed distribution and version of MPI. The following command would launch a parallel run on 16 processors under linux:

mpirun –np 16 –machinefile ./machines.linux /usr/local/bin/gpyro_propest

The syntax may be slightly different for different MPI distributions or under Windows.

# format of Input files

Gpyro reads plain ASCII text files that consist of a sequence of Fortran namelist groups (e.g., &GPYRO_GENERAL) and entries associated with each Namelist group (e.g., P0 = 101325.). The available Namelist groups and entries for each Namelist group are given in Appendix A.

The Namelist groups specific to standalone simulations are listed in Table 1. Those Namelist groups specific to the property estimation algorithm are discussed in Section 14.1.

Table 1: Description of Gpyro Namelist groups

| Namelist group name | Description | Reference |
| --- | --- | --- |
| &GPYRO_GENERAL | General parameters. | Chapter 4.0 and Table 6 |
| &GPYRO_OUTPUT | output parameters. | Chapter 13.0 Table 3 and Table 17 |
| &GPYRO_SPROPS | condensed phase properties. | Chapter 9.0 and Table 11 |
| &GPYRO_GPROPS | Gas phase properties. | Chapter 10.0 and Table 12 |
| &GPYRO_RXNS | Condensed phase reactions. | Chapter 11.0 and Table 13 |
| &GPYRO_GYIELDS | Condensed phase reactions gaseous yields matrix. | Chapter 11.0 and Table 14 |
| &GPYRO_HGRXNS | gaseous phase reactions. | Chapter 12.0 and Table 16 |
| &GPYRO_HGYIELDS | Gas phase reactions gaseous yields matrix. | Chapter 12.0 and Table 15 |
| &GPYRO_IC | Initial conditions. | Chapter 7.0 and Table 9 |
| &GPYRO_GEOM | Geometry. | Chapter 6.0 and Table 8 |
| &GPYRO_ALLBC | Boundary conditions. | Chapter 8.0 and Table 10 |
| &GPYRO_CASES | Cases to run. | Chapter 5.0 and Table 7 |

Standard Fortran Namelist group conventions apply. The first character of a Namelist group must be an ‘&’, and the last character must be a ‘/’. An input file consists of a series of Namelist groups. The order in which the Namelist groups appear in the input file does not matter, but a Namelist group may appear only once in an input file. For example, the following is valid:

&GPYRO_GENERAL TAMB           = 293.

P0             = 101325. /

But the following is not valid:

&GPYRO_GENERAL TAMB           = 293. /

&GPYRO_GENERAL P0             = 101325. /

# Governing Equations and Model Activation

Gpyro solves a coupled set of conservation equations describing heat transfer, condensed-phase decomposition, gas generation, and transport phenomena within thermally degrading porous materials. Depending on the selected modeling options, the solver may additionally account for internal gas pressure evolution, and gas-species transport.

The present chapter describes the governing equations implemented in Gpyro together with the associated modeling assumptions and activation parameters. For each physical model, the corresponding input options required to enable or configure the formulation are identified. Unless otherwise specified, model activation parameters are defined in the &GPYRO_GENERAL namelist group.

## Mass and Species Conservation

Gpyro adopts a Lagrangian formulation in which the control volume ΔV of each computational cell evolves in time to follow the amount of condensed-phase material present in the cell. As decomposition proceeds and mass is lost due to gas generation, the volume adjusts accordingly, ensuring consistency between mass and geometry.

The total mass conservation over a control volume ΔV is written as:

| dmdt=d(ρ ΔV)dt=-ωfg'''ΔV | (1) |
| --- | --- |

Where ρ​ denotes the bulk density and ωfg''' is the volumetric gas generation rate resulting from chemical reactions.

For each condensed-phase species A, a conservation equation is formulated as:

| dmAdt=d (YAρ ΔV) dt=ωnet,A'''ΔV | (2) |
| --- | --- |

where ​YA is the mass fraction of species A, and ωnet,A''' is the net rate of production of that species.

The net production rate is defined as the difference between formation and destruction rates:

| ωnet,A'''=ωf,A'''-ωd,A''' | (3) |
| --- | --- |

The formation and destruction terms are obtained by summing the contributions of all K condensed phase reactions (See section 11.0):

| ωd,A'''=k=1KωdA,k''' ; ωf,A'''= k=1KωfA,k''' | (4) |
| --- | --- |

For a given reaction k, the destruction rate ωdA,k''' is equal to zero if the reaction does not consume species A, and equal to the reaction rate rk​ if it does. Similarly, the formation rate ωfA,k''' is zero if species A is not produced by reactionk, and equal to θkrk​ if it is, where θk​ denotes the stoichiometric coefficient associated with reaction k.

The total gas formation rate is obtained by summing the contributions of all reactions:

| ωfg'''=k=1Kωfg,k''' ; ωfg,k'''=1-θkrk | (5) |
| --- | --- |

Additional details regarding reaction mechanisms and associated parameters are provided in Section 11.0.

## Energy Conservation

The conservation of energy is expressed over a control volume (ΔV) as:

| d(hm)dt=∂V k.∇T. n dS +Qs'''ΔV | (6) |
| --- | --- |

where h denotes the specific enthalpy, m the condensed-phase mass, and T the solid temperature, k=kx,ky,kz the anisotropic thermal conductivity, and n the outward normal vector of the volume.

The volumetric source term Qs''' represents the combined effects of radiation absorption, gas–solid heat exchange, chemical reactions, and additional heat transfer mechanisms. It is written as:

| Qs'''=-∂qr''∂z+Qs,g'''+A=1MhAωnet,A'''+k=1KQk'''+Qcl''' | (7) |
| --- | --- |

Chemical source terms

The term A=1MhAωnet,A''' involving the summation over the M codesned species accounts for the variation of sensible enthalpy associated with chemical reactions. In this expression, hA denotes the sensible enthalpy of species A, while ωnet,A''' represents the net rate of production or consumption of that species.

The contribution k=1KQk'''corresponds to the heat released or absorbed by chemical reactions. The formulation of this term and the associated input parameters are described in Section 11.5.

Radiation absorption

Radiative heat transfer within the material is modeled using a Beer–Lambert-type attenuation law. The radiative heat flux is expressed as:

| qr''z=ϵQextexp-0zκz'dz' | (8) |
| --- | --- |

where ϵ denotes the surface emissivity, Qext is the externally imposed radiative heat flux, and κ is an effective absorption coefficient.

Gas solid thermal exchange

The gas–solid heat exchange term Qs,g''' is controlled through the parameter QSG_MODEL in the GPYRO_GENERAL namelist group. When the option 'NONE' is selected, no thermal interaction is considered, and Qs,g'''=0.

When the option 'THERMAL_EQUILIBRIUM' is used, the gas and solid phases are assumed to be locally at the same temperature, and the exchange term is expressed as:

| Qs,g'''=mgas''cpgdTdz | (9) |
| --- | --- |

where mgas'' is the gas mass flux and cpg the gas heat capacity.

QSG_MODEL='THERMAL_EQUILIBRIUM_GENERAL' corresponds to a more general formulation, in which the full gas-phase energy balance is retained (See user guide):

| Qs,g'''=ρgψcpg∂T∂t+m″cpg∂T∂z-∂∂zψkg∂T∂z | (10) |
| --- | --- |

This formulation is included for consistency with earlier versions of the model. However, it is not recommended for general use, as it does not significantly improve predictive capability and has not been fully validated.

An alternative formulation based on a two-temperature approach, in which the gas-phase temperature was explicitly solved, was introduced in Gpyro version 0.8. In that formulation, the gas–solid heat exchange term was expressed as Qs,g'''=hcv(T-Tg). where Tg denotes the gas temperature. This approach is no longer available in the current version of Gpyro, as the solver does not explicitly resolve the gas-phase temperature, despite its description in earlier user guides.

volumetric heat loss

In addition, an optional heat loss term may be included to represent convective heat transfer at boundaries that cannot be explicitly resolved in 1D configurations. This term is written as:

| Qcl'''=-hlc(T-Tamb) | (11) |
| --- | --- |

Where hlc is an effective heat transfer coefficient in W.m-2.K-1 and Tamb is the ambient temperature. The parameter hlc is specified through VHLC  while Tamb is defined via TAMB, both in the GPYRO_GENERAL namelist.

## Gas Transport

Gas transport in Gpyro may be modeled using different levels of approximation, depending on the desired physical fidelity and computational cost. The selected formulation determines how gaseous products generated by pyrolysis are transported through the porous condensed phase and released at the boundaries.

The simplest approach assumes instantaneous release of gaseous products through the exposed surface. In this formulation, no internal gas accumulation or pressure gradient is resolved within the material. The gas mass flux is directly obtained by integrating the gas generation rate over the remaining material thickness:

| mgas''(z)=zLωfg'''(z')ΔV | (12) |
| --- | --- |

where mgas'' is the gas mass flux, ωfg'''  is the volumetric gas generation rate resulting from chemical reactions,  z is the local depth coordinate, and L is the thickness of the condensed phase. This approximation is computationally efficient and is generally appropriate when internal pressure gradients remain negligible.

A more detailed formulation may be employed when internal gas transport through the porous medium is significant. In that case, gas accumulation and pressure-driven flow are explicitly resolved. The gas-phase mass conservation equation is written as:

| dmgdt=dψρgΔVdt=∂V mgas''. n dS+ωfg'''ΔV | (13) |
| --- | --- |

where mg=ψρg is the gas mass contained within the control volume, ψ is the porosity, ρg is the gas density.

The gas mass flux is modeled using Darcy’s law:

| mgas''=Kν∂P∂z-gρg | (14) |
| --- | --- |

where K is the permeability, ν is the gas kinematic viscosity, P is the gas pressure, and g is the gravitational acceleration vector.

The gas density is obtained from the ideal gas law:

| ρg=PMRTg | (15) |
| --- | --- |

Where M is the gas molecular weight, R is the universal gas constant, and Tg is the gas temperature.

Combining the continuity equation, Darcy’s law, and the ideal gas relation yields the governing pressure equation:

| dψPMRTgΔVdt=∂V Kν∇P-PgMRTg . n dS+ωfg'''ΔV | (16) |
| --- | --- |

When this formulation is activated, the gas pressure field is explicitly solved, and the gas mass flux is subsequently computed from Darcy’s law.

The pressure solver is activated by setting the parameter SOLVE_PRESSURE = .TRUE. . Gravitational acceleration components are specified through the parameters GX, GY, and GZ. When SOLVE_PRESSURE = .FALSE., the instantaneous gas release approximation is used instead, which is the default behavior.

## Gas Species Transport

In addition to the bulk gas transport formulation described previously, Gpyro may also solve a species transport equation for each gas-phase species. This formulation allows the spatial distribution of gaseous species within the porous medium to be resolved and is particularly important for oxidation problems, where oxygen penetration into the material must be explicitly modeled. The transport equation for gas-phase species j is written as:

| dYjψρgΔVdt+∂V mgas''Yj. n dS=∂V ψρgD ∇Yj. n dS+ωfg,j'''ΔV | (17) |
| --- | --- |

where Yj is the gas-phase mass fraction of species j, D is the effective diffusion coefficient, and ωfg,j''' is the volumetric source term associated with the production or consumption of species j.

The diffusion coefficient is calculated using the Chapman–Enskog formulation described in Section 10.5.

The first term on the left-hand side represents transient gas species accumulation within the porous medium, while the second term accounts for advective transport driven by the bulk gas flow. The right-hand side includes diffusive transport and chemical source terms.

This equation is solved only when the parameter SOLVE_YJ = .TRUE. is specified. By default, gas species transport equations are not solved.

## Gas-Phase Energy Conservation

Earlier versions of the Gpyro documentation presented a formulation in which the conservation of energy in the gas phase could be explicitly solved. In particular, the Gpyro version 0.8 user guide described a gas-phase energy equation and the associated modeling assumptions. However, this capability does not appear to be available in the publicly released source code of Gpyro version 0.8, nor in the current public version of the solver. It is possible that this formulation existed in an internal or unpublished development version of the code.

For completeness, the governing equation originally proposed for gas-phase energy conservation is recalled here:

| dhgψρgΔVdt+∂V mgas''hg. n dS=∂V ψρgD ∇hg. n dSp+Qg'''ΔV | (18) |
| --- | --- |

where hg denotes the gas-phase specific enthalpy and Qg''' is the volumetric gas-phase energy source term. The gas-phase energy source term is expressed as:

| Qg'''=-Qs,g'''+k=1KQg,k'''+l=1LQg,l''' | (19) |
| --- | --- |

The first term represents gas–solid thermal exchange, while Qg,l'''  corresponds to the heat release associated with homogeneous gas-phase reactions l. The term Qg,k''' represents the fraction of heat generated by condensed-phase reactions k that is transferred directly to the gas phase.

The parameter SOLVE_GAS_ENERGY=.TRUE. was originally intended to activate this formulation. However, no operational implementation of the gas-phase energy solver is currently available in the public releases of Gpyro, and the parameter therefore has no effect in the present version of the code.

# Setting up simulations to run

Gpyro supports simulations in 0D, 1D, 2D, and 3D. The procedures for constructing the mesh geometry for each of these cases are presented in Section 6.0. Once the mesh definitions are available, the simulations to be executed are specified in the namelist group &GPYRO_CASES. See Table 7.

A single input file can be used to run several simulations in succession. This is particularly useful for parametric studies in which certain conditions are varied from one case to another. Parameters such as material thickness, applied heat flux, ambient gas composition, front-face heat transfer coefficient, ignition time, flame heat flux, thermophysical properties, and reaction kinetics can be modified between cases. Each simulation is linked to a predefined mesh, ensuring that geometry and boundary conditions are correctly applied.

## Defining Simulations and Batch Mode

The total number of simulations to be executed is given by NCASES. For each case ICASE (from 1 to NCASES), the mesh to be used is specified through IMESH(ICASE), and the simulation end time in seconds is set with TSTOP(ICASE).

The time step used for all simulations is defined in the &GPYRO_GENERAL namelist group through the parameter DT0.

The example below defines a single simulation lasting 5000 s using mesh 1, which must already have its geometry, initial conditions, and boundary conditions defined elsewhere in the input file:

&GPYRO_CASES NCASES = 1

IMESH(1) = 1 , TSTOP(1) = 5000/

## 0D Calculations (ATG)

A 0D simulation corresponds to a single homogeneous particle with negligible gradients of temperature (Bi≪1) and species. This is an idealized model of thermal analysis experiments such as TGA and DSC. Quantities such as particle temperature, mass, and species mass fractions vary temporally but not spatially. In 0D simulations, the particle temperature is specified as a function of time (initial temperature plus a constant linear ramp rate).

For 0D calculations, set ZEROD(ICASE) to .TRUE. in the &GPYRO_CASES group for the desired case. This mode is typically used for TGA or DSC experiments, where the sample is subjected to a constant heating rate. The rate, expressed in °C/min, is set using BETA(ICASE).

The example below specifies two 0D cases: Case 1 runs for 3600 s with a heating rate of 10 °C/min, and Case 2 runs for 1800 s with a heating rate of 20 °C/min.

&GPYRO_CASES NCASES   = 2

IMESH(1) = 1

ZEROD(1) = .TRUE.

TSTOP(1) = 3600. , BETA(1)  = 10.

IMESH(2) = 1

ZEROD(2) = .TRUE.

TSTOP(2) = 1800. , BETA(2)  = 20./

# Building Meshs

The &GPYRO_GEOM group contains all parameters required to define the geometry (See Table 8). Multiple distinct meshes can be defined, each with its own geometry, initial conditions, and boundary conditions. Each mesh runs independently. Defining multiple meshes is only useful when running in batch mode (See Section 5.1). The total number of meshes is specified by the NMESH variable.

## Mesh Definition and Dimensionality

For a mesh with index IMESH (from 1 to NMESH), the geometry is defined as an orthogonal, regular, 3D rectangular mesh. Its dimensions in the x, y, and z directions (in meters) are specified using the parameters XDIM(IMESH), YDIM(IMESH), and ZDIM(IMESH), respectively. The number of cells in each direction is set through NCELLX(IMESH), NCELLY(IMESH), and NCELLZ(IMESH). The dimensionality of the simulation is configured as follows:

- 0D  (ATG) : NCELLZ(IMESH)= NCELLX(IMESH)=NCELLY(IMESH)= 1, (and see Section 145.2).

- 1D: NCELLZ(IMESH) > 1 and  NCELLX(IMESH) = NCELLY(IMESH) = 1 (refinement only in the z-direction)

- 2D: NCELLZ(IMESH) > 1, NCELLX(IMESH) > 1 and NCELLY(IMESH) = 1 (refinement in x and z-directions)

- 3D: NCELLZ(IMESH) > 1,  NCELLX(IMESH) > 1,  NCELLY(IMESH) > 1

## Cell Size, Stability Constraints, and Solver Direction

To accurately capture pyrolysis phenomena, developer experience indicates that the cell size in the degradation direction should be smaller than 0.1 mm. In 2D and 3D simulations, particular care must be taken when choosing the number of cells for two main reasons. First, increasing the number of cells directly increases computational cost, which can rise very quickly in 3D. Second, very small cells in directions solved explicitly by the thermal solver can require prohibitively small time steps to satisfy stability constraints. For an explicitly solved direction (e.g., x), the time step must satisfy the Von Neumann stability criterion :

| Δt<Δx22α=k2ρcpΔx2 | (20) |
| --- | --- |

where α is the thermal diffusivity. This constraint becomes critical when fine cells are used in all directions. For example, with Δx=0.01 mm and a typical thermal diffusivity of α=10-5m2.s-1, the time step must be smaller than 5× 10-6s.

To avoid this limitation, the SOLVER_MAIN_DIRECTIONS option in the &GPYRO_GENERAL namelist allows specifying one direction to be solved implicitly. The stability constraint is then removed for the chosen implicit direction. Since pyrolysis is often dominant along a single axis (typically z) it is recommended to use fine cells in z while keeping coarser cells (e.g., ≥ 1 cm) in x and y. For example, setting SOLVER_MAIN_DIRECTIONS = 'z' with Δx=Δy=1 cm and keepind Δz=0.01 mm yields a stability limit of Δt<5 s, which is more than sufficient for most simulations.

The parameter SOLVER_MAIN_DIRECTIONS can take the values 'x', 'y', 'z'.

## Subdividing the Mesh into Multiple Regions and Creating Complex Geometries

A mesh can be subdivided into multiple regions, each with its own initial conditions and boundary conditions (see Section 8.0). This is done using objects called obstructions. Each obstruction represents a rectangular parallelepiped region within the mesh. The total number of obstructions is specified using the parameter NOBST in the &GPYRO_GENERAL namelist.

For each obstruction IOBST (from 1 to NOBST), its volume is defined by the coordinates X1(IOBST), X2(IOBST), Y1(IOBST), Y2(IOBST), Z1(IOBST), and Z2(IOBST). These values delimit the obstruction’s occupied space within the mesh. In 1D simulations, only Z1 and Z2 need to be specified. In 2D simulations, X1, X2, Z1, and Z2 must be specified.

When multiple meshes are defined, the optional parameter IMESH(IOBST) can be used to specify the index of the mesh to which the obstruction belongs.

Obstructions are essential for modeling multilayer materials, as each obstruction can represent a distinct layer with its own material properties. They are also useful in modeling experiments such as cone calorimeter tests, where insulating substrates or support blocks (e.g., underlying bricks) need to be represented as separate volumes.

If two obstructions overlap, the last one defined in the input file takes precedence and will occupy the overlapping volume.

If at least one obstruction is defined, the computational domain will be restricted to the regions occupied by the obstructions. Any mesh cells outside all defined obstructions will be ignored by the solver. This feature enables modeling of complex geometries within a single mesh. For example, an I-beam can be represented by one mesh containing three obstructions, each corresponding to a distinct block of the beam.

## Defining Initial Conditions

The initial condition for each mesh IMESH is specified through the parameter DEFAULT_IC(IMESH). This value must be an integer corresponding to the index of an initial condition type defined in the &GPYRO_IC namelist. An initial condition type determines, for example, the initial composition of both solid and gas phases, the initial temperature, and the initial pressure, as described in Section 7.0.

When the geometry contains multiple obstructions, each obstruction IOBST can be assigned its own initial condition by setting the parameter ICNUM(IOBST). This parameter works in the same way as DEFAULT_IC but applies specifically to a given obstruction, allowing different parts of the domain to start with different states.

## Defining Boundary Conditions

Boundary conditions for a mesh are set with DEFAULT_SURF_IDX(IMESH, 1:6), where the second index corresponds to one of the six faces of the rectangular domain. Each face index refers to a boundary condition defined in the &GPYRO_ALLBC namelist, such as temperature, pressure, gas composition (see Section 8.0). The correspondence between face indices and their spatial orientation is shown below:

Table 2: Orientation index of the boundary faces.

| Index | Orientation | Direction |
| --- | --- | --- |
| 1 | West | –x |
| 2 | East | +x |
| 3 | South | –y |
| 4 | North | +y |
| 5 | Top | +z |
| 6 | Bottom | –z |

For one-dimensional simulations, only faces 5 and 6 need to be specified; in two dimensions, faces 1, 2, 5, and 6 are required; in three dimensions, all six faces must be assigned a boundary type.

Obstructions can also have individual boundary conditions assigned through the parameter SURF_IDX2D(IOBST,1:6). This follows the same logic as for meshes. If two obstructions are in contact with a given face, no boundary condition needs to be set for that face, as it does not represent an external boundary.

## Examples of Geometry Definitions

This section provides illustrative examples of mesh definitions using the &GPYRO_GEOM namelist in Gpyro. Each example demonstrates typical configurations for different simulation scenarios.

Example 1 : Simple 1D Block

This example defines a single 1D block of 5 cm thickness, refined in the z-direction with 50 cells. No obstructions are used; the computational domain corresponds to the entire mesh.

&GPYRO_GEOM

NMESH = 1

ZDIM(1)    = 0.05

NCELLZ(1)  = 50

DEFAULT_IC (1) = 1

DEFAULT_SURF_IDX(1,5) = 1

DEFAULT_SURF_IDX(1,6) = 2/

Note that, since  Gpyro V2, for 1D cases, XDIM, YDIM, NCELLX, and NCELLY default to 1 and do not need to be defined. This is not the case with older versions.

Example 2 : 3D Block with Multiple Layers

A 3D mesh is defined with three layers represented as obstructions. Each layer has its own initial condition index, typical for simulating multilayer materials.

&GPYRO_GEOM

NMESH = 1

NOBST = 3

! Mesh dimensions (m)

XDIM(1)    = 0.10

YDIM(1)    = 0.10

ZDIM(1)    = 0.04

! Number of cells

NCELLX(1)  = 20

NCELLY(1)  = 20

NCELLZ(1)  = 400

! Default initial condition for whole mesh

DEFAULT_IC(1) = 1

! Boundary conditions for six faces

DEFAULT_SURF_IDX(1,1) = 1

DEFAULT_SURF_IDX(1,2) = 1

DEFAULT_SURF_IDX(1,3) = 1

DEFAULT_SURF_IDX(1,4) = 1

DEFAULT_SURF_IDX(1,5) = 2

DEFAULT_SURF_IDX(1,6) = 3

! Obstructions representing layers

! Top layer (1 cm)

X1(1) = 0.0  ; X2(1) = 0.10

Y1(1) = 0.0  ; Y2(1) = 0.10

Z1(1) = 0.03 ; Z2(1) = 0.04

ICNUM(1) = 3

! Middle layer (1 cm)

X1(2) = 0.0  ; X2(2) = 0.10

Y1(2) = 0.0  ; Y2(2) = 0.10

Z1(2) = 0.02 ; Z2(2) = 0.03

ICNUM(2) = 1

! Bottom layer (1 cm)

X1(3) = 0.0  ; X2(3) = 0.10

Y1(3) = 0.0  ; Y2(3) = 0.10

Z1(3) = 0.0  ; Z2(3) = 0.02

ICNUM(3) = 2/

Notes that  in older versions of Gpyro require SURFIDX2D definitions to override default initial conditions, since Gpyro V2, these are only needed if modifications to the default are required.

Example 3 : I-Beam Geometry

This example defines an I-beam within a single mesh using three obstructions: one vertical web and two horizontal flanges.

&GPYRO_GEOM

NMESH = 1

NOBST = 3

! Mesh dimensions (m)

XDIM(1)    = 0.20

YDIM(1)    = 0.20

ZDIM(1)    = 0.20

! Number of cells

NCELLX(1)  = 100

NCELLY(1)  = 100

NCELLZ(1)  = 100

! Default initial condition

DEFAULT_IC(1) = 1

! Boundary conditions for all six faces

DEFAULT_SURF_IDX(1,1) = 1

DEFAULT_SURF_IDX(1,2) = 1

DEFAULT_SURF_IDX(1,3) = 1

DEFAULT_SURF_IDX(1,4) = 1

DEFAULT_SURF_IDX(1,5) = 1

DEFAULT_SURF_IDX(1,6) = 1

! Obstruction 1 — vertical web

X1(1) = 0.09 ; X2(1) = 0.11

Y1(1) = 0.00 ; Y2(1) = 0.20

Z1(1) = 0.00 ; Z2(1) = 0.20

ICNUM(1) = 1

! Obstruction 2 — bottom flange

X1(2) = 0.00 ; X2(2) = 0.20

Y1(2) = 0.00 ; Y2(2) = 0.20

Z1(2) = 0.00 ; Z2(2) = 0.03

ICNUM(2) = 1

! Obstruction 3 — top flange

X1(3) = 0.00 ; X2(3) = 0.20

Y1(3) = 0.00 ; Y2(3) = 0.20

Z1(3) = 0.17 ; Z2(3) = 0.20

ICNUM(3) = 1

! Different Boundary condition on this surface

SURF_IDX2D(3,2)= 2

/

# Initial Conditions

The initial conditions are defined in the namelist group &GPYRO_IC (See Table 9). This group specifies the physical state assigned to a region at the beginning of the simulation (See Section 6.4) . The &GPYRO_IC namelist provides a flexible way to initialize both simple homogeneous states and more complex material mixtures, including gas-phase composition when required by the model configuration. The total number of distinct initial condition sets is specified by the parameter NIC. Each set is identified by its index IC and may include several properties.

## Condensed-phase temperature

TMP_INITIAL(IC) defines the initial temperature of the solid phase in kelvin. The default value is 300 K.

## Gas-phase temperature

TMPG_INITIAL(IC) defines the initial temperature of the gas phase, also in kelvin. The default value is 300 K. This parameter is only relevant if the gas-phase energy conservation equation is solved (see Section 4.5).

## Pressure

P_INITIAL(IC) specifies the initial pressure in pascals. The default value is 101 300 Pa. This parameter is only considered when pressure equations are solved (see Section 4.3).

## Condensed species mass fractions

For a condensed species of index ISPEC defined in the &GPYRO_SPROPS namelist (see Section 9.0), its initial mass fraction in kg/kg is defined through YI0(IC,ISPEC). By default, all fractions are zero, and the sum over all indices from 1 to NSPEC must equal one. This feature allows a material to be represented either as a pure component or as a mixture of species.

## Gas species mass fractions

Similarly, the initial gas composition can be defined using YJ0(IC,IGSPEC), which specifies the mass fraction of gaseous species IGSPEC (see Section 10.1). This represents the mass of a given gas species relative to all gas species, not the mass of gas relative to the total mass (solid + gas). The default is zero, and the sum over all species indices from 1 to NGSPEC must also equal one. This option is only relevant if gas advection–diffusion is activated (see Section 4.4).

## Example

The following example defines two distinct initial condition sets.

&GPYRO_IC NIC=2

TMP_INITIAL(1)= 273.15

YI0(1,1)=0.7

YI0(1,3)=0.3

TMP_INITIAL(2)= 273.15

YI0(2,3)=1

/

Here, two sets are introduced (NIC=2).The first set assigns a condensed-phase temperature of 273.15 K, with the material represented as a mixture of 70% species 1 and 30% species 3. The second set also has an initial temperature of 273.15 K, but the material is composed entirely of species 3.

# Boundary conditions

Boundary conditions define the interaction between the computational domain and its external environment. They are not required in 0D calculations, since the governing equations reduce to ordinary differential equations that only depend on the initial conditions. For higher-dimensional simulations (1D, 2D, 3D), boundary conditions must be specified for each exposed surface of the mesh (See Section 6.5). Boundary conditions are defined in the namelist group &GPYRO_ALLBC (see Table 10).

## Time-dependent boundary condition definition

The boundary conditions are defined as a sequence of temporal blocks. The total number of blocks is set by NSURF_IDX, and each block is indexed by IBC = 1…NSURF_IDX. For every block, the keyword SURF_IDX(IBC) assigns a boundary surface index, which links several temporal blocks into a single boundary condition. Each block is also associated with a specific time, defined by T(IBC) in seconds. At any instant lying between the times of two consecutive blocks, the applied boundary condition is obtained by linear interpolation of the values defined in those blocks. For times larger than the last time block associated with a given boundary surface index, the boundary condition remains constant and equal to the values prescribed in the final block. This formulation provides flexibility to represent evolving environments, such as ignition, where an additional heat flux can be applied to account for flame exposure.

As discussed in the geometry definition (see Section 6.5), the parameters DEFAULT_SURF_IDX and SURF_IDX2D must reference one of boundary surface index defined in &GPYRO_BC. This mechanism ensures that each geometric surface is consistently associated with the correct boundary condition, which may itself evolve according to its temporal blocks.

## Thermal boundary conditions (condensed phase)

At the solid surface, two types of thermal boundary conditions can be imposed.

### Fixed temperature

A fixed surface temperature can be prescribed through the parameter TFIXED(IBC). If the value is negative, no fixed temperature is applied, which corresponds to the default setting.

### Imposed heat flux

Alternatively, a boundary condition can be defined by imposing a heat flux on the surface. The total heat flux is expressed as the sum of three contributions: convection, external radiation, and re-radiation.

Convection

The convective heat flux at the boundary is written as hc(Tinf-Tsurf), where hc​ is the convective heat transfer coefficient (in W.m-1.K-1) specified by the parameter HC(IBC), Tinf​ is the external gas temperature defined by TINF(IBC), and Tsurf is the surface temperature.

Gpyro also provides an optional correction to account for the effect of pyrolysis gas blowing on convective heat transfer. In this formulation, the effective convective heat transfer coefficient is modified using a Couette-flow approximation [9]:

| hc=mgas''cpgexpmgas''cpghc,0-1 | (21) |
| --- | --- |

where mgas''is the outward gas mass flux, cpg is the gas heat capacity, and hc,0 is the convective heat transfer coefficient in the absence of blowing. This formulation is activated by setting BLOWING = .TRUE. in the &GPYRO_GENERAL namelist group. When this option is enabled, the parameter HC(IBC) is interpreted as the reference coefficient hc,0.

External radiation

An external radiative flux can be prescribed via QE(IBC) (in W·m⁻²). Only a fraction of this flux is absorbed, depending on the surface emissivity (see Section 9.2.4). The absorbed energy can either be deposited directly at the surface or, for more realistic semi-transparent materials, distributed within a penetration depth inside the solid (see Section 9.2.4). This option can, for example, reproduce the radiative heat flux imposed in cone calorimeter experiments.

Re-radiation

The surface also exchanges thermal radiation with its environment. This is represented through the Stefan–Boltzmann law written as qrerad=ϵσ(Tinf4-Tsurf4) , where ϵ is the surface emissivity and σ is the Stefan–Boltzmann constant. This contribution is enabled when the parameter RERADIATION(IBC)=.TRUE.. Since re-radiation is a physical process that always occurs, the default value of this parameter is .TRUE.

## Thermal boundary conditions (gas phase)

When the gas-phase energy conservation equation is solved (See section 4.5), similar types of thermal boundary conditions apply. In this case, the corresponding parameters are QEG(IBC) for external flux, HCG(IBC) for convection coefficient, TINFG(IBC) for external gas temperature, and TFIXEDG(IBC) for imposed fixed temperature. These parameters play roles analogous to those used in the condensed phase but are applied to the gas temperature field.

## Gas transport boundary conditions

### Instantaneous release boundary condition

In the simplest formulation, the gas generated inside the material is assumed to be instantaneously released at the surface. In this case, it is optionally possible to impose a gas inflow at the bottom surface (–z direction) using the parameter MDOTPP(IBC), which specifies the gas flow rate in g·m⁻²·s⁻¹. This can represent, for example, gas injection in fixed- or fluidized-bed configurations, or controlled gas supply in experimental setups. The default value of MDOTPP is zero, and in this case only positive values are allowed. This additional inflow can only be imposed at the bottom surface.

The gas enters the domain at a specified temperature, which can be imposed through the parameter TMPG(IBC). When this option is used, the temperature difference between the injected gas and the solid is automatically taken into account by the thermal solver.

### Pressure-driven gas flow boundary condition

When the pressure solver is activated (see Section 4.3), boundary conditions can be prescribed in two different ways:

Imposed pressure

The pressure at a boundary can be prescribed directly using PRES(IBC) (in Pa). In this case, the mass flux across the boundary is computed consistently with Darcy’s law, as a function of the pressure gradient and the material permeability.

Imposed mass flux

As in the instantaneous-release case, a gas flux is imposed through MDOTPP(IBC) (in g·m⁻²·s⁻¹) and by setting PRES(IBC) to an arbitrary negative value. This flux can be specified across any surface. Positive values correspond to inflow, whereas negative values represent outflow. Setting MDOTPP(IBC)=0 corresponds to an impermeable boundary condition.

## Gas species transport boundary conditions

When the optional species advection–diffusion solver is activated (see Section 4.4), the gas‑phase species boundary condition is determined by whether an external mass flux is prescribed.

Inlet with prescribed mass flux

If a non‑zero input mass flux is imposed at a boundary (MDOTPP(IBC) ≠ 0), the solver uses the external gas‑phase composition specified by YJINF(IBC,J) to define the incoming species fractions for each gaseous species J.

| Yj,surf=Yj,inf | (22) |
| --- | --- |

Boundary without prescribed mass flux

If no mass flux is prescribed (MDOTPP(IBC)=0), the boundary condition follows a convective transfer relation :

| -ψρgDdYjdz =hm(Yj,inf-Yj,surf) | (23) |
| --- | --- |

where Yj,inf is the mass fraction of species j in the external gas phase, it is again taken from the user‑specified YJINF(IBC,J) values, and Yj,surf is the species mass fraction computed at the material surface.  hm is the mass transfer coefficient (kg·m⁻²·s⁻¹) is provided through the parameter HM(IBC).

By default, all  YJINF values are set to zero, and the user must ensure that their sum over the NGSPEC species equals to one.

Practical guidance

At an Outflow surfaces, in most practical cases, advection dominates over diffusion (high Peclet number). For such surfaces, a free-outlet formulation is preferred to allow the gas to exit freely:

| dYjdz=0 | (24) |
| --- | --- |

To implement this, set HM(IBC)=0.

In Diffusion-dominated cases (e.g., very slow smoldering, gentle airflow), a starting estimate for hm can be obtained using the heat/mass transfer analogy:

| hm=hccpg | (25) |
| --- | --- |

## Example

&GPYRO_ALLBC

NSURF_IDX=7

SURF_IDX(1)= 1

HC(1)      = 0

QE(1)      = 0

RERADIATION(IBC)=.FALSE.

SURF_IDX(2)= 2

T(2)       = 0

HC(2)      = 10

TINF(2)    = 300

QE(2)      = 50

SURF_IDX(3)= 2

T(3)       = 49

HC(3)      = 10

TINF(3)    = 300

QE(3)      = 50000

SURF_IDX(4)= 2

T(4)       = 50

HC(4)      = 10

TINF(4)    = 300

QE(4)      = 65000

SURF_IDX(5)= 2

T(5)       = 449

HC(5)      = 10

TINF(5)    = 300

QE(5)      = 65000

SURF_IDX(6)= 2

T(6)       = 450

HC(6)      = 10

TINF(6)    = 300

QE(6)      = 50000

SURF_IDX(7)= 3

TFIXED(7)  =  800

/

In this example, SURF_IDX = 1 is defined with a single temporal block (IBC =1). The condition corresponds to an adiabatic surface: convection is disabled with HC(1) = 0, radiation is set to zero, and re-radiation is deactivated.

SURF_IDX = 2 is defined with five temporal blocks (IBC from 2 to 6). This boundary condition reproduces the heat flux imposed in a cone calorimeter test. Initially, a radiative flux of 50 kW·m⁻² is applied. At 50 s, ignition is assumed to occur, and the flame adds an additional 15 kW·m⁻², leading to a total radiative flux of 65 kW·m⁻². At 450 s, extinction occurs, and the flux drops back to 50 kW·m⁻². Convection is prescribed with hc = 10 W·m⁻²·K⁻¹ and Tinf= 300 K, while re-radiation is activated by default.

SURF_IDX = 3 (IBC=7) corresponds to a fixed-temperature condition, where the surface is maintained at 800 K.

# Condensed-phase thermophysical properties

## Material Definition

Gpyro allows the user to define multiple condensed-phase materials, either solid or liquid. All materials initially present in the sample, as well as those generated during thermal degradation, must be declared. Material properties are specified in the &GPYRO_SPROPS namelist (See Table 11).

The total number of condensed-phase species is set using the parameter NSSPEC. Each species is assigned a unique name via the NAME(IDM) parameter, where IDM is the index of the material ranges from 1 to NSSPEC. Names should be concise, without spaces (underscores are allowed), e.g., wood, char, ash, glass_fibers.

## Thermal properties

Each condensed-phase species, identified by its index IDM, must be assigned a set of thermophysical properties that govern heat transfer and thermal degradation, that include thermal conductivity, specific heat capacity, density, and radiative parameters.

### Thermal conductivity

Thermal conductivity is defined independently in each spatial direction using the parameters KX(IDM), KY(IDM), and KZ(IDM), expressed in W.m-1.K-1. For one-dimensional simulations, only KZ is required. In two-dimensional cases, KX and KZ must be specified. Each conductivity component may vary with temperature according to a power-law expression:

| k(T)=k0​​TTref​n | (26) |
| --- | --- |

where Tref​ is a reference temperature defined in the &GPYRO_GENERAL namelist via the parameter TREF, and n is the exponent specified via NKX(IDM), NKY(IDM)or NKZ(IDM), depending on the direction.

In porous materials, radiative heat transfer within the solid phase may be optionally modeled by enhancing the effective thermal conductivity. This is achieved by adding a radiative term to the standard conductivity expression using the Rosseland approximation :

| keff=k(T)+γσT3 | (27) |
| --- | --- |

Here, γ (unit in m) is an optional user-defined material-specific coefficient specified via the parameter  GAMMA(IDM), and σ is the Stefan–Boltzmann constant.

### Specific Heat Capacity

The specific heat capacity is defined using the parameter C0(IDM), in units (J·kg⁻¹·K⁻¹). As with conductivity, it may vary with temperature according to a power-law model. The exponent is specified via the parameter NC(IDM).

To account for phase change (melting), Gpyro introduces an apparent heat capacity model (see the technical user guide Equation 9c). The melting behavior is controlled by three parameters, TMELT(IDM)the melting temperature in (K), DHMELT(IDM)the latent heat of melting in (J·kg⁻¹), and SIGMA2MELT(IDM), the variance of the Gaussian distribution in (K2) that defines the width of the melting peak. If melting is not to be modeled, these parameters should note be use.

### Bulk density

The bulk density of the material is specified via the parameter R0(IDM) in (kg/m³). It refers to the mass of the material per unit of total volume, including both the solid matrix and any internal voids or pores. In porous materials, this value represents the apparent density as seen at the macroscopic scale and therefore includes the contribution of porosity. It differs from the intrinsic or skeletal density, which accounts only for the mass and volume of the solid phase. As with other properties, temperature dependence may be introduced using a polynomial law, with the exponent defined by NR(IDM).

Note that bulk density is coupled to the local deformation and stoichiometry of the reactions involving the species (See Section 11.2).

### Radiative Properties

The condensed-phase species must also be assigned radiative properties that govern the emission and absorption of thermal radiation at the material surface. These include EMIS(IDM), the surface emissivity (dimensionless), and KAPPA(IDM), the absorption coefficient in (m-1). The absorption coefficient defines the typical penetration depth of external radiation into the material. To model a surface that absorbs all incident radiation at the surface only, KAPPA should be set to a large value (e.g., 9.0E9).

### Example

This example illustrates the definition of two condensed-phase species: a polymeric foam and its degradation product, char. The thermal properties of each material are specified in the GPYRO_SPROPS namelist using indexed parameters.

The first material, foam, is defined with a constant thermal conductivity of 0.15 W·m⁻¹·K⁻¹. The bulk density is set to  40 kg·m⁻³, representing the apparent density including porosity. The specific heat capacity is temperature-dependent and follows a power-law relation cpT=1200T/Tref1.2 J·kg⁻¹·K⁻¹ . The surface emissivity is set to 0.9, and the absorption coefficient of 2700 m⁻¹ defines the penetration depth of external radiation into the material.

The second material, char, is defined with a base thermal conductivity  of 0.1 W·m⁻¹·K⁻¹ and includes an additional radiative contribution modeled via the Rosseland approximation. The effective conductivity is given by keff=0.1+0.01σT3 W·m⁻¹·K⁻¹. The bulk density is 20 kg·m⁻³, and the specific heat 1200 J·kg⁻¹·K⁻¹ . The emissivity is 0.85. To ensure that radiation is absorbed only at the surface, the absorption coefficient is set to a large value: KAPPA(2) = 9D9.

&GPYRO_SPROPS  NSSPEC = 2,

NAME(1)  = 'foam',

K0Z(1)   = 0.15,

R0(1)    = 40,

C0(1)    = 1200,

NC(1)    = 1.2,

EMIS(1)  = 0.9,

KAPPA(1) = 2700,

NAME(2)  = 'char',

K0Z(2)   = 0.1,

GAMMA(2) = 0.01

R0(2)    = 20,

C0(2)    = 1200,

EMIS(2)  = 0.85,

KAPPA(2) = 9D9/

## Transport Properties in Porous Media

The following parameters describe the ability of a condensed-phase material to resist gas transport. These properties are relevant only when modeling gas flow within porous solids. They are active only if the gas-phase transport equation is solved, i.e., when SOLVE_PRESSURE = .TRUE. in the GPYRO_GENERAL namelist. If gas transport is not considered, these parameters may be omitted.

### Solid density

The intrinsic solid density, denoted ρs, is specified via the keyword RS0(IDM) in units of (kg·m⁻³). This value represents the true density of the solid matrix, excluding porosity, and is distinct from the bulk density ρ, which includes the volume of pores. The porosity of each species is computed as ψi=1-ρiρs,i. The mixture-averaged porosity is then calculated as ψ=Xiψi.

### Permeability

The intrinsic permeability of the porous material i Ki=Kix, Kiy, Kiz is defined using the parameters PERMZ(IDM), PERMX(IDM), and PERMY(IDM) for the z, x, and y directions, respectively. It is expressed in m2 and determines the resistance of the porous matrix to gas flow. the mixture-averaged permeability is computed as  K=XiKi.

An alternative option is activated by setting KOZENY_CARMAN=.TRUE. in the GPYRO_GENERAL namelist (default: .FALSE.). In this case, permeability evolves with porosity according to the Kozeny–Carman model that link porosity to permeability [ref]:

| K=ψ31-ψ2XiKi | (28) |
| --- | --- |

### Pore diameter

The mean pore diameter (dp in m), is specified via the keyword PORE_DIAMETER(IDM). This parameter is used in the computation of the volumetric heat transfer coefficient between the gas and solid phases. The coefficient is derived from a Nusselt number correlation that depends on the pore diameter and is discussed in the technical reference guide.

# Gas-phase thermophysical properties

Parameters controlling gas-phase thermophysical properties are specified via the &GPYRO_GPROPS Namelist group. A summary of all parameters is presented in

Table 12. This namelist is used to define the gaseous species produced or reacting during pyrolysis, as well as non-reacting species such as those in the ambient atmosphere (e.g., nitrogen, oxygen).

## Gaseous species definition

The total number of gaseous species is set with the keyword NGSPEC, which must be at least equal to 1, even if the gaseous species conservation equation is not explicitly solved.

Each gaseous species is identified by an index IGAS (from 1 to NGSPEC ) and named via the parameter NAME(IGAS). This parameter is a character string that must be unique for each species (no spaces are allowed). Examples include ‘oxygen’, ‘nitrogen’, or ‘pyrolysate1’.

## Special oxidative species

In the case of oxidative reactions, it is necessary to specify which gaseous species acts as the oxidizer, most commonly oxygen. The keyword IO2 assigns the index of the oxidizer species. This parameter is required whenever oxidative reactions are included (See Section 11.4.3).

## Gas heat capacity

The gas-phase specific heat capacity (J·kg⁻¹·K⁻¹) is prescribed via the keyword CPG. For simplicity, all gaseous species are assumed to share the same heat capacity, independent of temperature. This parameter is required only when gas/solid heat exchange is modeled or when the gas-phase energy conservation equation is solved. As an example, the heat capacity of air is approximately 1000 J·kg⁻¹·K⁻¹, which is the default value.

## Gas molecular weight

The molecular weight of each species (g·mol⁻¹) is specified via the keyword M(IGAS). This property links pressure, density, and temperature through the ideal gas law (ρg=RMPTg) where  Mis the molar mass, R is the universal gas constant, and Tg is the temperature. For reference, the molecular weight of air is 28.96 g·mol⁻¹, which is used as the default value.

## Diffusive gas properties

All gas species are assigned the same mass diffusivity D (in m2.s-1), which is taken as the diffusivity of the oxidative species in a background gas. The diffusivity is calculated using the Chapman–Enskog theory [ref]. It depends on pressure P and Temperature T and is given by:

| ν=D=0.018829T31MA+1MBPσAB2ΩD,AB | (29) |
| --- | --- |

where MA and MB are the molar masses of the oxidative species and the background species (See Section 10.4). σAB and ΩD,AB are parameters derived from the properties of the oxidative and background species. They are defined as:

| σAB=12(σA+σB) | (30) |
| --- | --- |

| ΩD,AB=α1T*α2+α3expα4T*+α5expα6T* T*=Tϵ/kAB ; ϵ/k AB=ϵ/kA ϵ/kB | (31) |
| --- | --- |

with constants:

α1=1.080794   ;   α2=-0.16033   ;  α3=0.605009

α4=-0.88524   ;   α5= 2.115672   ;  α6=-2.98308

Here, σ and ϵ/k are the intrinsic Lennard–Jones parameters of the oxidative and background species.

In the &GPYRO_GPROPS Namelist, the index of the background species is specified by IBG, and the index of the oxidative species by IO2 (see section 10.2 The Lennard–Jones parameters  SIGMA(IGAS)(σ specified in Å) and EPSOK(IGAS) (ϵ/k specified in K)  are required only for these two species. Reference values for common gases are provided in Appendix C. Gas properties  and are taken from ( Bird et al. [ref]).

## Gas viscosity

In Gpyro, the gas kinematic viscosity ν (in m2.s-1) is assumed equal to the gas mass diffusivity D (Sc=ν/D=1). This approximation is commonly used for gases. Consequently, the gas viscosity required for Darcy’s law is taken as the diffusivity values described in Section 10.5).

## Example

The following is a sample &GPYRO_GPROPS line, consistent with the &GPYRO_GYIELDS example line given in the next sections.

&GPYRO_GPROPS NGSPEC    = 4

IBG       = 3

IO2       = 2

CPG       = 1000.

NAME(1) = 'pyrolysate', M(1) = 100.

NAME(2) = 'oxygen'    , M(2) = 32. ,SIGMA(2)=3.433,EPSOK(2)=113

NAME(3) = 'nitrogen'  , M(3) = 28.2,SIGMA(3)=3.681,EPSOK(3)=91.5

NAME(4) = 'products'  , M(4) = 44. /

# Condensed-phase reactions

Condensed-phase reactions describe the transformation of  condensed species (solid or liquid ) into other condensed products and gaseous species. Pure gas-phase reactions can also be modeled but are presented separately (see Section 12.0). This section focuses on condensed-phase reactions.

Gpyro allows the definition of multiple condensed phase reactions, which may occur in parallel, sequentially, or in mixed schemes. These reactions can represent various processes, including oxidative or catalytic pathways, phase changes, or purely thermal decomposition. Several kinetic models are available, offering flexibility in describing different reaction mechanisms.

Input parameters are provided through two namelist groups:

- &GPYRO_RXNS, which specifies the number of reactions, the condensed species involved (reactant and product), kinetic parameters, oxygen dependence, deformation factors, and reaction enthalpies (See Table 13).

- &GPYRO_GYIELDS, which defines the gaseous species consumed or produced by each reaction, and their relative stoichiometric proportions (species-yield matrix) (See Table 14).

The formulation and parameter definitions follow the formalism described in the Technical Reference (Chapter 3) [ref].

## Declaring Condensed-Phase Reactions

### Declaring a reaction

The total number of condensed-phase reactions is defined in the namelist &GPYRO_RXNS through the keyword NRXNS. Each reaction is identified by its index IRXNS  from  1 to NRXNS.

In Gpyro , the general formulation of a condensed-phase reaction k is written as:

| A→θkB+1-θkG | (32) |
| --- | --- |

Where A is the only condensed reactant, B is the only condensed product, G represents all gaseous species consumed or generated by the reaction, and θk is the stoichiometric coefficient of the condensed product (See Section 11.2)

The condensed reactant (A) name is specified with CFROM(IRXNS) and that of condensed product (B) with CTO(IRXNS). Both names must exactly match species names declared in &GPYRO_SPROPS and are case-sensitive. Associated gaseous products or reactants, as well as their stoichiometry, are defined separately in &GPYRO_GYIELDS (See section 12.0).

### Special case: non-charring reactions

In the default formulation, every reaction must produce a condensed product. However, in some cases the condensed phase volatilizes completely (non-charring reactions), and no solid residue is left. This type of decomposition can be represented by assigning the keyword CTO(IRXNS) = 'gases'. When this option is used, the entire mass of the condensed reactant is converted into gaseous species, whose distribution is then specified in &GPYRO_GYIELDS.

### Special case: multiple condensed products

Since Gpyro restricts each reaction to a single condensed product, reactions that conceptually generate several condensed products must be decomposed into multiple parallel reactions. Each parallel reaction represents the formation of one of the products, while sharing the same kinetic parameters (see Section 11.4) and enthalpy of reaction (see Section 11.5) as the original process. The stoichiometric coefficients must then be adjusted individually to reproduce the correct mass balance for each product (See Section 11.2).

## Condensed-Phase Stoichiometry and  Deformation

### Definition of the Stoichiometric Coefficient

As seen in Equation (32) separates the fraction of the condensed reactant mass that remains as condensed product from the fraction converted into gas (1-θk). The detailed stoichiometry of gaseous species is discussed in Section 11.4 here, the focus is placed on the condensed phase.

In Gpyro, θk is not imposed directly. Instead, it is determined from the densities of the reactant and product, together with a deformation factor χk, according to:

| θk=1+ρBρA-1χk | (33) |
| --- | --- |

where ρA and ρB are the densities of the condensed reactant and product (see Section 9.2.3). The parameter χk, which is imposed via the keyword CHI(IRXNS), gives the user explicit control over deformation, and together with the densities, determines the value of θk.

### Deformation and Volume Change

The volumetric deformation induced by a reaction can be quantified as the ratio between the final volume after reaction (Vf) and the initial volume before reaction (Vini) and it is noted Dk. This ratio is expressed in terms of densities and stoichiometry as:

| Dk=VfVini=ρAmBρBmA=ρAθkρB | (34) |
| --- | --- |

Thus, given the density of the reactant (ρA) and the stoichiometric coefficient (θk), the desired deformation can be reproduced by selecting an appropriate value of ρB to respect Equation (33). The corresponding value of χk is then deduced from Equation (34) Erreur ! Source du renvoi introuvable. to ensure consistency between stoichiometry and deformation.

Some values of the deformation factor χk correspond to characteristic physical situations:

- When χk=0, then θk=1 (Equation (33)), meaning that the reaction is purely solid–solid, and the resulting deformation is governed solely by the density ratio of reactant and product.

- When χk=1, then θk=ρB/ρA. In this case, the reaction corresponds to a typical charring process with no net volume change, as deduced from Equation (34).

- If χk<1  and ρB<ρA , the material shrinks, meaning its volume decreases. The smaller χₖ is, the more pronounced the shrinkage.

- If χk>1  and ρB<ρA , the material swells, leading to an increase in volume. The larger χₖ is, the stronger the swelling effect.

## Gaseous stoichiometry: the species‑yield matrix GYIELDS

The gaseous stoichiometry for each condensed-phase reaction is specified in the &GPYRO_GYIELDS namelist through the parameter GYIELDS(IGAS,IRXNS). These entries form the species yield matrix ys,j,k described in the Technical Reference [ref].

As shown in Equation 6 , for a given reaction k, the total fraction of the reactant mass converted into gases is (1-θk). The contribution of a particular gaseous species j is:

| θk,j=ys,j,k(1-θk) | (35) |
| --- | --- |

Here, Ys,j,k corresponds to GYIELDS(IGAS,IRXNS). A positive value indicates that the species is produced, a negative value means it is consumed (acting as a reactant), and a value of zero means the species does not participate in the reaction, which is the default. For each reaction IRXNS, the sum of all GYIELDS(:,IRXNS) entries must equal 1 to ensure mass conservation.

By defining this matrix, it becomes possible to fully specify which gaseous species participate in each reaction, both as reactants and as products, as well as their stoichiometric proportions.

## Kinetics of Condensed-Phase Reactions

### General formulation

The rate of a condensed-phase reaction k, denoted rk​ (in kg.m-3.s-1), represents the destruction rate of the condensed reactant A. In Gpyro, this rate is expressed as:

| rk=gkαAkkTgkYO2 | (36) |
| --- | --- |

where:

- gkαA is the kinetic model function, which depends on the conversion degree of the species A noted as αA​.

- kkT is the temperature-dependent rate constant, given by the Arrhenius law.

- gkYO2 accounts for the effect of local oxidative concentration (if applicable).

The most common case is thermal decomposition without oxidation, where gkYO2=1.

Note: The kinetic description and associated parameters in Gpyro are not necessarily equivalent to those found in other literature sources. It is strongly recommended to validate kinetic data against Thermogravimetric Analysis (TGA) experiments to ensure consistency with the intended behavior. Given the complexity of these parameters, performing inverse modeling on TGA data can be an effective strategy to obtain values that match experimental observations (see Chapter 14.0).

### Temperature Dependence (Arrhenius Law)

The influence of temperature on the reaction rate constant is described by the Arrhenius expression:

| kk(T)=Zkexp-EkRT | (37) |
| --- | --- |

In this formulation, Zk represents the pre-exponential factor in s-1, which is specified for each reaction through the parameter Z(IRXNS). The term Ek corresponds to the activation energy in kJ.mol-1 and is defined by E(IRXNS). The constant R is the universal gas constant, and T is the local temperature in Kelvin.

### Kinetic models

The kinetic term in the reaction rate expression accounts for how the availability of the condensed reactant and its conversion state influence the reaction progress. In Gpyro, this term is written as:

| gkαA=fk(αA)αAmkρYA(1-αA) | (38) |
| --- | --- |

Available models

Here, fk(αA) represents the kinetic model function, it can take different forms depending on the kinetic model selected. This choice is controlled by the parameter IKINETICMODEL(IRXNS), which refers to an index in the kinetic model presented in Table 3. 3. The default model is the n-th order model (IKINETICMODEL(IRXNS)=0), which is the most commonly used one.

For models that require an order parameter (nk), the user must specify ORDER(IRXNS). For catalytic reactions (IKINETICMODEL = 9), additional parameter KCAT(IRXNS) (catalytic constant) and ICAT(IRXNS) (index of the catalytic species) are required.

Additionally in Equation (38), the parameter mk specified by M(IRXNS) introduces an additional exponent on αA​ that can be used to adjust the shape of the reaction rate curve. The default value is zero

Table 3: Available kinetic models in Gpyro.

| IKINETICMODEL | Description | fk(αA) |
| --- | --- | --- |
| 0 | nth order | 1-αAnk |
| 1 | Nucleation and nucleus growing | 1nk1-αA[-ln1-αA]nk |
| 2 | Phase boundary reaction | 1-αAnk |
| 3 | Diffusion – plane symmetry | 12αk |
| 4 | Diffusion – cylindrical symmetry | 1-ln1-αA |
| 5 | Diffusion – spherical symmetry | 32 11-αA-1/3-1 |
| 6 | Diffusion – Jander’s type | -32 1-αA-1/3αA |
| 7 | Potential law | 1nkαA1-nk |
| 8 | Reaction order | 1-αA1-nknk |
| 9 | Catalytic | 1-αAnk(1+Kcat,kαicat) |

The Conversion Rate

In Gpyro, the conversion degree of a condensed species, denoted as αA, represents the fraction of the initial material that has reacted. It varies from 0 (completely unreacted) to 1 (fully consumed). Two formulations are available to compute this quantity (See Equation (39)).

The default and most general formulation is activated when CONVENTIONAL_RXN_ORDER = .FALSE. in the &GPYRO_GENERAL namelist. This approach ensures that αA remains within the [0, 1] range for all species, including those produced as intermediate products. It provides a physically consistent definition of conversion degree, especially in complex reaction networks.

In contrast, the alternative formulation, which is closer to that used in other pyrolysis codes such as FDS, is selected by setting CONVENTIONAL_RXN_ORDER=.TRUE.. While this version also keeps αA between 0 and 1, it does not necessarily span the full range from 0 to 1. As a result, it may underestimate the extent of conversion, particularly for intermediate species, and does not fully capture the intended definition of the conversion degree. Unless you have a specific reason to use the alternative formulation, it is strongly recommended to keep CONVENTIONAL_RXN_ORDER=.FALSE..

| (1-αA)=matm0(t=0) if CONVENTIONAL_RXN_ORDER=.TRUE.ma(t)mat=0+0tωf,a'''ΔVdθ if CONVENTIONAL_RXN_ORDER=.FALSE. | (39) |
| --- | --- |

Oxygen Dependence

To account for oxidation, three options are available depending on the parameters IO2TYPE(IRXNS) and ORDERO2(IRXNS) (noted nO2,k) as express in Equation (40). By default, ORDERO2(IRXNS)=0 and IO2TYPE=1, which means the reaction is not oxidative.

| gkYO2=1, if &n02,k=0 1+YO2nO2,k-1, if &n02,k≠0 and IO2TYPE=0YO2nO2,k if n02,k≠0 and IO2TYPE=1 | (40) |
| --- | --- |

As a reminder, you should specify which gaseous species represents oxygen, as described in Section 10.2.

Critical Temperature

Each reaction can be assigned a critical temperature TCRIT(IRXNS). When the local temperature is below this threshold, the reaction rate is set to zero:

| rk=0 if T<Tcrit,k | (41) |
| --- | --- |

This feature is useful when modeling materials that undergo a significant change in properties at a specific temperature. For example, you can represent such behavior by defining two separate condensed species: one that exists below the critical temperature and another that exists above it, each with its own properties. An instantaneous reaction occurring at the critical temperature can then be used to switch from one species to the other.

## Enthalpy of reaction

The enthalpy of reaction is a thermodynamic quantity that refers to the energy released or absorbed during a chemical transformation k under reference conditions. It is denoted by ΔHk and expressed per unit mass (J·kg-1) or per mole.

In practice, defining ΔHk is not straightforward. Experimental determination is often ambiguous because it is difficult to separate latent heat effects, the energy required for volatilization, and sensible heat changes. Even for a single polymer, reported values of reaction enthalpy can vary by an order of magnitude depending on how Differential Scanning Calorimetry (DSC) data are interpreted and which methodology is applied [ref]. Consequently, the pyrolysis modeling community lacks a unified and rigorous standard for defining ΔHk.

To address this, Gpyro provides two approaches for computing the volumetric heat source term Qk''' (in J·m-3.s-1) from the enthalpies of reaction:

### Constant Heat of Reaction

The most common approach is to assume a constant enthalpy of reaction. This is achieved by setting CONSTANT_DHVOL=.TRUE. in the &GPYRO_GENERAL namelist (default option). Under this assumption, the volumetric heat source term is expressed as:

| Qk'''=-ωfB,k'''ΔHsol,k-ωfg,k'''ΔHvol,k | (42) |
| --- | --- |

where ΔHsol,k and ΔHvol,krepresent the solid and volatile contributions to the enthalpy of reaction, specified by DHS(IRXNS) and DHV(IRXNS) respectively. The terms ωfB,k''' and ωfg,k'''​ are the production rates of condensed and gaseous products, while ωdA,k''' is the destruction rate of the reactant species (all in kg·m-3.s-1).

If ΔHvol,k=ΔHsol,k​, this simplifies to:

| Qk'''=-ωdA,k'''ΔHsol,k | (43) |
| --- | --- |

It is recommended to set these two values equal so that the heat of reaction is consistently defined per kilogram of the reactant species A​ consumed by reaction k.

Positive values of ΔHsol,k and ΔHvol,kcorrespond to endothermic reactions, while negative values indicate exothermic reactions. By default, DHV(IRXNS)=DHS(IRXNS)=0.

### Conservative Formulation of Heat of Reaction

A more rigorous approach ensures that the volumetric heat source term satisfies energy conservation. This is activated by setting CONSTANT_DHVOL=.FALSE. in the &GPYRO_GENERAL namelist. In this case, the source term is computed as:

| Qk'''=-ωfB,k'''ΔHsol,k0-ωFg,k'''ΔHvol,k0- ωfg,k'''hA(T)-hg(T) | (44) |
| --- | --- |

Here, the user specifies DHV(IRXNS) as :

| ΔHvol,k0=hg0-hA0 | (45) |
| --- | --- |

which is the difference in reference enthalpy between the gas and the condensed reactant, and DHS(IRXNS):

| ΔHsol,k0=hB0-hA0 | (46) |
| --- | --- |

which is the difference in reference enthalpy between the condensed product and the condensed reactant.

This formulation guarantees that the energy balance accounts for both chemical enthalpy changes and sensible enthalpy differences, as detailed in the Technical Guide (see Chapter 3.5).

The main challenge with this approach lies in determining accurate reference enthalpies, which are sometimes available for simple gases [ref] but are often difficult to characterize for complex condensed phases. Furthermore, when multiple reactions and steps are involved, the definitions of ΔHvol,k0and ΔHsol,k0 must remain consistent across all reactions to ensure a coherent reference enthalpy framework for all species.

## Example

Here, an example is provided to illustrate how to define a condensed-phase reactions. It is important to note that the following is only a partial input file, where only the parameters necessary to define the reactions are explicitly specified. Although the procedure for declaring reactions in Gpyro may initially seem complex, this example demonstrates that, in practice, the process is straightforward.

&GPYRO_GENERAL

CONSTANT_DHVOL = .TRUE. ! Default option

CONVENTIONAL_RXN_ORDER = .FALSE. ! Default option

…

/

&GPYRO_SPROPS  NSSPEC = 4

NAME(1) = 'foam_virgin', R0(1) = 2200.0,  …

NAME(2) = 'foam_critic', R0(2) = 2000.0,  …

NAME(3) = 'char',        R0(3) = 400.0,   …

NAME(4) = 'ash',         R0(4) = 200.0,   …

/

&GPYRO_RXNS NRXNS = 4

!Reaction 1 : critical transition

CFROM(1) = 'foam_virgin', CTO(1) = 'foam_critic'

E(1) = 0.0, A(1) = 1.0E+18

TCRIT(1) = 500.0

CHI(1) = 0.0

!Reaction 2 : decomposition into char + CO

CFROM(2) = 'foam_critic', CTO(2) = 'char'

E(2) = 220.0, A(2) = 1.0E+12

CHI(2) = 0.5

!Reaction 3 : oxidation of char into ash + CO

CFROM(3) = 'char', CTO(3) = 'ash'

E(3) = 180.0, A(3) = 5.0E+10

ORDER(3) = 3, IO2TYPE(3) = 1, ORDERO2(3) = 1

CHI(3) = 1.0

DHS(3) = -1.0E+5, DHV(3) = -1.0E+5

!Reaction 4 : volatilization of char into HCN

CFROM(4) = 'char', CTO(4) = 'gases'

E(4) = 300.0, A(4) = 2.0E+11

DHS(4) = 1.0E+5, DHV(4) = 1.0E+5

/

&GPYRO_GPROPS  NGSPEC = 3

IO2 = 1

NAME(1) = 'O2'

NAME(2) = 'CO'

NAME(3) = 'HCN'

/

&GPYRO_GYIELDS

! Reaction 1 : foam_virgin → foam_critic (no gases, force sum = 1.0)

GYIELDS(1,1) = 1.0

! Reaction 2 : foam_critic → char + CO

GYIELDS(2,2) = 1.0

! Reaction 3 : char + O2 → ash + CO

GYIELDS(1,3) = -1.0   ! oxygen consumed

GYIELDS(2,3) = 2.0    ! CO produced

! Reaction 4 : char → gases (HCN)

GYIELDS(3,4) = 1.0

/

First reaction

The first reaction introduces a critical transition. Below 500 K, the material exists as foam_virgin, and once the temperature reaches 500 K it instantaneously transforms into foam_critic. Since χ = 0, the stoichiometric coefficient is θ1= 1, which means the transformation is purely solid-to-solid without mass loss. The density change, however, induces a small deformation:

D1=ρvirginρcriticθ1=22002000=1.1

Thus, the volume expands by about 10%. No gases are released, but to avoid an error message, GYIELDS(1,1) is set to 1.0.

Second reaction

The second reaction models the decomposition of foam_critic into char and carbon monoxide. With χ2=0.5, the stoichiometric coefficient becomes

θ2=1+ρcharρcritic-1χ2=60%

The associated deformation is

D2=ρcriticρcharθ2=3

Since neither IKINETICMODEL(2) nor ORDER(2) are specified, the default kinetic model applies: a first-order thermal decomposition reaction. The reaction rate is therefore expressed as:

r2=ρYcriticZ2exp-E2RT

Finally, since neither DHS(2) nor DHV(2) are defined, the heat of reaction is assumed to be zero.

Third reaction

The third reaction describes the oxidation of char into ash and carbon monoxide. In this case, oxygen is explicitly consumed, with GYIELDS(1,3)=−1.0, while two CO molecules are produced, with GYIELDS(1,3)=2.0. The oxidative behavior is controlled by IO2TYPE(3)= 1 and ORDERO2(3)=1, indicating first-order dependence on oxygen concentration. . Note that this will not occur unless oxygen is present.

The stoichiometric coefficient of the condensed phase is calculated as:

θ3=1+ρashρchar-1χ3=50%

Since CHI(3)=1, the reaction corresponds to a charring process without net deformation.

The negative enthalpy values (DHS(3) and DHV(3)) specify that the oxidation is exothermic. The reaction follows an nth-order model with order three for the condensed species, combined with a first-order dependence on oxygen. The reaction rate is therefore expressed as:

r3=1-αchar2ρYcharY02Z3exp-E3RT

Fourth reaction

Finally, the fourth reaction represents a non-charring process, where char volatilizes entirely into HCN. Because CTO(4) = "gases", no condensed product remains and θ4=0. The enthalpy values defined here are positive, indicating that this is an endothermic process.

# Homogeneous gas-phase reactions

In addition to heterogeneous reactions involving the condensed and gaseous phases, the user may also specify any number of homogeneous gas phase reactions. Such reactions can be used to account for tar cracking where large hydrocarbon molecules are broken into smaller hydrocarbon molecules or the oxidation of pyrolysate as it flows through a char layer toward the surface where both the oxygen concentration and temperature are highest.

For convenience, a summary of all input parameters related to gas-phase reactions is provided in Table XX of the Appendix.

## Declaring Gas-Phase Reactions

The total number of homogeneous gas-phase reactions is defined in the namelist &GPYRO_HGRXNS through the keyword NHGRXNS. Each reaction is identified by its index IGRXNS, ranging from 1 to NHGRXNS.

In Gpyro, the general formulation of a gas-phase reaction l is written as:

| yg,A,lAl+yg,B,lBl→∑yg,j,lGj | (47) |
| --- | --- |

Here, Al and Bl are the gaseous reactants, while Gj represents the gaseous species j. The coefficients yg,j,l form the homogeneous gaseous species yield matrix, analogous to the condensed-phase yield matrix described in Section 11.3.

A species Gj is considered a product if yg,j,l>0. If yg,j,l=0, the species does not participate in the reaction, and if yg,j,l<0, the species acts as a reactant. This formulation allows for multiple reactants to be specified in a single gas-phase reaction.

However, for practical reasons, Gpyro isolates two primary reactants (Al and Bl ​) that are explicitly declared by the user. Their names are given through CREACTANT1(IGRXNS) and CREACTANT2(IGRXNS). This structure makes it possible to directly control the reaction rate law as a function of the concentrations of two designated species, while any additional reactants or products are accounted for through the yield matrix. Both names are case-sensitive and must match those defined in &GPYRO_GPROPS.

The yield coefficients yg,j,l are then defined in the namelist &GPYRO_HGYIELDS through the parameter HGYIELDS(IGAS,IGRXNS). By default, all entries are zero. To ensure correct stoichiometry, the entry corresponding to Al must be set to −1, and the sum over all gaseous species of HGYIELDS(:,IGRXNS) must equal zero.

## Chemical Kinetics

The rate of a homogeneous gas-phase reaction l is expressed as:

| rl=ψAlPlBlqlTblZlexp-ElRTg | (48) |
| --- | --- |

Where:

- ψ is the porosity (See Section 9.3.1),

- Al and Bl are the molar concentrations of the two reactants,

pl and ql are the partial reaction orders, specified via P(IGRXNS) and Q(IGRXNS),

- bl is an empirical temperature exponent, defined by B(IGRXNS),

- Zl is the pre-exponential factor (s-1), given by Z(IGRXNS),

- El is the activation energy (kJ·mol⁻¹), defined by E(IGRXNS),

- R is the universal gas constant,

- Tg is the local gas temperature in Kelvin.

## Heat of Reaction

The heat of a gas-phase reaction, per unit mass of reactant 1 consumed (J·kg⁻¹), is specified with DH(IGRXNS). By convention, DH is negative for exothermic reactions and positive for endothermic reactions. If not specified, the default value is zero.

## Example

The following is an example of &GPYRO_HGRXNS  and &GPYRO_HGYIELDS line:

&GPYRO_HGRXNS NHGRXNS       =  1

CREACTANT1(1) =  'pyrolysate'

CREACTANT2(1) =  'oxygen'

P         (1) =  1.0

Q         (1) =  1.0

Z         (1) =  1D10

E         (1) =  195.0

DH        (1) = -15D6 /

&GPYRO_HGYIELDS HGYIELDS(1,1) = -1.0

HGYIELDS(2,1) = -3.0

HGYIELDS(3,1) =  0.0

HGYIELDS(4,1) =  4.0 /

This input specifies that reaction 1 consumes 1 kg of gaseous species 1 (‘pyrolysate’) and 3 kg of gaseous species 2 (‘oxygen’) and produces 4 kg of gaseous species 4 (‘products’). Species 3 does not participate in the reaction. The stoichiometry is enforced through the yield matrix, where negative coefficients correspond to consumed reactants and positive coefficients to produced species. From the kinetic definition, this is a second-order reaction, with partial orders pl=1 for pyrolysate and ql=1 for oxygen. The enthalpy of reaction is negative, meaning that the reaction is exothermic.

# Output files

In Gpyro, the dump routines allow users to specify which simulation data should be saved during the run. These settings are configured via the &GPYRO_OUTPUT line, and all output files are named using the string defined by the CASENAME keyword.

The available output types are organized into three main categories:

- Point quantities (values at specific locations),

- Profiles (spatial distributions in a specific direction),

- Smokeview files, used to visualize 2D/3D simulation results [ref].

Details of each output type are provided in Sections 0 to 13.4. The complete list of supported output quantities is available in Table 4. Note that not all quantities can be exported for every output type, limitations are indicated in the tables.

It is also possible to control the output frequency to capture rapid phenomena such as ignition or flame spread. This functionality is described in Section 13.5.

In addition to the standard output types, Gpyro generates several auxiliary files for diagnostics, performance monitoring, and debugging, as described in Section 13.6.

## Available quantitites to dump

A summary of all available input parameters for the &GPYRO_OUTPUT namelist is provided in Table 17.

Table 4: Quantities available for dump.

| Name | Index meaning | Description | Units |
| --- | --- | --- | --- |
| ‘TEMPERATURE’ |  | Condensed-phase temperature | ºC |
| ‘ENTHALPY’ |  | Condensed-phase enthalpy | J/kg |
| ‘YI’ | ISSPEC | Mass fraction of condensed-phase species | kg/kg |
| ‘XI’ | ISSPEC | Volume fraction of condensed-phase species | m3/m3 |
| ‘CI’ | ISSPEC | Concentration of condensed-phase species | kg/m3 |
| ‘YISUM’ |  | Sum of all condensed-phase mass fractions | kg/kg |
| ‘REACTION_RATE_K’ | IRXN | Rate of kth condensed-phase reaction | kg/m3-s |
| ‘YJ’ | IGSPEC | Mass fraction of gaseous species | kg/kg |
| ‘CJ’ | IGSPEC | Concentration of gaseous species | kg/m3 |
| ‘YJSUM’ |  | Sum of all gas-phase mass fractions | kg/kg |
| ‘REACTION_RATE_L’ | IHGRXN | Rate of ℓth homogeneous gaseous reaction | kg/m3-s |
| ‘S’ |  | Source term in condensed-phase energy eq. | J/m3-s |
| ‘QSC’ |  | Source term induces by chemical reaction | J/m3-s |
| ‘THERMAL_CONDUCTIVITY_Z’ |  | Effective thermal conductivity in z-direction | W/m-K |
| ‘THERMAL_CONDUCTIVITY_X’ |  | Effective thermal conductivity in x-direction | W/m-K |
| ‘THERMAL_CONDUCTIVITY_Y’ |  | Effective thermal conductivity in y-direction | W/m-K |
| ‘BULK_DENSITY’ |  | Condensed-phase bulk density | kg/m3 |
| ‘SOLID_DENSITY’ |  | Condensed-phase pure solid density | kg/m3 |
| ‘SPECIFIC_HEAT_CAPACITY’ |  | Condensed-phase specific heat capacity | J/kg-K |
| ‘DLTZN’ |  | Grid size in z direction | m |
| ‘DLTXN’ |  | Grid size in y direction | m |
| ‘DLTYN’ |  | Grid size in x direction | m |
| ‘PRESSURE’ |  | Overpressure | Pa |
| ‘MASS_FLUX_TOTAL_Z’ |  | z-direction mass flux | g/m2-s |
| ‘MASS_FLUX_TOTAL_X’ |  | x-direction mass flux | g/m2-s |
| ‘MASS_FLUX_TOTAL_Y’ |  | y-direction mass flux | g/m2-s |
| ‘GAS_TEMPERATURE’ |  | Gas-phase temperature | ºC |
| ‘GAS_ENTHALPY’ |  | Gas-phase enthalpy | J/kg-K |
| ‘GAS_DENSITY’ |  | Gas-phase density | kg/m3 |
| ‘TG-T’ |  | Gas temperature minus solid temperature | ºC |
| 'SHYI' |  | SHYI correction coefficient | J/m3-s |
| ‘PERMEABILITY_Z’ |  | z-direction Permeability | m2 |
| ‘PERMEABILITY_X’ |  | x-direction Permeability | m2 |
| ‘PERMEABILITY_Y’ |  | y-direction Permeability | m2 |
| ‘POROSITY’ |  | Porosity | m3/m3 |
| ‘D12’ |  | Gaseous diffusion coefficient | m2/s |
| ‘SHGP’ |  | Positive part of gas energy eq. source term | J/m3-s |
| ‘SHGM’ |  | Negative part of gas energy eq. source term | J/m3-s |
| ‘QSG’ |  | Rate of heat transfer from solid to gas phase | J/m3-s |
| ‘RYIDZSIGMA’ | ISPEC |  | kg/m2 |
| ‘UNREACTEDNESS’ | ISPEC | 1 -  | - |
| ‘GOMEGA3’ |  | Conversion rate of solid to gas | kg/m3-s |
| 'GGR’3 | IGSPEC | Gas generation rate | kg/m3-s |
| ‘RE’ |  | Reynolds number | - |
| ‘NU’ |  | Nusselt number | - |
| ‘HCV’ |  | Volumetric heat transfer coefficient | W/m3-K |
| ‘M/M0’ |  | Mass divided by initial mass in a cell | kg/kg |
| ‘TOTAL_MASS’3 |  | Total mass in the mesh | kg |
| ‘MLR’3 | IGSPEC1 | Flux of gaseous species | 4 |
| ‘MPPI’2,3 | ISPEC | Mass per unit area of condensed species | kg/m2 |
| ‘THICKNESS’2,3 |  | Total thickness | m |
| ‘DT’2,3 |  | Timestep | s |
| ‘N_ITERATIONS’2,3 |  | Number of iterations for convergence | - |

ISSPEC: 	Index of solid species

IGSPEC: 	Index of gaseous species

IRXN: 	Index of heterogeneous/condensed-phase reaction

IHGRXN: Index of heterogeneous gaseous reaction

1	Specify IGSPEC = 0 for total MLR.

2	Does not vary spatially

3	Available only as a point quantity

4	The unitie of  MLR depends on the simulation dimensionality. In 1D, it is expressed in g·m⁻²·s⁻¹, in 2D in g·m⁻¹·s⁻¹, and in 3D in g·s⁻¹. For 0D simulations (TGA), it is defined as the total mass of the sample divided by the initial mass, multiplied by 1000 and expressed per second. This unconventional unit is kept for backward compatibility.

## Point dumps

### Configuration

Several quantities can be exported as point values, as listed in Table 4. To activate this feature, the total number of point quantities must be specified using the keyword N_POINT_QUANTITIES.

For each quantity I (with I ranging from 1 to N_POINT_QUANTITIES), the following parameters may be defined:

- POINT_QUANTITY(I): name of the quantity to dump,

- POINT_Z(I): vertical coordinate of the point (in meters, with 0 at the upper surface),

- POINT_X(I): optional x coordinates, required only for 2D or 3D simulations,

- POINT_Y(I): optional y coordinates, required only for 3D simulations,

- POINT_IMESH(I): optional mesh identifier; required when using multiple meshes to specify which mesh the point belongs to, If POINT_IMESH(I)=0, the quantity will be dumped for all meshes.

- POINT_QUANTITY_INDEX(I): optional index, used for quantities requiring an additional identifier (e.g., species index for mass fractions ('YI') ).

Point quantities follow a Lagrangian approach: each point is associated with a computational cell at the start of the simulation. During the simulation, the geometry may deform (due to swelling, shrinkage, etc.), but the output always refers to the initial location of the assigned cell. Coordinates defined by the user are thus interpreted relative to the initial, undeformed mesh.

Some quantities can be requested via the point-dump mechanism even though they are not spatially localized. Examples include the mass loss rate (‘MLR’) or total sample mass (‘TOTAL_MASS’).

For such quantities, spatial coordinates (and POINT_Z(I), POINT_X(I), and POINT_Y(I)) are not required and can be omitted.

The output frequency is controlled by the DTDUMP_POINT parameter, which sets the time interval (in seconds) between successive dumps.

### Example

The configuration below shows how to request five point quantities:

&GPYRO_OUTPUT N_POINT_QUANTITIES      = 5

DTDUMP_POINT            = 0.1

POINT_QUANTITY(1)       = 'TEMPERATURE'

POINT_Z(1)              = 0.0

POINT_QUANTITY(2)       = 'YI'

POINT_QUANTITY_INDEX(2) = 1

POINT_Z(2)              = 0.01

POINT_QUANTITY(3)       = 'YI'

POINT_QUANTITY_INDEX(3) = 2

POINT_Z(3)              = 0.01

POINT_QUANTITY(4)       = 'MLR'

POINT_QUANTITY_INDEX(4) = 0

POINT_QUANTITY(5)       = 'MLR'

POINT_QUANTITY_INDEX(5) = 1

.

.

.

In this example, a total of five quantities are dumped every 0.1 seconds. The first is the top surface temperature at z = 0.0 m. The second and third are the condensed-phase mass fractions of species 1 and 2, extracted at 10 mm depth. The fourth requests the total mass loss rate (MLR), since the used index is 0. The fifth requests the mass loss rate of species 1 (using index 1).

### Output File Format

Point quantities are written to a CSV file named using the value provided by the CASENAME parameter, with the suffix  _summary.csv.

The first row of the file contains the column headers:

- The first column corresponds to time,

- The following columns correspond to the point quantities, listed in the order defined by their index I in the input (POINT_QUANTITY(1), POINT_QUANTITY(2), etc.).

Each subsequent row contains the data values at a given output time, as defined by DTDUMP_POINT.

## Profile dumps

Profile dumps allow the user to export the spatial distribution of a quantity along a given axis (x, y, or z) at specified locations in the remaining orthogonal directions. This functionality is compatible with 1D, 2D, and 3D configurations.

### Configuration

Quantities to dump

The number of profile quantities to output is defined using N_PROFILE_QUANTITIES parameter. Each profile J (from 1 to N_PROFILE_QUANTITIES)  to be dumped is defined by the parameter PROFILE_QUANTITY(J), which specifies the quantity to be extracted (see Table 4). It should be noted that not all quantities listed in Table 4 are available for profiles, as indicated by Note 3.

As with point quantities, PROFILE_QUANTITY_INDEX(J) is an optional parameter used to specify the index when needed, particularly for species-related quantities.

Profile Direction and Positioning

The profile orientation is specified using PROFILE_DIRECTION(J), while its spatial location depends on the dimensionality of the simulation and is defined by PROFILE_COORD1(J) and PROFILE_COORD2(J).

In 1D simulations: no coordinate needs to be specified, as the domain is inherently one-dimensional.

In 2D simulations : only PROFILE_COORD1(J) is required. It defines the position along the axis perpendicular to the profile direction. For example, if PROFILE_DIRECTION = 'z', the profile is extracted along the z-axis at a fixed x-coordinate (PROFILE_COORD1(J)).

In 3D simulations: both PROFILE_COORD1(J) and PROFILE_COORD2(J) are required. These define the fixed positions in the two directions orthogonal to the profile direction. If PROFILE_DIRECTION(J)= 'x', the profile is taken along the x-axis at constant y = PROFILE_COORD1(J) and z = PROFILE_COORD2(J). If PROFILE_DIRECTION(J)= 'y', the profile is taken along the y-axis at constant x = PROFILE_COORD1(I) and z = PROFILE_COORD2(J).If PROFILE_DIRECTION(J)= 'z', the profile is taken along the z-axis at constant x = PROFILE_COORD1(J) and y = PROFILE_COORD2(J).

Additionally, if multiple meshes are defined, the optional parameter PROFILE_IMESH(J) can be used to specify from which mesh the profile should be extracted. This parameter behaves similarly to POINT_IMESH(J) in point dumps.

Spatial and Temporal Sampling

The parameter DTDUMP_PROFILE controls the time interval (in seconds) at which all profiles are dumped.

The spatial sampling frequency for each profile can be adjusted individually using the optional parameter PROFILE_ISKIP(J). This parameter specifies how often values are recorded along the profile line, for example, a value of 5 records data from every fifth cell. By default, if PROFILE_ISKIP(J) is not specified, data is recorded at every cell. This feature is useful for finely meshed domains or to reduce output file size.

### Example

The input block below defines three profile outputs using the &GPYRO_OUTPUT namelist. This configuration assumes a 3D simulation, as all profiles specify both PROFILE_COORD1 and PROFILE_COORD2 to locate the extraction planes in space.

&GPYRO_OUTPUT N_PROFILE_QUANTITIES = 3

DTDUMP_PROFILE       = 1.0

PROFILE_QUANTITY(1)       = 'TEMPERATURE'

PROFILE_DIRECTION(1)      = 'z'

PROFILE_COORD1(1)         = 0.10

PROFILE_COORD2(1)         = 0.02

PROFILE_ISKIP(1)          = 10

PROFILE_QUANTITY(2)       = 'TEMPERATURE'

PROFILE_DIRECTION(2)      = 'x'

PROFILE_COORD1(2)         = 0.02

PROFILE_COORD2(2)         = 0.20

PROFILE_QUANTITY(3)       = 'XI'

PROFILE_QUANTITY_INDEX(3) = 1

PROFILE_DIRECTION(3)      = 'y'

PROFILE_COORD1(3)         = 0.02

PROFILE_COORD2(3)         = 0.05/

This input defines three profiles to be dumped every 1.0 second as controlled by the DTDUMP_PROFILE parameter.

Profile 1 extracts temperature along the z-direction at fixed positions x = 0.10 m and y = 0.02 m. The spatial sampling is defined by PROFILE_ISKIP(1) = 10, which means one out of every ten cells along the z-axis is recorded.

Profile 2 also extracts temperature, this time along the x-direction, again at fixed y = 0.02 m and z = 0.20 m. No PROFILE_ISKIP is specified here, so the default behavior applies: every cell along the x-axis is recorded.

Profile 3 extracts the mass fraction (XI) for species index 1, along the y-direction, at x = 0.02 m, z = 0.05 m.

### Output File Format

For each defined profile, a separate CSV file is generated. The filename includes the case name (CASENAME) followed by a suffix indicating the mesh index, profile index, and the dumped quantity. The naming convention is as follows:

casename_profile_IMESH_J_QUANTITY(INDEX).csv

For example, for mesh index 1, profile index 3, and quantity XI with species index 1, the file name will be:

CASENAME_profile_01_02_YI(01).csv

The first row of each file contains the initial coordinates of the sampled points along the specified profile direction (PROFILE_DIRECTION). These coordinates correspond to the cell centers and are determined based on the mesh structure and the optional spatial skip parameter (PROFILE_ISKIP). Each subsequent row represents a time step and begins with the simulation time (in seconds), followed by the values of the selected quantity at each sampled point.

It is important to note that the coordinates in the header represent the initial (undeformed) geometry of the domain. If material deformation occurs during the simulation, these coordinates no longer reflect the actual positions of the profile points. To reconstruct the true positions over time, the ‘DLTZN’ quantity can be output as an additional profile. This quantity provides the local cell height at each time step. By accumulating these local thickness values (e.g., summing them from the bottom of the profile), it becomes possible to reconstruct the deformed spatial coordinate of each point and accurately plot quantities as a function of the evolving geometry.

## Smokeview visualization

For 2D and 3D simulations only, Gpyro can generate output files compatible with NIST Smokeview, enabling graphical visualization of the simulation results. These files allow users to view transient 2D slices of various quantities such as temperature, density, or thermal conductivity directly within Smokeview.

### Configuration

The number of quantities to export for Smokeview visualization is specified using the parameter N_SMOKEVIEW_QUANTITIES. Each quantity I  must be configured using the following input parameters:

- SMOKEVIEW_QUANTITY(K): Name of the quantity to visualize (see Table 4),

- SMOKEVIEW_QUANTITY_INDEX(K): (Optional) Index used for species-specific quantities,

- SMOKEVIEW_IMESH(K): (Optional) Mesh index if multiple mesh exist,

- SMOKEVIEW_PLANE(K): Specifies the orientation of the slice plane. Acceptable values are 'xy', 'xz’, or 'yz'.

- SMOKEVIEW_LOCATION(K): Location along the axis perpendicular to the chosen plane, where the slice will be taken.

These inputs follow the same logic as for profile quantities. As with profile outputs, not all entries listed in Table 4 are available for Smokeview visualization; this limitation is indicated by note 3  in Table 4 .

### Example

Below is a configuration example that defines a Smokeview slice of the total reaction rate (REACTION_RATE_K) in the yz plane at x = 0.05, using mesh 1, with a dump interval defined every 1 second:

&GPYRO_OUTPUT N_SMOKEVIEW_QUANTITIES      = 1

DTDUMP_SMOKEVIEW            = 1.0

SMOKEVIEW_QUANTITY(1)       = 'REACTION_RATE_K'

SMOKEVIEW_QUANTITY_INDEX(1) = 0

SMOKEVIEW_IMESH(1)          = 1

SMOKEVIEW_PLANE(1)          ='yz'

SMOKEVIEW_LOCATION(1)       =0.05/

### Output File Format

Gpyro generates a main visualization file named CASENAME.smv. This file serves as the entry point and can be directly opened with Smokeview. The associated slice data are stored in binary .sf files with the following naming convention: CASENAME_XX.sf, where XX corresponds to the index of the dumped quantity.

For detailed usage and visualization options, refer to the official Smokeview documentation [ref].

## Changing the dump interval mid-simulation

In some situations, it may be desirable to change the interval at which Gpyro writes to disk. For example, the time scale associated with smolder propagation is much longer than the timescale associated with transition from smolder to flame. Consequently, disk dumps at an interval of 1 s may be appropriate during smolder propagation, but a smaller dump interval (e.g., 0.001 s) would be needed to “see” the transition to flame. For this reason, an option was added wherein the dump interval can be reduced when the temperature (solid or gas) increases above a certain value. More specifically, when the gas or solid temperature increases above TMP_REDUCED_DTDUMP (units of K), all output files are written to disk at an interval of REDUCED_DTDUMP (units of s).

## Other Output Files

In addition to profile and Smokeview outputs, Gpyro generates several auxiliary files that can be useful for monitoring, debugging, and performance evaluation.

### .out File

Gpyro automatically creates a text file named CASENAME.out. This file provides a complete record of all user-defined input parameters as interpreted by the solver. It is particularly useful for verifying that no input errors (e.g., typos in namelist groups or parameters) are present and that the configuration matches the user’s intention.

In addition to listing inputs, the .out file includes runtime summaries of key quantities, which allow users to monitor the progress and stability of the simulation.

### CPU Timing File

Upon successful completion of a simulation, Gpyro generates a file named CASENAME_timing.csv. This file details the CPU time consumed by the major subroutines. While primarily intended for debugging and performance optimization, this information can also help identify bottlenecks in complex simulations.

### Iteration Convergence File

If the optional parameter DUMP_DETAILED_CONVERGENCE = .TRUE. is set, Gpyro outputs a file named iterations.csv. This file logs the number of iterations performed by each solver at every time step. It is a valuable resource for debugging convergence issues or analyzing the numerical behavior of the solution process.

# Material property estimation

Gpyro is distributed with a companion material property estimation program (see Section 2.5) that is based on genetic algorithm optimization. It is recommended that the spreadsheet-based front end be used when running the material property estimation program because it is easy to make mistakes when manually setting up text files due to the (usually) large number of parameters to be optimized. Therefore, the discussion in this section is focused around the spreadsheet-based front end rather than the actual Namelist groups and keywords that are written to an input file by the spreadsheet-based front end.

All worksheets necessary to use the genetic algorithm property estimation program are contained in the spreadsheet-based front end. There are three worksheets where the user specifies relevant parameters for the property estimation process:  GA_GenInput, GA_phi, and GA_Vars. Each of these worksheets is explained below. Additionally, there are a number of worksheets in which experimental data must be placed. They are named 01, 02, 03 for experiment 1, 2, 3, and so on. The experimental data in these sheets must be laid out in a specific way, as will be described in detail below.

The user is cautioned that there is even less error checking in the Gpyro property estimation program than in the standalone Gpyro program and the inputs are somewhat more cryptic than for the standalone Gpyro program. Thus, it is easy to make a mistake during data entry, and the user should carefully check the model output to make sure the code is doing what the user intended. Often, runtime errors result from erroneous data entry, but the actual cause can be difficult to track down, even when the code is run from within a debugger.

The Gpyro property estimation program can be run either in serial or in parallel, and the serial executable file is the same as the parallel executable file. Parallelization is achieved using Message Passing Interface. The Gpyro property estimation program has been successfully run in parallel on more than 100 cores.

The following command executes the Gpyro property estimation program on a single processor:

gpyro_propest

Since the input file name must be gpyro.data, it is not necessary to specify an input file name as with the standalone Gpyro program. The name of the input file is hardcoded to avoid difficulties encountered with MPI i/o.

The parallel version has not been tested under Windows, but under Linux it is launched by issuing a command similar to the following:

mpirun –np 32 –machinefile machines.linux –machinedir ./ /home3/clauten/bin/gpyro_propest &

The syntax may be slightly different with LAM MPI instead of MPICH MPI, but this example tells the code to run on 32 cores (–np 32), look up the machine names to run on in the file macines.linux (–machinefile machines.linux) located in the current directory (–machinedir ./), and execute the binary file called gpyro_propest located in the /home3/clauten/bin/.

The Gpyro property estimation program can be halted gracefully (important when running in parallel across multiple nodes) by creating a file called “stop.stop” in the working directory. The Gpyro property estimation program only checks for the existence of this file at the completion of a generation, so it may take several minutes (or even several hours) to shutdown after the file stop.stop is created. The contents of stop.stop are irrelevant; the program merely checks to see if it exists and shuts down gracefully if it does.

## Format of input files for material property estimation program

The input file for the material property estimation program is the same as for the standalone Gpyro input file discussed above, plus three additional Namelist groups. Whereas the input file for the standalone Gpyro program can have any file name, the input file for the material property estimation program must be named gpyro.data (this simplifies running in parallel with MPI). The three additional Namelist groups that must be specified in this input file (and the analogous worksheet name in the spreadsheet-based front end) are listed in Table 5.

Table 5: Gpyro property estimation Fortran Namelist groups, associated worksheet in spreadsheet-based front end, and description of Namelist group entries.

| Namelist group name | Worksheet name | Description |
| --- | --- | --- |
| &GA_GENINPUT | GA_GenInput | General/miscellaneous parameters |
| &GA_PHI | GA_Phi | Fitness weightings |
| &GA_VARS | GA_Vars | Variables to be optimized |

An additional input file for each experimental measurement to be used in the property estimation process is required. The naming convention for these files is exp_xx_yy.csv where xx corresponds to IPHI (more on this in the fitness weightings section) and yy is ICASE (the index corresponding to the case number being simulated). The format of these input files is as follows:

NDT

t1, y1

t2, y2

t3, y3

.

.

.

where tn is the nth time point and yn is the experimental quantity (mass loss rate, temperature, or thickness) at that time. Mass loss rates must be specified in units of g/m2-s, temperatures in degrees Celsius, and thickness in meters. For example, the following is a valid experimental data file containing five temperature points:

5

0.0, 27.0

5.0, 50.0

30.0, 200.0

60.0, 400.0

500.0, 450.0

## Miscellaneous parameters – GA_GenInput worksheet

General parameters related to the genetic algorithm are specified in the GA_GenInput worksheet. The parameter NGEN is the number of generations, or evolution steps, that will be taken (unless the user halts execution prematurely). NINDIV is the number of individuals (or chromosomes) in a population. These concepts are explained in Section 7.2.2 of the Gpyro Technical Reference [1]. The parameter MAXCOPIES is the target (maximum) selection number described in Section 7.2.3 of the Gpyro Technical Reference [1].

If the code is run under Linux, then set LINUX = .TRUE.. This parameter isn’t very important, all it does is delete the stop file which tells the code to shutdown gracefully by issuing the shell command “rm -f stop.stop” under Linux and “del stop.stop” under DOS.

The parameter SIMULATED_EXPERIMENTAL_DATA should normally be set to .FALSE.. If it is set to .TRUE., then the Gpyro property estimation program generates a synthetic set of experimental data (instead of reading in actual experimental data) using the material properties specified elsewhere. This was used during the development process to evaluate how well the genetic algorithm could perform for idealized scenarios, i.e. infinitely accurate experimental measurements with a computational model that is a perfect representation of the physical processes occurring.

FITMIN and FITCLIP are two parameters that come in to play only when an individual has a very bad fitness, or the solution does not converge. Normally, these are set to zero. The variable FITEXPONENT is the parameter  in Equation 196 of the Gpyro Technical Reference [1]. The last two parameters affect reproduction (whereby offspring are generated from parents). Normally, offspring are generated as linear combinations of parents (Equation 201 of the Gpyro Technical Reference [1]). However, under some circumstances it may be useful to use the value of only one of the parents, i.e. an entire gene is used. This is analogous to setting  in Equation 201 of the Gpyro Technical Reference [1] to 1.0. The fraction of genes for which this is done is WHOLEGENEFRAC.

The parameter BRUTE_FORCE indicates whether genes for the offspring should be generated randomly, rather than through the normal evolution process. Setting this parameter to .TRUE. essentially disables the genetic algorithm’s ability to exploit promising solutions and the code reverts to a brute force search technique.

During the search process, some solutions that do not converge for the specified initial time step may be encountered. The code can either reduce the time step until a converged solution is obtained (as would be done for a standalone simulation) or immediately kill the trial solution that did not converge. This is controlled by the parameter KILL_NONCONVERGED_SOLUTIONS. It is recommended that this parameter be set to .TRUE. because if it is set to .FALSE.,  then some simulations that do not converge could require a very small time step and consume a large amount of CPU time, while other processors sit idle (if running in parallel)

## Fitness metric weightings – GA_phi worksheet

The fitness metric weightings, i.e. the  values in Equation 197 of the Gpyro Technical Reference [1], are specified in the worksheet GA_phi. Each  value multiplies a fitness function (Equation 196 of the Gpyro Technical Reference [1]) that quantifies the level of agreement between model predictions and experimental data. In this way, certain experiments or measurements can be given greater importance than others. The user should first specify the number of  values (the number of experimental measurements) in cell A1, labeled NGA_PHI.

In addition to the actual  value (specified in column F) a few additional parameters must be given. The integer IQE (index of qe) in column B corresponds to the case # listed in the qe worksheet. For example, if IQE = 2, then the  value specified is for case #2 in the qe worksheet.

Several types of experimental data can be used as fitness metrics:  cumulative mass loss, mass loss rate, temperature, and thickness. The code is told what type of experimental data the  value corresponds to in column C (TYPE). Here, the user enters one of four 3-character strings to indicate which type experimental data is being used: TMP (temperature, ºC), MLR (mass loss rate, g/m2–s), DLT (delta, or thickness, m), and CML (cumulative mass loss, g/m2).

Column D is labeled “zT”. This column tells the code the depth (location) of a particular temperature measurement. For example, zT = 0 indicates a surface temperature measurement, zT = 0.005 indicates an in–depth temperature measurement 5 mm below the surface, and so on. Column D has meaning only if TMP is specified in column C.

Column E is labeled tstop. This is a time in seconds that can be less than or equal to the simulation time specified in the qe worksheet. Setting a value of tstop less than the simulation time directs Gpyro to calculate the fitness metric only through a time of tstop. This is convenient if the surface temperature was measured but deemed accurate only prior to ignition (at which point flaming flame can add uncertainty to temperatures measured via thermocouple or optical methods). By setting the value of tstop for the  value corresponding to the surface temperature to equal the observed ignition time, the code will only use the measured surface temperature before the ignition time when evaluating that part of the overall fitness. The actual  value is specified in column F. There are no restrictions on the range of acceptable  values, except that it doesn’t make sense to have a negative  value. What matters is the relative values of different  values.

Finally, column G is a small number () that prevents the fitness metric from approaching infinity when the model calculation is very close to the experimental data point. See Equation 196 in the Gpyro Technical Reference [1] for further explanation.

## Variables to optimize – GA_Vars worksheet

The model input parameters to be estimated from the available experimental data are specified in the GA_Vars worksheet. Each variable to be optimized is called a gene, and the number of variables to be optimized is specified in cell A1.

Each variable (gene) should be numbered sequentially in column A starting in row 11. In Column B, the name of the worksheet containing the variable to be optimized is specified. The valid choices are indicated in column B, rows 3–8. When optimizing variables from the gyields or hgyields worksheets, the code performs normalization to ensure that the sum of all gyields add to 1 and that the sum of all hgyields add to 0, as is required for mass conservation. However, for this to work correctly, the user must include all nonzero yields as variables to be optimized. For example, if the column of the gyields matrix for a particular condensed phase reaction has four nonzero yields, then these four yields should be specified as variables to be optimized in the GA_Vars worksheet. The homogeneous gaseous yields are similar, except that it is not necessary to specify the yield of reactant 1 because it is always -1.

Column E is the variable (gene) type. The valid variable types for each sheet name are indicated in Column E from rows 3 to 9. For example, if sprops is specified in column B, then the valid variable types are K0Z (thermal conductivity at reference temperature), NKZ (thermal conductivity exponent), and so on. If gyields or hgyields is specified in column B, then it is not necessary to specify a variable type because the code automatically knows that gene must correspond to a species yield.

Two indices are entered in columns D and E (called, very descriptively, Index 1 and Index 2). The role of these two integers depends on the sheet name specified in column B. For example, if the sheet name in column B is sprops, then column C (Index 1) is ISSPEC, the index of solid species as indicated in the sprops sheet. Similarly, if the sheet name in column B is rxns, then column C is IRXN, the index of the reaction (specified in the rxns worksheet). Index 2 does not matter for variables from worksheets sprops or gprops. However, for variables from worksheets gyields and hgyields, Index 1 is the index of the gaseous species, and Index 2 is the reaction number. Thus, if gyields is specified in column B, Index 1 is set to 2, and Index 2 is set to 3, then the gene corresponds to the gaseous yield for the formation of gaseous species 2 from heterogeneous reaction 3.

Columns F and G (MinVal and MaxVal) are the lower and upper bounds of each variable, i.e. the values of aj,min and aj,max in Equation 195 of the Gpyro Technical Reference [1]. These should be specified to limit the search space to physically realistic values, i.e. it is known that the thermal conductivity of wood isn’t 1000 W/m-K. Column H is a logical parameter indicating whether the values of MinVal and MaxVal are the logarithm of the gene. For example, with a MinVal of 10 and a MaxVal of 12, if log is set to .TRUE. then the code will constrain the search from a minimum value 1010 to a maximum value of 1012;  but if log is set to .FALSE., then the code will search for values between 10 and 12.

The mutation probability (see Section 7.2.5 of the Gpyro Technical Reference [1]) is specified in column I. The parameter vmut in Equation 202b of the Gpyro Technical Reference [1] is specified in column J.

The integer in column K provides a convenient means to treat two genes as one parameter for reproduction purposes. For example, the logarithm of the pre-exponential factor and the activation energy are positively correlated, so it may be convenient to form offspring from two parents by applying the same weightings to both the log of the pre-exponential factor and the activation energy. This can be enabled by setting the value in column K to an integer greater than zero. Then, for reproduction purposes, the linear combination applied to the index of the gene specified in column K will also be applied to the current gene.

For solid properties, Index 2 is normally set to 0. However, if Index 2 is set to a solid species index (as specified in sprops) and both MinVal and MaxVal are negative, then the variable is actually the negative of the gene multiplied by the value of the analogous variable of the solid species corresponding to Index 2. This is confusing and is best illustrated by an example:  if Index 2 is set to 1, MinVal is set to -1, and MaxVal is set to -0.5, then the gene search space is between 0.5 and 1.0 multiplied by the value of the analogous gene for species 1. The same also applies to the rxns worksheet. Representing the genes in this way is convenient when it is known that one variable must be smaller or larger than another;  for example, the thermal conductivity of a char is likely lower than the thermal conductivity of the virgin material from which it was formed, and the activation energy of an oxidative reaction may be lower than the activation energy of an analogous anaerobic reaction.

## Experimental data – 01, 02, 03, etc. worksheets

The experimental data must be pasted into worksheet 01 for case # (or IQE) 1, worksheet 02 for case # (or IQE) 2, and so on. Each  value specified in GA_phi should have an analogous set of experimental data (with the exception of cumulative mass loss, which Gpyro automatically calculates by integrating the mass loss rate). The units for temperature measurements are ºC, mass loss rate is g/m2-s, and thickness is m. For a TGA experiment, mass loss rate data should be entered as 1000 × d(m/m0)/dt where m is the sample mass and m0 is the initial sample mass. Be sure to express this derivative as d/dt and not d/dT (time derivative instead of temperature derivative).

The experimental data must be entered according to a specific layout, and in an order that corresponds to the order in which the  coefficients are specified in the GA_phi worksheet. Column A is the time axis for the first experimental measurement for which a  value was specified for that IQE in GA_phi. Column B is the corresponding experimental data at each time specified in column A. Column C is blank, and the next set of experimental measurements start in column D, i.e. column D is the time axis for the second experimental measurement, and column E is the analogous experimental data at each time. Column F is blank, and columns G and H would contain the next set of experimental data. Column I is blank, and columns J and K would contain the next set of experimental data, and so on. The order in which the experimental data is specified must correspond to the TYPE order specified in GA_phi. Do not specify experimental data for any cumulative mass loss measurements because this is calculated by integration from the mass loss rate. It is easy to make mistakes in this part of the data input process, and the input routines haven’t been rigorously exercised, so be sure to check the output data to make sure that what the code has read in what you intended!

# References

[11] 	Lautenberger, C., “Gpyro – A Generalized Pyrolysis Model for Combustible Solids – Technical Reference Guide,” Version 0.800, May 1, 2014.

[22] 	Lautenberger, C.W., “A Generalized Pyrolysis Model for Combustible Solids,” Ph.D. Dissertation, Department of Mechanical Engineering, University of California, Berkeley, Fall 2007.

http://reaxengineering.com/docs/lautenberger_dissertation.pdf

[33] 	Lautenberger, C. & Fernandez-Pello, A.C., “Generalized Pyrolysis Model for Combustible Solids,” Fire Safety Journal 44 819-839 (2009).

[44] 	Lautenberger, C. & Fernandez-Pello, A.C., “A Model for the Oxidative Pyrolysis of Wood,” Combustion and Flame 156 1503-1513 (2009).

[55] 	Dodd, A.B., Lautenberger, C. & Fernandez-Pello, A.C., “Numerical Examination of Two-Dimensional Smolder Structure in Polyurethane Foam,” Proceedings of the Combustion Institute 32: 2497-2504 (2009).

[66] 	Lautenberger, C., Kim, E., Dembsey, N. & Fernandez-Pello, C., “The Role of Decomposition Kinetics in Pyrolysis Modeling – Application to a Fire Retardant Polyester Composite,” Fire Safety Science 9: 1201-1212 (2008).

[77]	Lautenberger, C., Rein, G. & Fernandez-Pello, A.C., “Application of a Genetic Algorithm to Estimate Material Properties for Fire Modeling from Bench-Scale Fire Test Data,” Fire Safety Journal 41 204-214 (2006)

[88] 	Lautenberger, C. & Fernandez-Pello, C., “Optimization Algorithms for Material Pyrolysis Property Estimation,” Fire Safety Science 10 751-764 (2011).

[99] 	Lautenberger, C., “Gpyro3D:  A Three Dimensional Generalized Pyrolysis Model,” accepted for publication in Fire Safety Science (IAFSS Symposium), 2014.

[1010] 	McGrattan, K., Hostikka, S., McDermott, R., Floyd, J., Weinschenk, C., and Overholt, K., “Fire Dynamics Simulator Technical Reference Guide Volume 1: Mathematical Model,” NIST Special Publication 1018, Sixth Edition, National Institute of Standards and Technology, Building and Fire Research Laboratory, Gaithersburg, MD, 2015. SVN 22343.

# Appendix A. Nomenclature

|  | Description | Units |
| --- | --- | --- |
| Letters |  |  |
| cp | Solid heat capacity |  |
| cpg | Gas heat capacity |  |
| dp | Mean pore diameter | m |
| D | Mass diffusivity | m2.s-1 |
| D | Deformation | - |
| E | Activation Energy | kJ.mol-1 |
| hc | Convective heat transfer coefficient | W.m-2.K-1 |
| k | Heat conductivity | W.m-1.K-1 |
| K | Permeability | m2 |
| m | Mass | kg |
| M | Molecular weight | g.mol-1 |
| P | Pressure | Pa |
| r | Reaction rate | kg.m-3.s-1 |
| R | Universal gas constant | J.K-1.mol-1 |
| T | Temperature | K |
| qrerad | Thermal reradiation of the surface | W.m-2 |
| V | Volume | m3 |
| X | Volume fraction | m3/m3 |
| y | species yield |  |
| Y | Mass fraction | kg/kg |
| Z | pre-exponential factor | s-1 |
| Greek symbols |  |  |
| α | Thermal diffusivity | m2.s-1 |
| α | Conversion Rate | - |
| γ | Pore radiation coefficient | m-1 |
| ΔH | Enthalpy of reaction |  |
| Δt | Temporal time step | s |
| Δx | Spatial step in x-direction | m |
| Δy | Spatial step in y-direction | m |
| Δz | Spatial step in z-direction | m |
| ϵ | Emissivity of the surface | - |
| θ | Stoichiometric coefficient | kg.kg-1 |
| ρ | Solid bulk density | kg.m-3 |
| ρs | Solid skeleton density | kg.m-3 |
| ρg | Gas density | kg.m-3 |
| σ | Stefan–Boltzmann constant | W.m-2.K-4 |
| ν | Gas kinematic viscosity | m2.s-1 |
| χ | Deformation factor |  |
| ψ | Porosity | - |
| ω''' | Volumetric reaction rate | kg.m-3.s-1 |
| Subscripts |  |  |
| A | Quantity related to solid species A |  |
| B | Quantity related to solid species B |  |
| d | Destruction |  |
| f | Formation |  |
| g | Gas |  |
| i | Quantity related to the solid species of index i |  |
| inf | Quantity at infinity (outside boundary condition) |  |
| j | Quantity related to the gas species of index j |  |
| k | Quantity related to the condesed-phase reaction of index k |  |
| l | Quantity related to the gas-phase reaction of index l |  |
| net | Net production (formation - destruction) |  |
| s | Solid |  |
| sol | Contribution due to the solid phase |  |
| surf | Quantity at the surface of a boundary condition |  |
| vol | Contribution due to the volatile phase |  |
| x | Quantity in the x direction |  |
| y | Quantity in the y direction |  |
| z | Quantity in the y direction |  |

# Appendix B. Namelist group names and available keywords.

Table 6: &GPYRO_GENERAL keywords (general parameters)

| Keyword | Type | Description | Units | Default |
| --- | --- | --- | --- | --- |
| DT0 | Real | Section 5.1 | s |  |
| TAMB | Real | Section 4.2 |  |  |
| TREF | Real | Section 9.2.1 | s |  |
| P0 | Real | Section 4.3 |  |  |
| GX | Real | Section 4.3 |  | 0 |
| GY | Real | Section 4.3 | m.s-2 | 0 |
| GZ | Real | Section 4.3 | m.s-2 | 0 |
| VHLC | Real | Section 4.2 | m.s-2 | 0 |
| SOVE_GAS_YJ | Integer | Section 13.3.14.4 |  | .FALSE. |
| SOLVE_GAS_ENERGY | Logical | Section 4.5 |  | .FALSE. |
| SOLVE_PRESSURE | Logical | Section 4.3 |  | .FALSE. |
| CONVENTIONAL_RXN_ORDER | Logical | Section 13.3.111.4.3 |  | .FALSE. |
| BLOWING | Logical | Section 8.2.2 |  | .FALSE. |
| CONSTANT_DHVOL | Logical | Section 11.5 |  | .TRUE. |
| QSG_MODEL | String | Section 4.2 |  | ‘thermal_equilibrium’ |

Table 7: &GPYRO_CASES keywords (cases to run)

| Keyword | Type | Description | Units | Default |
| --- | --- | --- | --- | --- |
| NCASES | Integer | Section 5.1 |  | 1 |
| IMESH(ICASE) | Integer | Section 5.1 |  | 1 |
| TSTOP(ICASE) | Real | Section 5.1 | s |  |
| ZEROD(I) | Logical | Section 5.2 |  | F |
| BETA(I) | Real | Section 5.2 | °C/min |  |

Table 8: &GPYRO_GEOM keywords (geometry specification)

| Keyword | Type | Description | Units | Default |
| --- | --- | --- | --- | --- |
| NMESH | Integer | Section 6.0 |  | 1 |
| ZDIM(IMESH) | Real | Section 6.1 | m |  |
| XDIM(IMESH) | Real | Section 6.1 | m | 1 |
| YDIM(IMESH) | Real | Section 6.1 | m | 1 |
| NCELLZ(IMESH) | Integer | Section 6.1 |  |  |
| NCELLX(IMESH) | Integer | Section 6.1 |  | 1 |
| NCELLY(IMESH) | Integer | Section 6.1 |  | 1 |
| DEFAULT_IC(IMESH) | Integer | Section 6.4 |  |  |
| DEFAULT_SURF_IDX(IMESH,1:6) | Integer | Section 6.4 |  |  |
| NOBST | Integer | Section 6.3 |  | 0 |
| IMESH(IOBST) | Integer | Section 6.3 |  | 1 |
| Z1(IOBST) | Real | Section 6.3 | m |  |
| Z2(IOBST) | Real | Section 6.3 | m |  |
| X1(IOBST) | Real | Section 6.3 | m |  |
| X2(IOBST) | Real | Section 6.3 | m |  |
| Y1(IOBST) | Real | Section 6.3 | m |  |
| Y2(IOBST) | Real | Section 6.3 | m |  |
| ICNUM(IOBST) | Integer | Section 6.4 |  | DEFAULT_IC |
| SURF_IDX2D(IOBST,1:6) | Integer | Section 6.4 |  | DEFAULT_SURF_IDX |

Table 9: &GPYRO_IC keywords (initial condition specification).

| Keyword | Type | Description | Units | Default |
| --- | --- | --- | --- | --- |
| NIC | Integer | Section 7.0 |  | 1 |
| TMP_INITIAL(IC) | Real | Section 7.0 | K | 300 |
| TMPG_INITIAL(IC) | Real | Section 7.0 | K | 300 |
| P_INITIAL(IC) | Real | Section 7.0 | Pa | 101300 |
| YI0(IC,ISPEC) | Real | Section 7.0 | kg/kg | 0 |
| YJ0(IC,IGSPEC) | Real | Section 7.0 | kg/kg | 0 |

Table 10: &GPYRO_ALLBC keywords (boundary condition specification).

| Keyword | Type | Description | Units | Default |
| --- | --- | --- | --- | --- |
| NSURF_IDX | Integer | Section 8.1 |  | 0 |
| SURF_IDX(IBC) | Integer | Section 8.1 |  |  |
| T(IBC) | Real | Section 8.1 | s | 0 |
| TFIXED(IBC) | Real | Section 8.2.1 | K | -1 |
| HC(IBC) | Real | Section 8.2.2 | W.m-2.K-1 | 10 |
| TINF(IBC) | Real | Section 8.2.2 | K | 300 |
| QE(IBC) | Real | Section 8.2.2 | W.m-2 | 0 |
| RERADIATION(IBC) | Logical | Section 8.2.2 |  | T |
| TFIXEDG(IBC) | Real | Section 8.3 | K | -1 |
| HCG(IBC) | Real | Section 8.3 | W.m-1.K-1 | 10 |
| TINFG(IBC) | Real | Section 8.3 | K | 300 |
| QE(IBC) | Real | Section 8.3 | W.m-2 | 0 |
| MDOTPP(IBC) | Real | Section 8.4 | g.m-2.s-1 | 0 |
| PRES(IBC) | Real | Section 8.4.2 | Pa |  |
| YJINF(IBC,IGSPEC) | Real | Section 8.5 | Kg/kg | 0 |
| HM(IBC) | Real | Section 8.5 | kg·m⁻²·s⁻¹ |  |

Table 11: &GPYRO_SPROPS keywords (solid properties).

| Keyword | Type | Description | Units | Default |
| --- | --- | --- | --- | --- |
| NSSPEC | Integer | Section 9.1 |  |  |
| NAME(IDM) | String | Section 9.1 |  |  |
| R0(IDM) | Real | Section 9.2.3 and 11.2 | kg.m-3 |  |
| NR(IDM) | Real | Section 9.2.3 | - | 0 |
| K0Z(IDM) | Real | Section 9.2.1 | W.m-1.K-1 |  |
| NKZ(IDM) | Real | Section 9.2.1 | - | 0 |
| K0X(IDM) | Real | Section 9.2.1 | W.m-1.K-1 | K0Z(IDM) |
| NKX(IDM) | Real | Section 9.2.1 | - | NKZ(IDM) |
| K0Y(IDM) | Real | Section 9.2.1 | W.m-1.K-1 | K0Z(IDM) |
| NKY(IDM) | Real | Section 9.2.1 | - | NKZ(IDM) |
| GAMMA(IDM) | Real | Section 9.2.1 | m | 0 |
| C0(IDM) | Real | Section 9.2.2 | J·kg⁻¹·K⁻¹ |  |
| NC(IDM) | Real | Section 9.2.2 | - | 0 |
| EMIS(IDM) | Real | Section 9.2.4 | - |  |
| KAPPA(IDM) | Real | Section 9.2.4 | m-1 | 9.109 |
| TMELT(IDM) | Real | Section 9.2.2 | K | 9.109 |
| DHMELT(IDM) | Real | Section 9.2.2 | J.kg-1 | 0 |
| SIGMA2MELT(IDM) | Real | Section 9.2.2 | K2 | 1 |
| PERMZ(IDM) | Real | Section 9.3.2 | m2 |  |
| PERMX(IDM) | Real | Section 9.3.2 | m2 | PERMZ(IDM) |
| PERMY(IDM) | Real | Section 9.3.2 | m2 | PERMZ(IDM) |
| RS0(IDM) | Real | Section 9.3.1 | kg.m-3 | R0(IDM) |
| PORE_DIAMETER(IDM) | Real | Section 9.3.3 | m |  |

Table 12: &GPYRO_GPROPS keywords (gas properties).

| Keyword | Type | Description | Units | Default |
| --- | --- | --- | --- | --- |
| NGSPEC | Integer | Section 10.1 |  | 1 |
| I02 | Integer | Section 10.2 and 10.5 |  |  |
| IBG | Integer | Section 10.5 |  |  |
| CPG | Real | Section 10.3 | J·kg⁻¹·K⁻¹ | 1000 |
| NAME(IGAS) | String | Section 10.1 |  |  |
| M(IGAS) | Real | Section 10.4 | g.mol-1 |  |
| SIGMA(IGAS) | Real | Section 10.5 | Å |  |
| EPSOK(IGAS) | Real | Section 10.5 | K |  |

Table 13: &GPYRO_RXNS keywords (condensed phase reaction properties).

| Keyword | Type | Description | Units | Default |
| --- | --- | --- | --- | --- |
| NRXNS | Integer | Section 11.1 |  | 0 |
| CFROM(IRXNS) | String | Section 11.1 |  |  |
| CTO(IRXNS) | String | Section 11.1 |  |  |
| CHI(IRXNS) | Real | Section 11.2 | - | 1 |
| Z(IRXNS) | Real | Section 11.4.2 | s-1 |  |
| E(IRXNS) | Real | Section 11.4.2 | kJ.mol-1 |  |
| IKINETICMODEL(IRXNS) | Integer | Section 11.4.3 and Table 3 |  | 0 |
| IO2TYPE(IRXNS) | Integer | Section 11.4.3 |  | 0 |
| ORDER(IRXNS) | Integer | Section 11.4.3 | - | 1 |
| ORDERO2(IRXNS) | Integer | Section 11.4.3 | - | 0 |
| M(IRXNS) | Real | Section 11.4.3 | - | 0 |
| KCAT(IRXNS) | Real | Section 11.4.3 | - |  |
| ICAT(IRXNS) | Integer | Section 11.4.3 |  |  |
| TCRIT(IRXNS) | Real | Section 11.4.3 | K | 0 |
| DHV(IRXNS) | Real | Section 11.5 | J.kg-1 | 0 |
| DHS(IRXNS) | Real | Section 11.5 | J.kg-1 | 0 |

Table 14: &GPYRO_GYIELDS keywords (the species‑yield matrix GYIELDS).

| Keyword | Type | Description | Units | Default |
| --- | --- | --- | --- | --- |
| GYIELDS(IGAS,IRXNS) | Real | Section 11.3 | - | 0 |

Table 15: &GPYRO_HGYIELDS keywords (the species‑yield matrix HGYIELDS).

| Keyword | Type | Description | Units | Default |
| --- | --- | --- | --- | --- |
| HGYIELDS(IGAS,IGRXNS) | Real | Section 12.1 | kg | 0 |

Table 16: &GPYRO_HGRXNS keywords (gas phase reaction properties).

| Keyword | Type | Description | Units | Default |
| --- | --- | --- | --- | --- |
| NHGRXNS | Integer | Section 12.1 |  | 0 |
| CREACTANT1(IGRXNS) | String | Section 12.1 |  |  |
| CREACTANT2(IGRXNS) | String | Section 12.1 |  |  |
| P(IGRXNS) | Real | Section 12.2 | - | 1 |
| Q(IGRXNS) | Real | Section 12.2 | - | 1 |
| B(IGRXNS) | Real | Section 12.2 | - | 0 |
| Z(IGRXNS) | Real | Section 12.2 | s-1 |  |
| E(IGRXNS) | Real | Section 12.2 | kJ.mol-1 |  |
| DH(IGRXNS) | Real | Section 12.3 | J.kg-1 | 0 |

Table 17: &GPYRO_OUTPUT keywords (output quantities).

| Keyword | Type | Description | Units | Default |
| --- | --- | --- | --- | --- |
| CASENAME | String | Section 13.0 |  | 'gpyro' |
| N_POINT_QUANTITIES | Integer | Section 13.2.1 |  | 0 |
| DTDUMP_POINT | Real | Section 13.2.1 | s |  |
| POINT_QUANTITY(I) | String | Section 13.2.1 |  |  |
| POINT_QUANTITY_INDEX(I) | Integer | Section 13.2.1 |  |  |
| POINT_Z(I) | Real | Section 13.2.1 | m |  |
| POINT_X(I) | Real | Section 13.2.1 | m |  |
| POINT_Y(I) | Real | Section 13.2.1 | m |  |
| POINT_IMESH(I) | Integer | Section 13.2.1 |  | 1 |
| N_PROFILE_QUANTITIES | Integer | Section 13.3.1 |  | 0 |
| DTDUMP_PROFILE | Real | Section 13.3.1 | s |  |
| PROFILE_QUANTITY(J) | String | Section 13.3.1 |  |  |
| PROFILE_QUANTITY_INDEX(J) | Integer | Section 13.3.1 |  |  |
| PROFILE_DIRECTION(J) | String | Section 13.3.1 |  |  |
| PROFILE_COORD1(J) | Real | Section 13.3.1 | m |  |
| PROFILE_COORD2(J) | Real | Section 13.3.1 | m |  |
| PROFILE_IMESH(J) | Integer | Section 13.3.1 |  | 1 |
| PROFILE_ISKIP(J) | Integer | Section 13.3.1 |  | 1 |
| N_SMOKEVIEW_QUANTITIES | Integer | Section 13.4.1 |  | 0 |
| DTDUMP_SMOKEVIEW | Real | Section 13.4.1 | s |  |
| SMOKEVIEW_QUANTITY(K) | String | Section 13.4.1 |  |  |
| SMOKEVIEW_QUANTITY_INDEX(K) | Integer | Section 13.4.1 |  |  |
| SMOKEVIEW_PLANE(K) | String | Section 13.4.1 |  |  |
| SMOKEVIEW_LOCATION(K) | Real | Section 13.4.1 | m |  |
| TMP_REDUCED_DTDUMP | Real | Section 13.5 | K | 9. 109 |
| REDUCED_DTDUMP | Real | Section 13.5 | s |  |
| DUMP_DETAILED_CONVERGENCE | Logical | Section 13.6.3 |  | F |

Table 18: &GA_GENINPUT keywords (property estimation general/miscellaneous).

| Keyword | Default value | Units | Description |
| --- | --- | --- | --- |
| NINDIV | 250 |  | Population size (#of individuals) |
| NGEN | 100 |  | Number of generations (evolution steps) |
| MAXCOPIES | 6 |  | Maximum number of copies - target sel rate |
| LINUX | .TRUE. |  | Is this run under Linux? |
| SIMULATED_EXPERIMENTAL_DATA | .FALSE. |  | Generate synthetic experimental data? |
| RESTART | .FALSE. |  | Restart from file restart.restart? |
| FITMIN | 0.0 |  | Set very bad fitness to this |
| FITCLIP | 0.0 |  | If fitness is <= this or NaN, set fitness to this |
| FITEXPONENT | 2.0 |  | Raise fitness value to this exponent |
| WHOLEGENEFRAC | 0.5 |  | Fraction of genes that are not linear combins. |
| BRUTE_FORCE | .FALSE. |  | Use brute force (no genetic algorithm?) |
| KILL_NONCONVERGED_SOLNS | .TRUE. |  | Kill solutions that don’t converge? |

# Appendix C. Gas properties

| Substance | Molecular Weight | Lennard–Jones Parameters |  |
| --- | --- | --- | --- |
|  | M (g/mol) | σ (Å) | ε/k (K) |
| Light elements: |  |  |  |
| H₂ | 2.016 | 2.915 | 38.0 |
| He | 4.003 | 2.576 | 10.2 |
| Noble gases: |  |  |  |
| Ne | 20.183 | 2.789 | 35.7 |
| Ar | 39.944 | 3.418 | 124. |
| Kr | 83.80 | 3.498 | 225. |
| Xe | 131.3 | 4.055 | 229. |
| Simple polyatomic substances: |  |  |  |
| Air | 28.97 | 3.617 | 97.0 |
| N₂ | 28.02 | 3.681 | 91.5 |
| O₂ | 32.00 | 3.433 | 113. |
| CO | 28.01 | 3.590 | 110. |
| CO₂ | 44.01 | 3.996 | 190. |
| NO | 30.01 | 3.470 | 119. |
| N₂O | 44.02 | 3.879 | 220. |
| SO₂ | 64.07 | 4.290 | 252. |
| F₂ | 38.00 | 3.653 | 112. |
| Cl₂ | 70.91 | 4.115 | 357. |
| Br₂ | 159.81 | 4.268 | 520. |
| I₂ | 253.82 | 4.982 | 550. |
| Hydrocarbons: |  |  |  |
| CH₄ | 16.04 | 3.822 | 137. |
| C₂H₂ | 26.04 | 4.221 | 185. |
| C₂H₄ | 28.05 | 4.232 | 205. |
| C₂H₆ | 30.07 | 4.418 | 130. |
| C₃H₈ | 44.09 | 5.061 | 254. |
| i-C₄H₁₀ | 58.12 | 5.341 | 313. |
| n-C₅H₁₂ | 72.15 | 5.769 | 343. |
| n-C₆H₁₄ | 86.17 | 5.909 | 413. |
| n-C₈H₁₈ | 114.22 | 7.451 | 320. |
| Cyclohexane | 84.16 | 6.093 | 324. |
| C₆H₆ | 78.11 | 5.270 | 440. |
| Other organic compounds: |  |  |  |
| CH₄ | 16.04 | 3.822 | 137 |
| CH₃Cl | 50.49 | 3.375 | 855. |
| CH₂Cl₂ | 84.94 | 4.759 | 406. |
| CHCl₃ | 119.39 | 5.430 | 327. |
| CCl₄ | 153.82 | 5.881 | 327. |
| C₂N₂ | 62.04 | 4.38 | 339. |
| COS | 60.08 | 4.13 | 335. |
| CS₂ | 76.14 | 4.438 | 488. |

Gpyro and its coupling to FDS is not supported or endorsed by NIST or VTT in any way.